SANTIAGO HIGH SCHOOL

ARCHITECTURE DESIGN 1A/1B

Software: REVIT 2018

Instructor:

BILL BROWN

# Email bbrown@cnusd.k12.ca.us

# Table of Contents

Email bbrown@cnusd.k12.ca.us	1

Table of Contents	2

Revit Interface	5

Doors in Revit	16

Windows in Revit	21

Plan Regions: Clerestory Windows	31

Floors and ceilings.	34

Roofs	38

Putting your work onto paper in REVIT	45

Sheet Naming Convention	48

Section views in Revit	49

Furniture, fixtures and viewing in Revit	55

Controlling what you see	58

More Sections	60

Interior Elevations	62

Revit Schedules	75

Controlling what you see (More)	83

Smart Wall Creation	93

Details in Revit	95

Smart Wall Creation Continued	95

Smart Brick Wall	102

Rooms	104

Dimensioning in Revit	113

Creating a Site Plan in Revit	117

Basic Stairs in Revit	128

Slab Foundation Plan	137

Callouts	148

Area Plans	158

Creating Materials in Revit 2018: Basics	176

Applying Smart Material Patterns to Floors	197

Creating Fill Pattern	201

Applying Materials To Interior Elevation Surfaces: Your 2 Options	206

View Templates	216

Creating Custom Linestyles	218

Filters	220

Electrical Plan	222

Electrical Legend	231

Wall Type Legend	233

Creating a Gypsum Board Ceiling	234

Wall Sweeps	238

Wall Sweeps: Base and Crown Molding	241

Creating a “Smart” Furniture Family	252

Material as a Parameter: Family	270

Families: Steps to Creating Families	273

Title Block Family	274

Smart Window Tag	280

Drafting Views	284

Keynotes	287

Creating the Keynote Legend	296

Grouping	301

Creating Custom/Shared Parameters	306

Room Schedules	312

Room Bounding…Controlling where your Room Goes	315

Sheet List	316

Shared Coordinates Demo	317

Note Blocks	325

Phasing	329

Copy Monitor	331

Purging	343

Transfer project Standards	344

Phasing for Tenant Improvements	347

“Stealing” From Other Revit Projects	349

Creating a .PDF of your Revit Sheet	350

Key Schedule	351

Demo: Structural Components in Revit	354

Revit 2018 Path Issues Solution	370

Spanish Character Floor Plan 2017-18	371

A Home for Mr. Bond	374

Drawings Required  (you will have close to 20 sheets!)	375

Residential Floor Plan Checklist	379

Bathroom Elements: Checklist	381

A HOME FOR MR. BOND: Floor Plan Design Notes	382

Cover Sheet Checklist	384

Site Plan Checklist	385

Presentation Plan Checklist	386

Interior Elevation Checklist	387

Foundation Plan Checklist	388

Exterior Elevation Checklist	389

Kitchen Plan Checklist	390

Section Checklist	391

Schedule Checklist	393

Project 2 Assignment	394

Revit Resources	402

Curtain Systems	403

New Features in Revit 2018	424

# Revit Interface

Note to teachers: I have set up this tutorial so you may “get lost” during a section and not be way behind. Simply open the current drawing to get back on pace. The tutorial is also set up so you may go to a specific area to try a command without having to do the entire tutorial. If you want to go straight to creating a roof, open Revit2018demo4.rvt.

I also set this tutorial up to be presented once by me to you (the teacher), then presented to your students/employees.

I prefer that during my presentation, you do not take notes. That’s what this book is for. I prefer that you watch what I do, then do what I do. If you are frantically taking notes…..you are not watching me  …making it tougher get all the details.

You may reproduce this document as much as you want for use in the classroom….

Professionals not employed by Bill Brown do not have permission to use or duplicate the information in this book.

All of the files we will use in the workshop will be made available.

I have bolded some text as reminders to me to discuss issues as they come up.

I think you will like Revit as a tool to create your architectural drawings. It’s not AutoCAD or AutoCAD Architecture, so some of your normal drawing strategies will change some….Students do not need to know AutoCAD to learn Revit, but general Architecture knowledge is important.

Let’s get started!

Revit is a database.

Give analogy to filling out a contest form with your name, address, age, income, interests, gender, occupation, etc. Companies leverage that data to target mailings to go to a group more likely to buy their product. Explain how Revit does the same with the building model.

Make a few walls. Esc to finish. Put two 24 X 48 windows in the walls.  Create the Windows with their head height at 6’8” ft. RC on window….Properties…set head height. For the purposes of this exercise, make sure your two windows are facing to the front of the house.  Put the South elevation on a sheet to print.  RC on Sheets…New sheet…take the E size paper. Go to the elevation view.  Dimension the head height.  It should be 6’8” ft. tall.  Go to the printing sheet.  See seven foot dimension?  Go back to the plan view.  Change the Windows to 8 ft. head height.  Now show the plotting sheets again.  It's all linked in a database!  What changes in one view, changes in all the views.

Show “Ribbon”		Project Browser		Option Bar

Architecture			Views				Chain

Insert				Level 1			Height

Annotate			Ceiling Plan

Modify

Massing & Site

Collaborate

View

Manage

The “Ribbon” in Revit 2018

Show Arrow to Change ribbon

The “Option Bar” in Revit 2018

The ”Project Browser” in Revit 2018

Show Properties toolbar in standard spot

Show tear off of Properties Toolbar

Show View display Toolbar and its commands…

Walls

Architecture….Wall Architectural….. show different types of “out of the box” walls under “Change Element Type”.

Unconnected

Move pencil icon to screen.

Make some walls in a typical cross-gable mass.  Don't worry about size.  Select the chain box before starting. Make them Generic 5”. Height: Level 2

Show Wall Properties

Escape to end.

Modify wall sizes.

To modify horizontal sizes, you must pick on vertical walls.  Once you pick on a vertical wall, you may select the dimension and modify it.  Left select another vertical surface, and modify the dimension. Pick the dimension symbol to lock your sizes. You should have just modified a horizontal size.

Temporary dimensions (the blue ones) control your model size.

The building will be 60’ long, 50’ deep, garage wall 25’ long, back right wall 30’ deep.

To modify vertical sizes, you must pick on horizontal walls.  Use the same technique to modify the vertical distances.  Left select outside the house to end the command.

Hint: The wall to pick, will be the wall that is moved by the temporary (blue) dimensions.

Erase the dimensions for now. LC on dim….. Delete.

Open the South elevation in the Project Browser.  Notice the wall height?  The walls were drawn to an “Unconnected” height. Explain

Pick all of the walls. Properties. Change the top constraint height to Level 2.

Discuss why going to Level 2 is a better strategy than Unconnected 20’ ….by making Level 2 at 12’. Then at 15’….then at 50’! Get it? That will be a big deal later in the project.

Discuss “Location Line” choices (In the Option Bar): Wall Centerline, Core Centerline, Finish Face: Exterior, Finish Face: Interior, Core Face: Exterior, Core Face: Interior.  You may adjust this now, or later.  It's your preference.

Note: base constraint: Level one is set at zero.

OK.

Now choose South elevation to show change in wall height.  It should be 10 ft.

Show how to drag Level Line closer to building from South Elevation….from East too.

Discuss DO NOT EVER EVER EVER DELETE LEVELS!! All the objects assigned to the level get deleted…walls, doors, windows, floors, roofs, ceilings, etc….not good

Back to Floor Plan Level 1 (In Browser)

Select on the front, left wall.  Properties….Wall to: exterior-brick on CMU (concrete masonry unit).  Left select.  Note change? Esc to exit temporary dimension.

### Show your view in more detail

RC in open space. LC on View properties. Display Level: Medium. Now show wall. (Each view can have its own display level: course, medium or fine). EXPLAIN BOX

OK

3D icon (to show a 3-D view). It’s the little house icon in upper left area of screen….

The “View Cube”.

Show the “Steering Wheel”

These are tools to display/view your model…..

Display bar at lower, left corner of screen

Scale

Detail Level

Shading

Explain that these tools control what we see and how we see stuff on THIS view

Note the wall is facing the wrong way. LC on wall. RC. Change wall’s orientation. Should be good now. You can use the blue arrows to flip the walls also. (show in Floor Plan Level 1 View).

Change all the walls so the blue arrows are outside….

Note: The blue arrows should be OUTSIDE to make your wall as it was designed.

Wrong						Correct

Show Properties Display Dialog

You have learned how to create and adjust the size of walls using temporary dimensions, modify the style of the walls, and control the display level of the drawing. You may have noticed that Revit keeps track of your changes in all views, not just the one you made the change in!  All the information is stored in Revit's database.

Save as: revit2018demo1

# Doors in Revit

Back to Floor Plan Level 1.

Architecture.  Door. Notice all single flush doors with various sizes.  Great, but what if you want a different type door?

Load Family.

US Imperial library folder.  See all the content?

Doors folder. Residential

Let's grab a front door: Door-Exterior-Single-Entry-Half Arch Glass-Wood_Clad

Open

Pick the 32X96 and 36X96 Types

Hold down the control key and select them.  Open.

Load Family….Doors….Residential

Let’s grab a garage door: Door-Garage-Embossed_Panel

Open

No choice on Types on this family. 2 came in: 96X84 and 192X84

Load Family….Doors….Residential

Let’s grab an interior Closet door:Door-Interior-Double-Sliding-2_Panel-Wood

Open

Pick the 48X80 and 72X80 Types

Hold down the control key and select them.  OK

Load Family….Doors….Residential

Let’s grab a French double: Door-Exterior-Double-Full Glass-Wood_Clad

Open

Pick the 60X96 and 72X96 Types

Hold down the control key and select them.  Open.

Each of these door types are called a “Family”.  Each family has a set of preselected sizes attached to the door type.  You can set your own sizes at any time, or modify the sizes that were preselected.

Now try the selector.  Notice each family has some size choices, some you were able to pick out of the “Type catalog” and some just came in with the family. Again, you can make your own sizes at any time.

Let's start with a nice front door.  Door-Exterior-Single-Entry-Half Arch Glass-Wood_Clad that is 36 in. by 96 in. Put the door into the front wall near the garage.  Anywhere in that corner area.  Note the dimensions above in blue (temporary)?  Make it 2 ft. 6 in. from the left wall. SHOW WITNESS LINE. Enter.  Now switch the swing of the door, so the door is on the left and swinging into the house.  Notice options.  Notice the door is tagged.  Do you see the tag on placement box above?  Escape twice to exit the command.

Left Click (LC) on the witness line!

Add two garage doors, Door-Garage-Embossed_Panel 96X84 on the left front wall.  Just get them in.  We will adjust them later.  Escape twice to exit.  Pass your mouse over the garage doors.  Do you see the yellow box with the description of the door?

Show Flipping doors

LC on the left garage door.  Do you see the dimensions?  Change the left one to a random size like 7 feet from left to center of door.  Change the right door to the same distance from the right.

Add an interior wall.

Architectural….Wall Architectural….. Interior 5 in. partition (2-hr).  Put vertically somewhere in the middle.  Adjust the wall 15 ft. from the right side by selecting the blue dimension.  Escape twice to exit the command.

The walls Function can be changed after placement…so no worries!!!

Example: Those outside walls we created will be load bearing, exterior 6” walls:

Pick the 5” outside walls

LC (left click) on walls

RC…..select all instances…..visible in view EXPLAIN

In Properties box, Change to Exterior-EIFS on Mtl.Stud

Structural

Check Structural

Structural Usage: Bearing

Click in white space to exit

Add an Opening

Architecture

Component

Place a Component

Load Family….Openings….Passage Opening Cased….Open

Change element Type

Opening Cased 34X80

Place in wall

Show Lineweights ON/OFF

Show Detail Level: Course/Medium/Fine

Place an interior door

Architecture….Door

Single Flush 30”by 80”.  Put it somewhere not in the middle of the wall.

Add a French door.  Door.  Door-Exterior-Double-Full Glass-Wood_Clad 60X96 (yes….I know…it’s an exterior door…just do it!). Make 16 ft. from the other door using temporary dimensions and Witness Lines. Escape to exit.

Remember….when using temporary dimensions….the object you pick is the one that moves!

Check out the doors in 3-D view. Show moving and docking of properties box.

Check out the model by using Orbit tool on the Steering Wheel and move your mouse to spin the model!

Esc

View.

Tile. (top, right corner area)

See the drawings that have been created.

Show Navigation bar with Zoom Tool (top, right corner)

Zoom all to fit

Maximize a window

Show “Switch Windows”…allowing you to quickly cycle between open views.

Show CNTL-TAB Cycling through views technique

You have learned to load a family, control the location of an object with temporary dimensions and witness lines, and rotate the 3D model.

Save as revit2018demo2

# Windows in Revit

Click on Floor Plan Level 1.  Plan view.

Architecture

Window.

Notice that in the Properties box, Fixed is your only choice.  Let's get a variety of windows from the library.  It's the same process, we used when we got the doors.

Load Family.

Windows folder.

Get the:

Arched top with trim

Casement 3X3 with trim

Slider with trim.

Double hung with trim

Open.

Now the four additional Window families are added to your selector.

Note: Windows and Doors are “Wall-based Families”. They can only be placed in a wall. The wall “hosts” (Revit term) the doors and windows. If you delete the wall, the doors and windows go bye-bye too!

Let's put three double-hung Windows 16 in. by 48 in. equally spaced on the East wall.  Check “Tag on Placement”.

Just put them on the wall anywhere!  Ignore the blue (temporary) dimensions.  Escape twice to exit.  Notice they include a Window tag.  Also notice they are not equally spaced. The double blue arrows should be facing outside….just like the walls.

Pick on the North exterior wall.

Note the Modify Walls tab that appears on the pull-downs.

Properties.

See the Location Line: Centerline? That’s because that’s how you drew the wall……

Cancel

Esc

Annotate

Aligned

Do you see the green line in the center of the wall?  Notice the box above that says prefer wall centerlines.  That is the default setting.  Let's change it to “wall faces”.  Now back to wall. Now notice that the exterior of the wall is selected as you move your mouse over it.  Left selected on the top horizontal wall.  Slide mouse down to the center of the first Window, left select on the first Window.  Continue sliding your mouse down and left select on the second Window.  Continue sliding your mouse down and select on the third Window.  Continue sliding your mouse down and select the bottom horizontal wall.  Move your mouse to the right to place the dimensions.

Do you see the blue dimensions?  Permanent (black) dimensions can be locked an unlocked.  See the uneven sizes between Windows? Click on the EQ symbol.  Notice the Windows are equally spaced automatically.  Now dimension, the entire wall by selecting on the top horizontal wall, then the bottom horizontal wall and slide your mouse to the right to place the dimension.  Do see the overall size? Lock it. The contractors would be directed to place the Windows equally spaced on that existing wall.  If that wall changed size, the Windows would remain equally spaced on it.  Escape twice to quit.

Show east elevation by double-clicking on East in the Browser.

Zooming to see trim on Windows. Type SD. See the shade mode? Now type HL.

Show icons on bottom to do the same thing. Show  ….Options…..User Interface…keyboard shortcuts: Customize ….cancel

Changing a window type

Let's say you wanted the middle Window on the East wall to be a Fixed 36 in. by 48 in. instead of the Double Hung Window you already placed….

Left select the center window, you should still be an elevation view, but really doesn't matter what view you’re in.

Properties

Change the Window to a fixed 36 in. by 48 in. on the selector bar.  Left select again over the Window.

Change the Head Height to 8 ft., contrasting the other two Windows.

Place two more of those windows on the North wall like this:

Note:

Instance Properties: just that one 36X48 window (the other 36X48 windows would not change.

Type Properties: ALL of the 36X48 windows would change

Show the Instance and Type property changes on the 16X48 windows

Let's say we want to make the window taller, but not one of the default sizes.  How about 42 in. by 66 in., which is not a default size.

Left select on Window in the center of the East Wall.

Properties

Edit Type

Type Properties box comes up.

Change the Height to 66”

Change the Width to 42”

Discuss changing dimensions here….NO

Fix your mistake by going back in the type properties of the window and set it like this:

Here is the correct way:

Pick the window

Duplicate

Call it SHS4266.  OK.

You'll now make your own size window in the” fixed” family.  Set the width to 42” and the height to 66 in.  Apply.  OK.   Do see the change? Set the Head Height to 8’.

Pick on your new window

Go to the Properties….Fixed pulldown .

Do see the new Fixed:SHS4266 window you just created?

IMPORTANT: don't change the preset family sizes!...  Make your own by the above technique(cloning and renaming).  You don't want to mess with the default sizes. Explain consequences of not doing this correctly.

View the South elevation.  Now go back to the plan view, Level 1.

Architecture

Window

Unselect: “Tag on Placement” on the Option Bar.

Now let's put an arched top 30 in. by 60 in. with trim on the front, right wall.  Notice the Window is not tagged.  Go to the South elevation again.  Do see the Window?

Annotate.

Tag by Category.

Click on Window.  Notice the tag is now on the elevation view.  Occasionally, you'll want to tag your Windows on the elevation view instead of the floor plan view.  Revit allows you do this!

Do see the tag number on the elevation view for the Window we just created?  Go back to the level one view.  Try tagging the same Window.  Notice the same tag number is assigned.

Again, all the information that you're putting into Revit is in a database.  It is all being stored for use later in our schedules, other views, etc.

# Plan Regions: Clerestory Windows

You may run into a situation where you want to see a higher window than the cut plane will see. If you raise the cut plane, you will lose all your other windows. Here is the work-around:

Place a fixed window 16X24 on the South/Front wall. Turn Tag on Placement ON (blue)

See it….YES!

Properties.

Set the sill height at 7’0” …making the head height 9’0”.

See the window  ?

Show South Elevation. See window? Yes. Show 3D view. See window? Yes. Show Floor Level 1. Can’t see the window at all now, RIGHT? Explain why.

Esc

Properties

View Range: Edit

Change Cut Plane Offset from 4’ to 8’

Change Top Offset to 9’

OK…OK

See result? Ouch!!! Discuss.

Change Cut Plane back from 8’ to 4’

Top from 9’ to 7’6”

That didn’t work!

Here is the solution….drum roll please……….a “PLAN REGION”

View

Plan Views

Plan Region

Properties

View Range (edit)

Change Cut Plane from 4’ to 8’

Top to 9’

OK

Create a box (using line tools…top left corner) around the area where the window is placed. They will be magenta.

Finish Edit Mode (see the green check mark)

See it?

(green lines will not print)

Show how to modify the plan region and what to avoid.

You have learned how to use temporary dimensions to control design intent (equally space), create a new size object by duplicating/cloning another, and create a plan region.

Save as revit2018demo3

# Floors and ceilings.

Floors

In the view bar, click on level one.

Architecture

Floor Architectural.

Floor properties option Ribbon is displayed

Pick walls (icon in line tools area).  Double lines are span direction.

You should see magenta lines.

Floor Properties. Note the different “out of the box” floors. We will leave it 12” Generic for now. Show Edit Type

Finish Edit Mode (green check)

Note: Walls are being selected to “Face of Core”

Show in 3-D.

Use View Cube to rotate.

Note: floor could be inside or outside core of wall.

The floor is not really close to being perfect….the garage floor would be lower than the house floor (water), the floor is 12” thick with no material assigned, and the edges of the floor are not correct. We will address this later.

Ceilings.

I want to make a purposeful mistake on the ceiling to show you the ceiling plan dynamics.

Go to Floor Plan Level One.

Architecture

Ceiling….Automatic Ceiling

Let's cover that small 15 ft. area on the right with the ceiling.  Zoom in on the area.

Properties

Let's select 2’X4’ ACT System.

Move mouse over area.  See the red rectangle?

Revit anticipates the area to include the ceiling!  Left select.

OH NO…..ERROR!  “None of the elements are visible in floor plan level on...”  Why?

Because the display representation for a ceiling is looking up, not looking down. Escape twice to exit.

Switch views to the Ceiling Plan Level 1.  There it is!  Now look at it in 3-D view.  View Cube….

Show the view properties/view range for the ceiling view. Get it?

Still in Ceiling Plan level one.  Let's put a ceiling over the garage area, but a Gyp board/metal stud ceiling.

Architecture

Ceiling

Compound Ceiling GWB on Mtl. Stud

Height Offset: 7’

Click Sketch Ceiling icon.

Select chain box.

Use lines to create ceiling.

Don't worry about mistakes. You can fix it later, if necessary.  Stay on the inside of walls.

RULE: magenta (sketch) lines must be “closed loops”.

Finish Ceiling.

Show in 3-D view.

Notice the ummmmm…..design flaw with the ceiling relationship with the garage doors.  Our ceiling is a little low, so let's adjust it.  Select the ceiling in the garage by left selecting.  It should turn blue.

Properties.

Height offset from Level.  It is at 7 ft. Let's set it to 9 ft. OK.

The ceiling should be okay now.  Let's change that ceiling style to a two by four compound, like the other one.  Escape twice to exit.  Show ceiling plan level one.

LC on ceiling.

Properties

Compound Ceiling 2X4 ACT System (some call it a “T-Bar Ceiling”).

The ceiling grids can be selected and moved and aligned to your needs. Show ceiling components and Model Pattern assigned to ceiling.

Select Ceiling

Edit Boundary button.

It should turn magenta.  Select the top horizontal line.  See dimensions and blue dot grips?  You can adjust with dimensions or stretch/change the shape.  Grab the center of the line with the left mouse click.  Hold down the mouse and drag up.  Easy!  Green Check.  Show in 3-D. Show how to change the shape using sketch lines. Show adding a hole in the ceiling.

Let’s see how many drawings have been created.

View Tile….See all the drawings? Same file!

View…Zoom (magnifying glass icon)….….All to Fit

You have learned how to create a floor and a ceiling in “sketch mode”, add a hole in a floor or ceiling, and utilize a ceiling plan.

Save as revit2018demo4

# Roofs

Let’s add a simple hip roof with a 4/12 pitch on this house.

Go to Floor Plan Level 1.

Architecture.

Roof.

Roof by Footprint.

Lowest level notice box comes up.  It's warning you to move the roof to level two.  Good idea…otherwise, the roof would start on the ground!  Click yes.

Let's set the overhang to 1’0”.

Uncheck extended into wall.

Check Defines slope

Pick Walls button.

Properties

OK. Pick the exterior walls you want to cover with roof.  The green hidden lines represent the overhang.  The magenta lines are the roof.

Highlight all of roof

Properties

Change the Slope to 4/12.

Edit Type

Change to Wood Rafter 8” Asphalt Shingle-Insulated. A bit smarter than the Generic 12.

Finish Roof.

No roof?  Now select Level 2.  Some roof, but cut off.  It should look like it got sliced horizontally.

It did! Explain 4’ cut plane at Levels. Now try Site Level.  Complete.  That level shows the complete roof (it’s view range is 200’ up).  Show in 3-D.

How to change our roof pitch in Revit.

Floor Plan Site

LC on Roof.  It will turn blue.

Edit Footprint button.

You'll see purple lines with some triangles near them.

Window to select all of it.

Properties.

Adjust the rise to 12” and overhang to 2’.

Green Check

Finish Roof

Change from hip to gable roof.

There is not a “button” to change a roof from one type to another. Maybe in the future??

LC on roof in 3-D view (you could do this on Site level too).  Roof should turn blue.  Let's make that right side hip where the three windows are, into a gable.

Edit Footprint.

You should see purple slope arrows and lines.  Pick the LINE (not arrow)one over our wall.

Uncheck defines slope.

Leave extend into wall off.

Green Check

Notice the air conditioning element!!!

Let's do the other side, over the garage….this time in Site view

LC on roof.

Edit Footprint.

Pick line over garage doors.  Uncheck both defines slope and extend into wall.

Finish roof.

3-D rotate.

More air-conditioning!!! Let’s fix that.

Select the garage wall.

Attach: Top/Base

Pick Roof.

Done!!

Do the same with the East wall.

Note: Windows can go in that gable wall area

Show elevation view, to demonstrate the applied roof pitch.

Change it back to 4/12 pitch and 1’ overhang.

Change from a gable to a shed

Let’s put a shed roof over the front door (off the front wall).

Floor Level 2

Put your Display settings on “Wireframe” so you can see the walls below the roof.

Architecture

Roof

Roof by Footprint

Properties

Base offset from level: -1’6”

Roof Type: Wood rafter 8” asphalt…

OK

Lines: Rectangle tool

Uncheck defines slope

Create rectangular sketch for roof

Esc…Esc

Pick front horizontal line

Check defines slope

Set pitch to 1/12

Green Check

East Elevation

Notice our shed roof is a little too high? (front was base height)

LC on roof

“Bump” (Revit term) it down to desired height using the down arrow key. Bump it down to -2’0”

Roof gutters

You can add roof gutters very easily in Revit.

Site

Architecture

Roof

Roof: Gutter.

Pick the roof edges, in which you want to place roof gutters.  Click into white space to exit the command.  Show in 3-D.  Show south elevation.  You should see the gutter clearly in both views.

You can change/add the roof fascia in the same matter.

You have learned how to create a hip, shed, flat and gable roof, change the pitch and overhang, add gutters, and change the physical properties of the roof.

Save as Revit2018demo5

# Putting your work onto paper in REVIT

The Revit title blocks are under SHEETS in the Project Browser.

Pick Sheets (all).

Right-click.  New sheet.

There is a default E1 30X42 Horizontal paper size loaded.

We want a different size….. pick Load….

Imperial Library

Title blocks.

D 22X34 Horizontal.

Open.

Now you have two choices.  Pick the D 22X34 Horizontal size sheet.

OK.

Let's put the four elevations on the sheet.  Let's select East elevation.  It should be highlighted.  Drag it into the sheet by holding the left mouse button down, then releasing it in the sheet.  Move the view to where you want it on the sheet.  Left select.  Pick outside to exit.  Zoom in to see your work.  Repeat the process for the other three elevations.  Notice the elevation marks and the labels.  All automatic!  You can zoom and pan without losing your scale.  It's all tied together in a database.

Try this.  Go to your South elevation in the Sheet.  Let's make the Level lines a little shorter (horizontally).

Show Activate View….Deactivate View

Activate the East Elevation. Drag the level lines closer to the building

Notice the West line moves too!!! They are “planes”

Continue the process until all the Level lines are closer to the building.

Show how to shorten the Title bar.

Naming Sheets

Zoom in on the lower right corner of the title block (showing the sheet name and number)

Go back to Sheets in the Project browser.

Double-click on Sheets.  Pick A101-unnamed.

Right-click.  Rename.

Name the sheet EXTERIOR ELEVATIONS. Give sheet number A501 OK. Notice the sheet updates automatically!

See the + sign next to A501-EXT. ELEVATIONS. Click on it.  See the four elevation links?  All organized, all in the database.  Double-click on the little icon next to elevation east.  Note that it takes you to the East elevation.  Also notice the East elevation is highlighted.

Go to Floor Plan Level 1. See the elevation tags? They update as elevations are placed on a sheet. Move them a little closer to the building.

Show Blue Elevation line and issues with it being in the building

Dimension scale will not be an issue in Revit. YEA!

Let's make a sheet for floor plan one.

Click on Sheets.

RC

New sheet.  Let's take a D size sheet again.  OK.

Pick Level 1 in Floor Plans.  Drag in.  Done.

In Title Block…. Rename to A201-FIRST FLOOR

Note the project Browser updates automatically!

Select in viewport

RC….Activate View

See bar at bottom?

Change scale to 3/16”=1’0”

RC…Deactivate View

Move to center

Note Title Bar Scale!

# Sheet Naming Convention

C100.00 GENERAL/ SITE

EX: COVER	C101

SITE	C102

SITE DETAILS

COVER SHEET

DRAWING INDEX

A100		SLAB/FOUNDATION

A200		FLOOR PLANS

ENLARGED FLOOR PLAN AREAS

TYPICAL ROOM PLANS

ELECTRICAL

A300		RCP (CEILING PLANS)

CEILING DETAILS

A400		ROOF

A500		EXTERIOR ELEVATIONS

ELEVATION DETAILS

ENLARGED ELEVATIONS

A600		BUILDING SECTIONS

WALL SECTIONS

DETAIL SECTIONS

A700		INTERIOR ELEVATIONS

FINISH SCHEDULES

MATERIAL SCHEDULES

INTERIOR DETAILS

A800		DOOR/WINDOW SCHEDULES

A900		MISC

You have learned how to place views on a sheet, activate a view within a sheet, properly name a sheet, and utilize sheet naming conventions.

Save as revit2018demo6

# Section views in Revit

You can easily create section views in Revit.  The views are tied to the building model, so if the model changes, the section will change along with it.

Floor Plan Level 1

First let’s put a couple of cars in the garage.

Architecture

Component

Place a Component

Load Family

Entourage

Pick the RPC Beetle ….don’t pick the semi-truck…it won’t fit in the garage

Open

Place 2 Volksvaagons in the garage.

Show how to align the VW’s with the center of the doors

Modify

Align

Target

Source

Creating a Building Section

View.

Section.

Properties

Building Section.

(Show Wall, Detail too). Use pen to place section view. Drag the section line horizontally thru garage, as if you were cutting the VW’s in half.

The blue arrows flip the viewing direction.

Click in the white area to end the command.  You just created section one.

Now go to sections in the project browser.  Double-click on sections.  See Section 1?

Set the display level to Medium. See the structure of the roof and walls? Discuss

Change crop region so front door does not show, just the garage. (show options)

Show RC…GO TO VIEW from Section line

The following steps will keep the project browser “clean” and easy to track views:

In the Building Section View

Propeties… under Identity Data…..Title on Sheet….

Name it SECTION THRU GARAGE. This changes the title without renaming the view in the browser.

Let's make a new sheet for building sections.  Sheet A600 is the proper designation for building sections.

Drag the sectional view into the building section sheet A601.  Notice the section view is numbered, corresponding to the section view in the floor plan.  They are linked. See the name SECTION THRU GARAGE?

Show how to activate the view and change the scale on the sheet. Change it to: 1/4”=1’0”

Show how to turn off crop box (hide crop region icon at bottom of screen).

Deactivate view.

Let's make a new sheet for wall sections. A600 is the proper designation for wall sections. Name the new sheet WALL SECTIONS and give it a sheet number A602

Create a Wall Section.

Level 1 Floor Plan

View

Section

Properties

Choose “Wall Section” from bar

Go through the small window we used for the “Plan Region”

.

Adjust the section region box to be about 2’0” deep

Note Project Browser now has Wall Sections!!!

Note: The Wall Section has a different arrow (unfilled) than the Building Section….

LC on the Wall Section line

RC….. Go to view.

Change the Display Level to “Medium”

Change the scale to ½”=1’0”

Drag that view on Sheet A602.

Create another wall section through the garage door.  It should be Section 3.

Change the scale to ½”=1’0”

Drag that view into sheet A602. The Revit scale change selector at the bottom of the screen…very easy to set on the fly!

Turn off the Crop boxes on both views.

Go back to floor plan one.

Note the Building Section tags are populated SMARTLY with data pointing to the sheet the view is on.

Now Erase the window that section two passed through.  Now go back to sheet A600.02. Notice that the window is now gone.  Once something is erased anywhere in the building model, it is completely erased in the entire database in every view. That would include door, window schedules as well. Undo the deleted window. See it back in the section view!!

You have learned how to create building and wall sections, describe a view without changing the browser, and change the scale of a drawing in the sheet.

Save as Revit2018demo7

# Furniture, fixtures and viewing in Revit

Floor Plan Level 1

Let's try furniture first.

Architecture

Component.

Place a Component

Load Family.

Furniture.

Beds

Bed-standard

Open

Properties

Notice you have 4 TYPES of beds, King, Queen, Double, and Twin. We’ll talk more about these “types” later.

For now, Let's place a king-size bed in that 15 ft. wide area, against the North wall.  Notice the dimensions help you control placement, as you place the bed.  Escape twice to exit.  Left select on bed.  Do you see the dimensions?  You can adjust the distance the bed is from the walls.  Show SPACE BAR will flip/rotate the objects

Show location and loading of Furniture families

Fill the room with furniture. Desk, TV, Dresser, sofa…up to you. Just populate room so all 4 walls have furniture adjacent.

Look at it in 3D

Pick Roof…VH

Pick Ceiling…VH

Use Orbit Tool to see how your furniture went in. Any luck??? Need to rotate

Pick the furniture piece…..space bar

Back to Floor Plan Level 1

Move furniture as needed

Back to 3D

VV….

Check Ceilings and Roofs

More to come very soon on the visualization stuff!

Where do I find all those cool 3D objects?

Using the same process as the furniture, here is the location(s) of some common objects:

Explain what Revit is giving (partial cabinets) and why countertops are not on casework.

Kitchen and Bath Cabinets

Component. Place a Component. Load Family. Casework.

Plumbing Fixtures (sinks, tubs, toilets, water heater, etc)

Component. Place a Component. Load Family. Plumbing, Architectural. Fixtures

Specialty Equipment (washers, dryers, oven, range, dishwasher, microwave, etc)

Component. Place a Component. Load Family. Specialty Equipment. Domestic.

Lighting Fixtures (lamps, ceiling lights, sconces, spotlights, etc

Component. Place a Component. Load Family. Lighting. Architectural.

Openings (cased, elliptical, wall openings)

Component. Place a Component. Load Family. Openings

Electrical Fixtures (panels, outlets, switches)

Component. Place a Component. Load Family. Electrical. Architectural. Electrical Power

Sustainable design (bike racks, solar panel, light shelf, recycle bin, etc)

Component. Place a Component. Load Family. Sustainable design.

Discuss 3rd party downloadable families Pro’s and Con’s:

Revit City (free…but sometimes you get what you pay for)

You must join in order to download families. To save valuable lab time, you could download families at home…then bring them to class on a USB drive. You do not need Revit at home to access Revitcity.

http://www.revitcity.com/index.php

Autodesk Seek

Free content from Autodesk’s site. Good place to find families.

http://seek.autodesk.com

Insert….Load family…..

# Controlling what you see

Revit allows you to hide object and categories. You can hide them so they don’t print, or just temporarily to see other things. You can hide a single object without hiding all of the same type of objects. You can make objects transparent.

Place your 3D view on a sheet.

Title: 3D Views

Sheet Number A901

Be in 3D View

Temporary hide

Let’s say you want to hide a roof portion (not all of the roofs) in the 3D view so you can see the ceilings below it….

Select the roof to hide

Sunglasses

Hide element

Pick the ceiling toward East

Hide Element

Pick the left window on the East Wall

Hide Element

See the Cyan box??

Sunglasses

Reset Temporary Hide

They’re all back!

So the Sunglasses hide stuff temporarily…..you can pick a bunch of different objects and temporarily hide them….then turn them back on with one “Reset Temporary Hide”

Hiding an object permanently

Let’s say you want to hide the same roof section in the 3D view so you can see the ceilings below it, but not all the roofs:

Select the roof to hide

RC

Hide in view

Element

Light bulb

See red box? All red objects are hidden in this view. The “Analytical Nodes” you can leave them alone.

Select the red roof

Unhide Element (icon is at top, right corner)

Light bulb

Roof should be back

The light bulb allows you to turn off individual elements without turning off the whole category. You can only turn them back on with the light bulb. VV does not work.

Let's say you want to freeze the entire roof and the ceilings in the view.

Go to the 3-D view.

Properties.

Visibility Graphics Override.

Edit

Uncheck Roof and Ceiling.

OK.

Go back to floor level two, look at it.  Do the same with Ceiling Level 1. Do the same with Site. Notice the roof and ceiling are ONLY frozen in the 3-D view, where we turned it off. Revit freezing is VIEW SPECIFIC!

Pick on The Building Sections sheet A600.01.  Notice what's in the garage.  Everything is linked in the building model!

Go back into the 3-D view, in turn the roof and ceiling back on. Use the VV (shortcut)

To hide a type of object in a view, Use the VH (shortcut)

# More Sections

Flipping the section.

Floor Plan Level 1

Pick on the section Arrow.  Right-click. Flip section.

See the Double-blue (FLIP) arrows? Pick them…same effect!

Breaking the section lines

Select on a Building Section line.  See the little blue Z in center?  Pass your mouse over it.  See the “gaps in the segments”?  Left click on Z. Should have broken section line into two parts. You can drag down the blue dots/grips to show the part of the section line you want.  Want to join the section line into one again?  Select the line…Pick Z again. See it join?

Offset section

Place a 36X48 window on the East and West wall of the Garage, not aligned horizontally.

Offset section: Pick section line. Split Segment button. Pick the part of the line you wish to move. Slide mouse up/down to change location of section. Note: You can only go as far as the crop region boundary (green lines) on the split section.

Changing the section Head/Tail

Select on section line near section marker. See the blue circle with arrows near the section marker? Pass mouse over. See “cycle section head”? Pick on it. Pick again. You are toggling between 3 appearances: no line, thick line, and arrow.

Repeat for “tail” (other end of section line).

You have learned how to find components in the program and via external websites and control the visibility of objects in the view.

Save as revit2018demo8

# Interior Elevations

This is a neat one. Let’s set up some interior elevations in a room.

View (pulldown)

Elevation…Elevation

Properties

Select Interior Elevation

Place elevation marker inside room of choice.

Notice as you near a wall with the marker, the marker points to that wall. Pick a wall of choice so the marker points to it. Notice new interior elevation is created (in the project browser under elevations).

Double click on the interior elevation in the browser to see it.

Crop the box by picking the blue arrows (see above) and dragging your mouse.

See the blue line? That is where you are standing. Just like a Section line. In fact, elevations and sections are pretty much the same (in the way they work) in Revit. Discuss

Place the Elevation on a sheet.

Name the sheet INT ELEVATIONS A701

Back to Level 1

See the information in the elevation symbol? Page number…elevation number…

Here is a neat feature. Pick on the elevation marker.

Notice it changes to a diamond. See the blue boxes? You can check them to set up more elevations! Check them all. As you are checking them, notice in the browser, under elevations, new elevations are being formed. Drag the new elevations onto a sheet.

Getting rid of the Elevation Mark text

The Floor Plan is a little cluttered. A little trick….

Manage

Settings….Additional Settings

Elevation tags

½”Circle

Elevation Mark

Chose Elev Mark Body_Circle: Filled Arrow

OK

A little better?

Drag the other 3 interior elevations on sheet A701

That is Revit’s database being updated….it’s all tied together! If you add content later, EVERYTHING UPDATES…automatically!!!

You have learned how section and elevation annotations work, going to a view from the annotation symbol, creating Interior Elevations, and how the browser updates smartly as views are created.

Save as: revit2018demo9
Adding Multiple Floors in Revit

Add some windows on the South, North, and West Elevations. Different types…Explain that we will use them for schedules later.

3D View

Delete Roofs (we will be adding floors, so the roof will be higher)

South Elevation

Architecture

Level

Check Make plan View

Create 5 more levels…10’ apart….to Level 8

Show “Offset” strategy

Turn on Level markers on left side

Rename Level 8 to “Roof”	say YES to box asking about renaming.

3D View

Select the front wall (with entry door in it)

RC

Select all instances…in entire project (discuss)

Modify Walls (Tab)

Under clipboard: Copy

Pick in white space to exit

Modify

Paste

Aligned to selected levels

Choose 2,3,4,5,6,7

OK

Notice change?

Discuss what would happen if we copied to Level 1

Select CMU wall (the one with the garage doors)

Properties

Change top constraint to “Roof”

OK

Note: They don’t really build walls 70’0” tall as a single wall, but you can model it that way.

Notice the Garage doors did not copy up like the doors and windows in the last step. Prpbably not needing Garage doors at Level 6 .

Windows on this wall would be placed on Floor Plan levels

Floor Plan Level 2

Place two Archtop 30X60 windows with head height of 8’0”on the CMU wall.

South Elevation

Dimension from exterior wall to center of windows (use wall faces)

EQ

Discuss “look”

Try aligning windows with the Align tool.

Modify….Align

How does it look? You pick your look.

Select the two windows (use Cntl key)

Modify Windows (pulldown)

Clipboard…Copy

Paste

Aligned to pick level

Pick Level 3

See it? Paste

Aligned to selected levels

Select Levels 4-7

OK

See in 3D

Those lines between the floors look bad!?

Modify

Geometry

Join

Join Geometry

Select Level 1 south elevation wall (w/ front door)

Select the wall directly above

Notice the line goes away

Repeat this process for the other walls. DISCUSS JOIN GEOMETRY

Again, better to plan ahead on the exterior walls if you initially know how high they will be.

One last task.  Let's look back at the Exterior Elevation A501 sheet.

Whoa!! Discuss

We will have to make some new sheets!! Put the North and South Elevations on a sheet….and put the East and West Elevations on a different sheet.

Now look at the Building Section A601 sheet.  Activate the view. Turn on the crop region box. Pull it up to expose the ENTIRE Building Section. Turn off the crop again. Deactivate the view.

Notice how they were updating with us all along.  Building information modeling (BIM) in Revit allows us to focus more on our design, and less on coordinating our construction documents.

Show how to delete views on a sheet, create a new sheet, then add view again.

Try using same sheet number….note it won’t let you!!

You have learned to create multiple levels, strategies for multi-level buildings, and leveraging information from one floor to another.

Save as revit2018demo10

This file is now about 6 Megs as we created it. You should save the file on the hard drive and back it up with a USB drive Also, Revit is set to save backup copies of your saves, very much like AutoCad’s .bak system. You may want to set the number of backup copies to 3 so your hard drive doesn’t get filled up.

Explain the name.0001 and name.0002 back-up system in Revit

Note Revit2011demo20test2 is the most current

Note the files with the .000X are BACKUPS! If you open those, you will be getting an OLD file!

Note .0002 is more current than .0001. The bigger the .000X, the more current the file…BUT….still not the MOST CURRENT.

Controlling Number of Backups

File

Save As

Options

See Maximun backups? The default is 3.

# Revit Schedules

Door Schedule

You can be in any view in the Browser….

View

Schedules

Schedule/Quantities

Doors

Take defaults in New Schedule box

OK

We will set up the schedule to have the following columns:

Family…Add. If it’s on the right, the category will be in the schedule.

Mark (use Cntl key to select more than one at a time)

Finish

Frame Material

Head Height

Height

Level

Thickness

Type

Type Comments

Width

Add

OK. Note what you have.

Let’s also add a few of the Green Building Parameters

Fields

Edit

Heat Transfer Coefficient (U)

Solar Heat Gain Coefficient

OK

Discuss garbage-in/garbage-out analogy.

Add frame material to Door #1

Name it PAINTED PINE

Finish: OAK STAIN

Type comments: LOW VOC RATED

Level 1 Floor Plan

Select door 1

RC

Element properties

Instance Properties

See Materials and Finishes?

Database!!

See Identity data?

Yup….Database!!

Cancel…Cancel

So you can (carefully) modify the information an object carries in the schedule too!!

You can give custom/company standard titles to your schedule columns:

RC on Door Schedule in Browser

Properties

Formatting (tab)

Rename Fields (Formatting)

Family					Door Style

Finish					Finish

Frame Material			Fr. Mat

Head Height				Head Height

Height					Height

Level					Floor #

Mark					Door #

Thickness				THK

Type					Door Type

Type Comments			Notes

Width					Width

Heat Transfer Coefficient (U)		U-Value

Solar Heat Gain Coefficient		SHGC

Change the order they show on schedule to this:

You must be in the fields tab to move the fields up and down…..

Mark						Door #

Family						Door Style

Type						Door Type

Level						Floor #

Thickness					THK

Height						Height

Width						Width

Head Height					Head Height

Finish						Finish

Frame Material				Fr. Mat

Type Comments				Notes

Heat Transfer Coefficient (U)			U-Value

Solar Heat Gain Coefficient			SHGC

OK

Make a new sheet A801. Name it DOOR & WINDOW SCHEDULE

Bring into sheet

Stretch to make columns look “proper” DISCUSS

Discuss deleting, changing sizes and types in the schedule…the effect on the building model.

Windows Schedule

View

Schedules

Schedule/Quantities

Windows

OK

We will set up the schedule to have the following columns:

Count

Family

Head Height

Heat Transfer Coefficient

Height

Level

Solar Heat Gain Coefficient

Type Comments

Type Mark

Width

add

OK

Go ahead and type “Paint White” in the first Type Comment.

Say OK to the box…DISCUSS Type Properties

Drag the Window Schedule on the Door/Window Schedule sheet.

Show break to split schedule. Show how move icon puts it back.

Show schedule properties to adjust the database and appearance of the schedule.

Change the fields to the following names and put in this order:

Count						QTY

Type Mark					Win #

Level						Floor #

Family						Window Style

Height						Height

Width						Width

Head Height					Head Height

Heat Transfer Coefficient (U)			U-Value

Solar Heat Gain Coefficient			SHGC

Type Comments				Notes

Stretch the schedule.

Notice the QTY is not working so well….all 1’s???

LC on schedule

Properties

Sorting and Grouping

Uncheck itemize every instance Discuss

Sort by:

Type Type Mark…. then by Level

Switch them

Adjust the schedule on the sheet, controlling row height and length.

Grouping Schedule Parameters

You can group Parameters within a schedule.

In the Door schedule, we will group the “Dimensions”

LC in the THK cell…on the left. You will drag your mouse to the right. If you see the cells fill in black, you are doing it right.

Group

Discuss copying schedules depending on the use or development of the project and using Transfer Project Standards to reuse settings.

You have learned to create a door and window schedule, change its order, fields, filter, and sort. You learned the schedule is directly linked to the building model….if it changes in the model, it changes in the schedule….if it changes in the schedule, it changes in the model.

Save as revit2018demo11.rvt

# Controlling what you see (More)

Note: The most challenging part of learning Revit is figuring out the various ways people can hide stuff from you! Really. Making walls…easy. Dimensioning…easy. Finding that missing lighting fixture….sometimes…not so easy.

The solution? Let’s talk about the ways people can hide stuff from you….and why they use what they use.

You get this, Revit gets A LOT easier!

First:

Revit does NOT have layers.

Revit has OBJECTS.

When you hide something in Revit, it is VIEW SPECIFIC (it only goes away in that view)

OK…here we go….

View Visibility (VV)

Floor Plan Level 1

VV

See Model Categories?

These are your (3D) objects. The checks mean they are ON.

See Annotation categories?

These are your 2D objects. Get it?

Pick a window (any window)

VH (View Hide)

See how the windows are gone? The hole in the wall are still there because the wilnows are still there…just hidden!

3D View

Note windows are there.

South Elevation

Note windows are there too.

Again…when we turn something off in Revit, it is only in THAT VIEW.

To turn the windows back on?

VV

Check Windows.

They are back!

VH turns off ALL of the elements of that category (in this case, all of the windows in the view)

Sunglasses

Sometimes we just want to hide a bunch of elements to see something hidden…..but want to put it back quickly.

The sunglasses are a “temporary hide”

Show use of Sunglasses

Select 1st floor east wall

Sunglasses

Hide Element

Select a window on that east wall

Sunglasses

Hide Element

See the Cyan box? You know by that you have Temporarily Hidden Objects!

How to put it back?

Easy!!!

Sunglasses

Reset Temp Hide

Done!

You can isolate elements with the sunglasses

Pick on the CMU wall

Sunglasses

Isolate Element

Only the wall shows!

How to put it back?

Easy!!!

Sunglasses

Reset Temp Hide

Done!

Light Bulb

You can hide an individual object in a view, without hiding all of the same objects

Ex: Hide specific pieces of Furniture without hiding all of them

Floor Plan level 1

You want to see the Bed and Dresser, but not the other furniture

Select the furniture you want to hide:

RC

Hide in view

Elements

Should be gone!

Get it back?:

VV

Oh no…the furniture is still checked???? So how do we get it back????

Light bulb

See red?

Select object(s) in red

“Unhide Element”

Light bulb

See it come back?

Override Visibility

Let’s say the pattern on your floor is overwhelming your floor plan….you generally do not show floor patterns on a Floor Plan…there is a different drawing for that.

3D

Pick the floor

Change the Element Type to Wood Joist 10” Wood Finish

Back to Floor Level 1

See the floor pattern? It’s gotta go!

VV

Floors

Under Projection/Surface

Patterns

Pick in white box

Override

Uncheck “visible”

OK…OK

Floor material is gone (will not print), but it is still there…will tag…will be in all other views.

Transparent

Objects can be made Transparent.

Go to a 3D view

VV

Walls

Under Projection/Surface

Transparent

Pick in white box

Override

Use slider to dial in the transparency

Try about 70%

See thru the wall?

Undo that…

½ Tone

Objects can be made Lighter on a view.

Go back to the floor plan view

Turn Floor Pattern back ON

Floor pattern back on.

VV

Floors….Patterns….Override….Visible…OK….OK

The flooring pattern should be back

Scenario 2: You need to see the flooring in the view, but it overwhelms the drawing:

VV

Floors

Check “halftone”….OK

Notice the floor pattern gets lighter?

Crop Region

Go to an elevation view

Show Crop region icon (just to left of sunglasses)

Show how to turn ON OFF

Show how to “Crop” View

Show what happens when View is Cropped and Crop box is off

See how easy it is to hide something without an obvious way of finding it???

Show Do Not Crop

Back to Floor Plan level 1

Select the arrow on elevation marker

See blue line?

Show Crop Region

Get it?

Hide Crop Region (to left of sunglasses)

When an area or object is not showing, it may not be within the crop region!

Select the arrow on south elevation marker again

See blue line again?

Move the blue line into the garage

Go to South Elevation

Notice something is missing. This happens once in awhile….something to look for when an elevation isn’t looking right….

Put the blue line back outside the building. And yes, elevations and sections are essentially the same in Revit.

Underlay

A common visualization complain is: “why am I seeing my 1st Floor walls on my 2nd Floor?

Revit has the ability to show another level (grayed-out) on another level. It’s called an underlay.

The solution:

On the 2nd Floor

See the interior wall…ceilings….grayed out….

That is an “underlay” of Level 1 Floor Plan

LC in white space

Properties

Underlay: none (it would be set at Plan…Level 1 to show the Level 1 graphics)

OK

Underlays are a great tool to borrow locations of things and apply them to your level.

View Range

It is possible that you are not seeing your entities because the entities are out of your view range.

Example: The sill of a 2’ high window is at 16’ on a 20’ high wall. On your floor plan, the window is not showing. Why?

The View Range might not cover that high window.

On the 1st Floor

LC in white space

Properties

View Range

Edit

Discuss Primary Range, Cut Plane and View Depth (below primary range)

Model Categories vs. Annotation Categories

I can’t see my object! A familiar issue with Revit. Things get turned on and off…. and frequently, you have to find them. Here is a simple rule for narrowing down your search:

VV or VG (you do not hit ENTER after typing the 2 letters in Revit)

Model Categories: If the item you are looking for can be seen in a 3D view, it is in Model Categories. Walls, doors, lines, components, stairs, etc. The parts of the category are in subfolders…if you need to hide/unhide something like surface patterns and lines in the object.

Annotation Categories: 2D stuff like tags, dimensions, elevation marks, levels and section marks.

Imported Categories: Usually AutoCAD drawings that were imported into Revit. You will be able to turn layers on and off, as well as hide entire drawings.

Hiding object types quickly

VH will turn off an object type in one step. SHOW

# Smart Wall Creation

We will create a simple 2X4 wall, then a plumbing wall 2X6

Select the Interior 5” partition 2-hr wall

Edit Type

Duplicate

Name it: SHS 5/8"3.5core5/8"

Structure: Edit

Make it look like this:

Note a 2X4 is not really 2” by 4”!! It is 1-1/2” X 3-1/2”. I put the Gyp on Fin 1 (4) to make wall joining less of a problem.

OK…OK

Now we will make a plumbing wall/wet wall Plumbing walls have to be wider because some of the pipes can be wide and must be held down. A 2X4 wall is only 3-1/2” wide, so generally the pipes will not fit!

.

Select the SHS 5/8"3.5core5/8"

Edit Type

Duplicate

Name it: SHS 5/8"5.5core5/8"

Structure: Edit

Make it look like this

Save as revit2018demo12

# Details in Revit

We will create a Building Section, followed by a Wall Section callout, then a Footing Detail from the wall section.

Floor Plan Level 1

View

Section

Building Section

Create a Building Section horizontally thru center of building. Crop about 2 feet past section line. Set the scale to ¼” = 1’0”

Should be the second Building Section in the browser.

Go to that Building Section. Call it Section 4

# Smart Wall Creation Continued

Before we start, let’s make “smart walls” out of the East Elevation walls:

Select the lower east wall (with the three windows on it).

Properties

Edit Type

Duplicate

Name it:SHS1STUCCOVR1/2PLY5.5CORE5/8GYP

Structure: Edit

Click preview pane to see wall being formed

Build it as follows:

#	Function		Material		Thickness

1	Finish 2	Stucco				1”

2	Membrane	Vapor Retarder		0

3	Substrate 2	Wood Sheathing Plywood	1/2”

4	Core Boundary				0

5	Structure	Wood Stud Layer		5.5”

6	Core Boundary				0

7	Finish 1	Gyp Wall Board		5/8”

The wall should be 8-5/8”

Stud Material

Assign: Wood…Timber C24…up arrow

Done….OK

Clapboard

Assign: Wood…Siding Clapboard…up arrow

Double click on picture

Set color to purple

Set pattern to Model Pattern 4” parallel

Done….OK

OK….OK

Set Detail Level to Medium

Change wall orientation if necessary.

View

Thin Lines

See the components of the wall clearly now. Some people line to work this way when detailing.

Change the 2nd floor wall to 5/8GYP5.5ST5/8PLY2CLAP

3D

Show Match Type (paint brush) icon to do other walls above

Modify

Match Type

Pick Source wall

Pick target wall(s)

You may still have to change walls orientation….

Cool!!

Next, let’s put a floor on the 2nd Floor:

Floor Plan Level 2

Notice we can see the Level 1 stuff on the Level 2 floor. Here’s why….

UNDERLAY Explain

Set Underlay: None

Architecture

Floor

Floor Architectural

Properties

Choose 10” Wood Finish

Lines

Make floor inside walls (miss on purpose)

Use align command to make accurate

Finish Floor

Say No to message connecting walls to Floor Pick Detach Target(s) DISCUSS

See pattern?

You may have to set scale of Level 2 to ¼”=1’0” to see Section 4.

Back to Building Section

See the floor?

See the interior wall not stopping at the floor? Drag the wall down to the floor.

TURN OFF FURNITURE AND FIXTURES IN SECTION VIEWS

You can VH them.

Explain 2D overlay of the 3D geometry. Relate file size.

Hint #1: The more you create in 3D, the less “overlay” you have to create in 2D.

Hint #2: You will have to make decisions on what to create in 3D and when creating in 3D is just too time consuming and not cost effective.

Note: Your 2D overlay geometry will not show in other views (ie:the 3D model)

Save as revit2018demo13

# Smart Brick Wall

Smart Stone Wall

# Rooms

Floor Plan Level 1

We need to create some rooms in the plan:

Don’t worry about size….

Those words are just text in this example…..don’t add those yet…..

We need a 3 Bedrooms, 1 Bathroom, Living, Family, Foyer, Kitchen, Dining, Garage, and Hall

Leave some open plan rooms

Architecture

Room

Room Tag with Area (in Properties)

Pick inside each enclosed area

You should have rooms with a random number and the Area.

Name the rooms by double clicking on the name “room” and renaming it something logical.

Nice???

What about the area not bound by walls??

You will subdivide the space with a “Room Separator”

Architecture

Room Separator

Room separation: show

Create new Rooms in the new spaces

Pick inside rooms w/ room separation

Name the Rooms

You can VH the room separation lines.

Save as revit2018demo14.rvt

Color Fills/Schemes

Have Floor 1 ready with rooms.

Pick first room…..a Bedroom (not the room tag) move mouse until the X shows

Properties

Notice the room name is there

Note: We will use the “Department” parameter to organize our room types

Department: SLEEPING	OK

Pick a Bathroom

Properties….Identity Data

Department: SERVICE	OK

Pick a Living

Properties

Department: LIVING	OK

Pick Family

Properties

Department: LIVING (pick arrow so you don’t have to type it again)	OK

Pick a Bedroom 2

Properties

Department: SLEEPING	OK

Pick a Bedroom 3

Properties

Department: SLEEPING	OK

Pick the Hall

Properties

Department: CORRIDOR

Pick the Foyer

Properties

Name the room FOYER

Department: CORRIDOR

OK

Pick the Garage area

Properties

Name the room GARAGE

Department: NON-LIVING SPACE

OK

ESC twice to exit

Before we create the color scheme, we will DUPLICATE WITH DETAILING the Floor Plan Level 1.

DISCUSS STRATEGY ON DUPLICATING

RC over Level 1 in Browser

Duplicate View

Duplicate

Rename the Plan: Level 1 Presentation

Color Fill Presentation

In Level 1 Presentation

VH any annotation

Leave the Furniture and Fixtures in (if it is still there)

Annotate

Color Fill Legend

LC in screen

Space Type: Rooms

Color Scheme: Dept	OK

NEAT?!

Shade Walls

VV

Walls

Cut Patterns

Override

Pattern: Solid Fill

OK….OK

Changing the Color Scheme

LC on the Legend

Edit scheme

Discuss Duplicating the scheme

Duplicate the scheme

Name it “Shell Color Scheme”

Create new colors for the scheme….keep them light please….inkburners….

OK

Swap between schemes

You can change/create schemes by: 	Area

Base Finish

Ceiling Finish

Department

Floor Finish

Name

Number

Occupancy

Occupant

Perimeter

Wall Finish

Try erasing room separation line: See error? Explain

Delete room. See?? Undo the deleted room.

You have learned to create rooms, separations, apply properties to rooms, and create a color fill.

Save as revit2018demo15.rvt

# Dimensioning in Revit

Floor Plan Level 1

Annotate

Dimension (arrow)

See Types:

Linear Dimension Types

Angular Dimension Types

Radial Dimension Types

Diameter Dimension Types (new to Revit 2018)

SHOW PUSH PIN

See buttons:

Aligned

Linear

Angular

Radial

Diameter

Arc length

We will customize a Linear Dimension Type

Properties

See loaded types

DO NOT CHANGE EXISTING STYLE!!!!

Edit Type

Duplicate

Name it “Bills 3/32 Arial

OK

We are now working on the “clone”

Units Format (pick shaded box)

Uncheck “use project settings”

Rounding:  To nearest 1/4” (rounding can cause errors….use temporary dimensions to make sure components are placed to ¼”).

Check suppress 0 feet

OK

Centerline Pattern: Center ¼”

Centerline Tick mark: Diag 3/64

Tick Mark: Diag 3/64

Dim line snap distance: 1/2"

Tick Mark line weight: 7

Lineweight: 1

Text Offset:1/32”

Width factor: .8

Notice text size is 3/32. We’ll take that

Notice text font is Arial. We’ll take that too.

Apply

OK

NOTE: If the Type Properties dialog box is being “uncooperative”, drag the top of the box (between Parameter and Value)to the left to make it stop flipping!

Dimension

Aligned

Change element Type

Select Bills 3/32 Arial

Individual References

Discuss Wall faces, wall centerlines, center of core, faces of core

Face of Core

Dimension the front wall.

How does it look?

Note: You can change dimension styles on the fly. (Show)

You learned to create a new dimension style, not to mess with existing/default styles, and control the way your dimensions appear on the drawing.

Save as revit2018demo16.rvt

# Creating a Site Plan in Revit

Creating the property lines when given lengths:

Massing and Site (in Browser)

VH the Elevation annotations

Massing and Site (in Ribbon)

Property Line

Create by Entering Distances and Bearings

75’6 S60W 	Insert

110’0” S30E 	Insert

63’0” N60E

Add line to close (and pray site data given to you is accurate )

That last line should be about 110’ N23-31-01W

OK

Place on drawing

Click on white to Exit

Discuss dilemma of site not aligning with Floor Plan/Building Model

Rotating project to true north:

Select the Site Plan in the Project Browser

Notice the house is not lined up the way we want to fit effectively on the lot. If we rotate the house, ALL the views will adjust and our views will be majorly messed up!! Save work then show effects on floor plan and elevations when rotating. Undo that mess.

Here’s a solution:

Select the Site Plan in the Project Browser

Properties

Orientation: Set to True North

Nothing happens (yet)

Manage

Project Location

Position

Rotate True North

Rotate it 30 degrees (counterclockwise)

Only the floor plan/roof moves!

Move the property line to a good spot

Now look at the floor plan level 1. Has not moved.

Now look at elevations. All good

Discuss Project North (Floor Plan) vs True North (Site Plan)

North Arrow

Site Plan

Annotate

Symbol

North Arrow 2

Place it

Pick North Arrow

Clipboard Copy

Floor Plan Level 1

Paste Aligned to same place

Accurate North arrow!!!!

Note: ALL PLAN VIEWS SHOULD SHOW A NORTH ARROW

Create the topography:

Site (in Browser)

Massing & Site

Toposurface

Place Point icon

Set elevation to -10”…pick top-left corner

Set elevation to 3”…pick top-right corner

Set elevation to -2’3”…pick bottom right corner

(a surface needs at least 3 points)

Set elevation to -2’6”pick bottom-left corner

Green Check

SD…HL

Add property line annotation (bearings and lengths):

Annotate

Tag by Category

By Category

Touch property line

(not loaded) Yes

Annotations

Civil

Property line Tag (open)

Uncheck leader

Place tags

Should give you bearings and lengths you put into table!!

Edit family to explain what tag is doing: looking for something

Show door tag family (looks for “mark”)

Show Window tag family (looks for “type mark”)

Later, you can create annotation tags (families) that will look for what YOU want!

Discuss what would happen to annotation had you ROTATED site to make it work.

Label contour lines:

Site in Browser

Massing and Site

Label contours

Make a line that passes through all contours

You should see contours labeled in 1 ft increments

Let’s say you wanted 6” increments:

Massing & Site

Model Site (see that little arrow off to the right…select it)

There is your Site Settings control box.

Set Start to -5’0” (we went below 0’)

Set increment to 6”

OK

Label the new contour lines

Place Elevation marks:

Annotate

Spot Elevation

Properties

Target (project)

Uncheck leader

Place on drawing

You should see annotation for the corner elevations you created earlier.

Move spot elevation over topo to show effect….see what the tag is looking for??

Dividing up a toposurface

You may need to create a driveway, patio, or other entity that breaks up the toposurface or changes the material of the toposurface:

Discuss SUBREGIONS (spray-paint) vs Split Surface (change physical topography)

Site (in Browser)

SD See the brown dirt?

Massing & Site

Split surface

Select toposurface (anywhere)

It kind of grays-out

Use lines, arcs, etc to create the shape (closed loop).

Finish Split Surface

You can delete that small surface and create your own, with different topography) in its place or simply change the material of the area.

Select surface

Edit Surface

Change each corner point to -1’6”.

Delete any other points….careful, they are tough to see/find.

Material: (in Properties)

Change material to Grass

Notice it is not loaded. Revit 2018 has a new-look material library.

Misc…..Grass….up arrow (to right)….add material to document

Pick Grass

Double click on Grass (to get Material Editor)

Check Use Render Appearance  Discuss Shading vs Render

Done

OK

See in 3D

Subregion

Subregions are a little simpler. They simply change the surface material of the topo…..without physically changing it. Like spray-painting the surface

Floor Plan Site

Massing & Site

Subregion

Material:

Change Material to Concrete Exposed Aggregate

OK

Draw the shape

Finish Subregion

Now….let’s discuss what happened here. We have  FLAT grass and WAVY concrete!

What surfaces should be subregions?

What surfaces should be split surface?

DISCUSS

You have learned to create a site plan with property line, topography, annotation, and materials. You learned that the site can’t be rotated to position it to the building, but a view setting to True North solves the problem.

Save as revit2018demo17.rvt

# Basic Stairs in Revit

Floor Plan Level 1

VH the property Line

VH the Floor

Architecture

Stair

Stair by Sketch

Stair Properties (review how to change width, tread size, risers…show change if risers = 20..resulting in riser height of 6” at 10’

Now try 10 risers.: See error? Because riser height breaks rule set in the program.

Back to 18 risers….

Show Stair Rules

Edit Type

Calculation Rules: Edit

Get out of that box

Straight Stairs

Create a straight stair in the front of the building. Note riser count as you move your mouse. Notice “Center justification”. Notice Stairs created, Stairs Remaining.

Finish Stairs

Click in white space

3D View

South Elevation

Note the stair is going from Level 1 to Level 2

Delete the stairs

Back to Level 1

U-shaped Stairs

Stairs

Stair Properties

Set width to 4’0”

OK

LC in front of the building, starting in front of the far-right edge…Slide mouse across and to the left (notice 11” change in temporary dimensions and remaining risers)

Create 11 risers as you slide across.

LC.

Slide mouse straight down…using the green line to align with the last riser.  Don’t worry about how far now. LC. Slide mouse across and to the right well past the last riser location.

LC.

Notice landing is created between!

Finish Stairs

See in South Elevation

See in East Elevation

See in 3D

Back to Floor Plan Level 1

Adjusting the gap between the U-Shaped Stairs

Select Stairs (not railing)

Modify Stairs (pull-down at top)

Edit Sketch…You are now in “Sketch Mode”

Pick blue line on bottom

Drag up and down

Finish Stairs.

Show in 3D. Notice railing is attached automatically.

Back to Level 1.

Edit your stair’s footprint

Select stairs (not railing)

Modify Stairs

Edit Sketch

Delete the 2 higher horizontal green lines on upper stair

We will now create a new path for the stairs:

Boundary

Make green lines taper, so beginning of stairs is wider. Use mirror….

Finish

Show in 3D. Easy??

Level 1

Pick stairs

Modify stairs

Edit Sketch

Delete upper, green line

Let’s curve the left edge:

Boundary

Pick Start-End-Radius from the option bar

Create the arc

Finish Stairs

3D

Notice railing attaches correctly!

Adjusting Riser Shape

Pick stairs

Modify stairs

Edit Sketch

Pick the first riser on the lower stair

Delete it

Riser

Pick Start-End-Radius from the option bar

Pick end of boundary line

Pick other end of boundary line

Slide mouse to right a bit to create arc…set it to 2’6”

LC

Finish Stairs

See in 3D

Railings

Note: The railing can be changed or deleted by section.

3D View

Select a Railing

Change Element Type

Guardrail-Pipe

Handrail-Pipe

These are just the “out-of-the-box” Railings. You can make your own or get them pre-made.

Floor Plan Level 1

Select the edge of the stair to find the railing

LC on railing

Edit Path

Delete a railing sketch line

Finish Railing

See in 3D

Floor Plan Level 1

Let’s put the railing back in the same place:

Architecture

Railing

Sketch Path

Place magenta sketch line in the same place you erased the other one

Finish Railing

3D

Oops!! The railing is on the ground??

Why? Because you told it too …it didn’t know you wanted to put it on the stair.

Select the railing

Edit Path

SET RAILING HOST

Pick the stair (the stair hosts the railing)

Finish Railing.

Better?!

View

Building Section thru it. Adjust as needed.

You can delete the stairs.

You learned to create straight and U-Shaped Stairs, change the shape of the stairs, recognize stairs have rules (that can be changed), edit railing, and place railing on a flat surface.

Save as revit2018demo18.rvt

# Slab Foundation Plan

Make all of the 1st Floor walls 5/8GYP5.5ST5/8PLY2CLAP (except the Garage wall)

In Section 4

Pick First floor

Edit type

Change to Generic 12

Pick Generic 12

Properties

Edit Type

Duplicate

Name it: 5” CONC SLAB….OK

Structure: Edit

Thickness: 5”

Material: Concrete Cast In Place Grey

OK

Function: Exterior

Notice the floor is thinner!

Footing as a Sweep

Floor Plan Level 1

Note: To prepare for this “sweep” process, we need a section at the first line of the path.

Architecture

Component

Model In Place

Family: Structural Foundations	OK

Name it: EXT FTG	OK

Sweep

Sketch path

Create magenta line around building.

There is a problem….

The concrete footing does not go to the face of the wall! It goes to the face of the core!

Move the magenta “sketch” lines to the right place.

Note: The Garage is all wrong….it drops. We will deal with that later.

Set the material to Concrete Cast In Place Grey (same as slab)

Finish. The path is done. A sweep is a path AND a shape…so we are not done!

Edit Profile

Section 5 (get it?)

Open View

Set display to medium

Sketch the footing shape

Make sure the material is: Concrete Cast In Place Grey

Finish…Finish…Finish Model

Modify

Join

Join geometry

Pick footing….pick slab

JOINED! EXPLAIN

See in 3D from bottom

Turn off topo….VH it

Cut a section anywhere….you will see the slab foundation correctly!!!!

Save as revit2018demo15.rvt

Foundation Plan

Duplicate w detailing

Rename to “FOUNDATION PLAN”

Properties

View Range

Top:0

Cut: -1’4”

Bottom: -2’6”

View Depth Level: -2’6”

Show section and goal of cut

Manage

Additional Settings

Line Styles

New

Name it “FOUND EXT”

Weight: 8

Line: Solid

OK

Annotate

Detail Line

FOUND EXT

Use pick arrow or line

Tricky part: Garage to house

Solid line is where wall is on garage side

Make reference planes where drop is.

View Range

Top: 1’0”

Cut:  1’0”

See where wall is?

View Range

Top: 0

Cut:  -1’4”

Annotate

Detail Lines

<hidden>

Place hidden lines where footings underground are.

VH Structural Foundations to see hidden lines

CURBS

Turn Structural Foundations back on.

Now the curbs.

View Range

Top: 0

Cut:  -2”

See the curb lines and cuts?

Make solid lines at the curbs

VH the structural foundation again

Set the cut back to -1’4 (otherwise you would be cutting thru 5” concrete floor)

Dimension the Foundation Plan with class dimstyle (see page 112)

Add Foundation Notes (see page 388)

# Callouts

In Section 4

View

Callout

Properties

Wall section

Create the Callout to cover the footing area up to a bit above level 2

Notice you have a “Callout of Section 4” in your Wall Section Browser

Select the Callout…..RC…Go to view

Change detail level to medium

Set the scale at ½”=1’0”

We now have a shell of the area we need to detail, with some nice information already there.

What we do now will be 2D and not show in the other views.

Let’s place a “2X6 TREATED SILL PLATE” at the base of the wall, on top of the slab.

Annotate

Component

Detail Component

Load Family

Detail Items

Div 6-Wood and Plastic (tons of stuff in these libraries)

Wood Framing

Nominal Cut Lumber-Section

Select 2x4, 2x6, 2X8,2X10,2X12,4X4,4X6,4X8 (note true sizes)

OK

Select the 2x6 from Change Element Type

Hint: Use the space bar to flip the 2x6’s.

Hint: Use the arrow keys to “bump” the 2x6’s into place. You can also use the Align command to get them perfect. They should fit PERFECT!

Place the sill plate.

Place the 2X6’ below the window

Annotate

Insulation

Width 5-1/2”

Place the insulation and drag your mouse. This is called a repeating detail (cool eh?).

The next step is to create a detail section from our wall section

View

Callout

Detail View

Detail

Place it at the wall-slab area to create a “Footing Detail”

Pick Callout…..RC…Go to view

Set the scale to 1”=1’0”

Show leader adjustment using grips

Notice the Detail Components did NOT come over! DISCUSS

2D details are VIEW SPECIFIC

There is a little trick to bringing them over without re-creating….

Back to Wall Section Callout

Select the 2D elements you want (hold Cntl key)

Edit

Clipboard Copy

Back to the Detail View

Paste Aligned

Aligned to Same Place

COOL? Note the scales of the drawings were different….but it still worked. The Revit “Detail Components” are not completely dumb!

Note: you have another folder entry in the Project Browser called “Detail Views”

Go to the Detail View

LC in white space

Propeties

Identity Data

Title on Sheet

EXT FTG @ EAST WALL. This changes the title without renaming the view (helpful for drawing coordination)

OK

Anchor Bolt

Let’s bring in an anchor bolt to attach the foundation to the wall:

Annotate

Component

Detail Component (they are a little hard to find)

Load Family

Detail Items

Metals

Common Work Results for Metals

Metal Fastenings

Anchor Bolts Hook-Side

Open

Choose 5/8” size from Change Element Type…..Place it.

Make it 10” long…..I know it seems short. We just made our footing deep….

Show stretching vertically down. Show element properties control. Show mirror.

Rebar

Annotate

Component

Detail component

Load family

Detail Items

Concrete

Reinforcing Steel

Reinforced Bar Section

Open

Change Element Type

try Reinf Bar #6 in Browser

Use green lines to align! Place 2

Editing a Sweep

The architect made a mistake telling you to go down 2’6”….it should have been 1’6”.

Wall Section 5 (remember?)

Pick footing

Edit in place

OK to message

Pick footing again

Edit Sweep

Pick Footing again

Edit profile

See the magenta lines? That’s your sketch!

Change the depth to 1’6”

Green Check

Green Check

Green Check

Done!

Check your detail view….GOOD!

Check all of your sections…..everything updates!

Break Line

Annotate

Component

Detail component

Load family

Detail Items

Div 01-General

Break line

Open…..(Space bar to rotate) select break line to grip and change

Show the Detail Components folders that users need to make themselves familiar with….

Let’s put some ground in: (we won’t use the existing topo here)

VH the topo

Turn Linewights ON

Filled Regions

Annotate

Region

Filled Region

Line Style

<Invisible lines> So you won’t see the outside lines…..

Properties

Edit Type

Duplicate

Call it “Bills Earth”

OK

Fill Pattern (pick)

Pattern type: Drafting

Earth

OK….OK

Create the region that you want the earth to be in (it must be a closed shape)

Finish Region

Let’s add some notation:

Annotate

Text

Properties

Choose 3/32 Arial

Pick Two Segments for leader

Pick where the arrow will point to…..stretch it out to where you want the text to go. TYPE text to identify wall components

You may have to adjust the annotation crop region out a bit to see the text (you will eventually uncheck the crop region visible box before putting this view on a sheet)

Show Crop Region and Annotation Crop Region. Show how Text can disappear even if it is there

Place the wall section callout and the detail view on sheet A600.02

Note annotation symbols get smart on the Wall Section and the Floor Plan!

Let’s see all the drawings you have created:

Ctrl Tab will cycle you through the drawings you have created.

Window Tile will display all the drawings on the screen. Remember, each of those drawings is all part of the same file…..all tied together as a database of information!

You have learned to create detail views, apply 2D detail components to Section and Detail views, create filled regions, and add (dumb) text annotation.

Save as revit2018demo16.rvt

# Area Plans

Revit has an Area Analysis Tool to define and label spatial relationships. These are different than “Rooms” in Revit Architecture and “Spaces” In Revit MEP. In fact, Areas and Rooms can occupy the same place (on top of each other). Areas have different parameters than rooms….keep track of different information.

Open Area Plan 2018.rvt

Room and Area

Area and Volume Computation

Area Schemes

Out of the box, there are 2 schemes. Gross Building and Rentable

OK

Gross Building: Total Constructed Area of Building

Rentable: Area measurements based on the standard method of measuring floor area in Office Buildings.

Gross Building Area

Frequently it is necessary to find the total area of a floor plan. The “Gross Building Area Plan” is perfect for this and is really fast!

See Browser…No Gross Area Plan. We are not going to duplicate the Floor Plan Level 1.

Room

Area

Area Plan

Gross Building

Level 1

Check Do not duplicate existing views

OK

Say YES to the box. Discuss NO…..would create our own boundary lines…

Notice the new “Area plans” in the Browser. Automatic!

The Magenta lines are “Boundary Lines. They could be moved, new ones created….but those are good.

Notice the Area Tag is there showing the Area of the Floor Plan. Nice!

On a multi-story building, it is really fast to get the area of each floor…..

Rentable Area

Architecture

Area

Area Plan

Rentable

Level 1

Do not Duplicate

OK

Yes

Notice the new entry in the Browser….Automatic!

Notice the magenta lines are in a different place.

Architecture

Area Boundary

You will pick each wall (no window, sorry) so magenta lines are on each wall

Architecture

Area

Area

Pick in lower, left corner

Name it Tenant 1. It should be 688.50 sq ft

Pick on Area (see the X)

Area Type: Office Area

Done

Let’s do the same for the top, left space:

Area….Area

Pick top left space

Name it Tenant 2

Area Type: Office

Now the Corridor

Area…Area

Name it: Corridor

Area Type: Building Common Area

Now the Bottom Right “Store 1”

Area….Area

Name it Store 1

Notice the area is 273.44

Notice where the boundary lines are…….

Now change the area Type…

Area Type: Store Area

Check out the Area change! 317.79! Why? When you have Store area, you have to pay for all of the area! There are industry rules for how spaces are measured. Let’s say Store 1 was rented at $1000/sq ft. Which number would you want to use???

Repeat process for Store 2 in top, right corner.

Now the building “Core”. This is where stairs, escalator or an elevator would go. Notice where the magenta lines are….

Area….area

Name it “CORE”

Area Type: Major Vertical Penetration

Notice changes in boundaries! This is Revit applying the industry rules for areas when they are adjacent.

Area Color Fill Plan

We can add color to the Areas based on their Area Type

Annotate

Color Fill Legend

Place on drawing to the right of the floor plan

Space Type: Area Rentable

Scheme 1

Ok

Done!

Show how to

Show how to change colors:

Architecture

Room and Area

Color Schemes

New scheme

Name it SHS Scheme

Apply new colors

Ok

Pick on Legend

Edit Scheme

Pick SHS Scheme

OK

You can make as many schemes as you want!

Area Schedules

You can quickly create a schedule of the Areas

View Schedules Schedules/ Quantities

Areas Rentable

OK

Choose:

Name

Area

Area Type

Perimeter

Add

OK

Good!

Let’s total the Area Types….

Sorting and Grouping

Area Type

Check Footer

Title count totals

Formatting

Area Check Calc Totals

Place the Area plan and Schedule on a B- sized sheet.

So we learned how to quickly organize areas to give us information based on industry standards.

# Creating Materials in Revit 2018: Basics

Manage

Materials (icon)

Note the existing “out of the box) materials

We will make a few custom materials in this demo…

Hit the circle with the + sign at the bottom of the palette

Creat new material

“Default New Material” is created. It has no color, surface pattern, render appearance or filled in parameters. Perfect to start from scratch with!

We will start by creating a teal carpet

Find a good teal carpet swatch on the internet…save as a .jpg or .png

RC on Default New Material….Rename

Name it SHS Teal Carpet

Identity Tab

Description: Bolyu Ocean Teal 348 Carpet

Graphics( tab)

Shading		Color: Pantone….3265

OK

OK

Appearance

Open Information arrow to see contents

Name: SHS Teal carpet

Image: Assign captured Teal Carpet image.

OK…one down!

Let’s make a Terrazzo material. Remember….we are just making the materials. We have not applied them to floors, countertops, etc.

Manage

Materials

Create New Material

Identity: name it SHS Terrazzo (from Default New material)

Description: Western Terrazzo 78C

Graphics (tab):

Shading: Pick a color resembling your terrazzo. Since it is multi-colored, it won’t be perfect.

Surface pattern: Concrete

Cut Pattern: Concrete

Appearance

Name: Terrazzo

Image: Captured picture of Terrazzo

OK

Let’s do one more….

We will make a 4” Terracotta colored Ceramic Tile

Manage

Materials

Create New Material

Identity: name it SHS 4” Terracotta Ceramic Tile (from Default New material)

Description: 4” Terracotta Ceramic Tile

Graphics (tab):

Shading: Pick a color resembling terracotta. I used Pantone 7526		OK

Surface pattern: Model (button) …4” tile. Explain the difference between drafting pattern and model patterns. Remember…model patterns are “true size”.

Cut Pattern: Drafting…Crosshatch

Asset Browser

Open Appearance Library

Ceramic

3” Square Terracotta (close enough!)

Hit arrow to right

See appearance??

So you can use the “assets” in Revit, or create your own Render appearances!

Material Browser Controls

These double arrows close the properties info

Note the arrow direction changed… and just the materials list is showing.

These double arrows control what you are seeing in the materials palette. The default is:

Show all

List View

By name

32X32

Play with the settings!

Pick this one. It opens the Asset Browser materials (below materials).

Changing the scale of the picture you created as a material

Carpet is an easy one to bring in at the wrong scale.

SHS Teal carpet

Appearance

The carpet is too detailed. It would look cartoonish as a floor in our model…..

Pick on image…see image texture??

That is the picture that would be repeated in a 1’X1’ area of your floor.

Drag texture editor box down so you can see the editing options.

See size X?

SAee size y?

Let’s change to 48 (from 12). Now that same image would fill a 4’X4’ space.

Done!

Save as Revit2018demo19.rvt
Material Tags: Label Your Smart Materials With Smart Tags

Creating intelligent materials and tags can save you a lot of time and headache annotating your drawings. Coordination can be amazingly controlled with a little work up front. We will first create a smart material tag, then create some objects with smart materials, then smart tag the smart objects!

Ceiling Plan Level 1

Pick the ceiling over the garage

Change it from a 2X4 Compound to a GWB on Mtl stud

Edit Type

Structure Edit

Finish: Gyp Wall Board (pick material)

Material Editor comes up

Gyp Wall Board

Pick pencil to right

Material Editor comes up

Pick the arrow to the right of the material name

Descriptive Editor comes up

Set Description to GWB

Set Mark to 34

Note: We will use these two parameters when we tag our materials

GWB is the material

34 is the color

Graphics

Set Gyp Wall Board Graphics

Color: Orange

Surface Pattern: Sand (so we can see a little texture at ceiling)

Done

OK…OK….OK

You have created a smart material!

Annotate

Material Tag

Check Leader

Material

Pass mouse over ceiling (attempt to material tag the ceiling)

It tags “GWB” ….remember?? Smart….but not done.

Making the Custom Material Tag

Select on “GWB”

Edit Family (or you could have double-clicked on the tag)

Open material tag for editing: Yes

LC on Material Description

Edit Label

See sample Text?

Note Parameter Name: Description

See Category Parameters on left? These can be labels.

Cancel

Making a Custom Tag

Let’s create a custom material tag that looks for 2 parameters: Description and Mark

Description will be the material abbreviation and Mark will be a color code number.

Still in the family Editor….

Pick on Material Description

Edit Label

MTL for Sample Value (we will have 3 letter acronym for materials)

OK…

We need to see the reference planes to properly set up out text…

VV

Annotation categories

Check Show Annotation cat in view

See ref planes, dims come on

Horiz Align: Right

Our first label is done.

Create

Label

Click into space to the right of Description

Under select parameter: choose Mark

Hit green arrow to send to Label Parameters

Sample Value:123

Left Justify

OK

Make surrounding boxes work so 2 parameters do not run into each other…but are close together.

Save the tag as SHS Material Tag in your Families folder

File

Save as

Family

SHSMaterialTag.rfa  (family files are .rfa)

Load into Project

These two labels are what the tag is looking for. If we give our materials this information, the SHS Material tag will pick it up!

Now let’s tag the ceiling:

Pick the first material tag

Properties

Switch to your SHS Material Tag

See the GWB 34 ???

The tag is looking for “Material Desc” and “Mark”, which our material has!

(you may need to modify the family to keep the letters/numbers spaced properly)

Consider the drawing coordination issues solved if you create smart materials and tag them smartly….

You have learned to create a custom material, apply information to the parameters of the material that will be picked up by annotation tags. You applied the material to a system family (ceiling). You have learned what an annotation tag is and how it works. You created your own tag to “hunt” for chosen parameters.

Save as revit2018demo20.rvt

# Applying Smart Material Patterns to Floors

Patterns and Materials

Model vs Drafting

Model:

* Represents ACTUAL element appearance on building. Brick, Ceramic Tile, etc.

* They scale with model

* You can Move, Rotate, Align, Dimension to create size

Drafting:

* Represents in symbolic Form

* pattern stays same size regardless of the size.

What does this mean?

Drafting Pattern: Let’s say you put a (common) gyp-plaster drafting pattern in a space at 1/8” = 1’0”…. There might be 1000 dots in the space.

If you change the scale to 1/4”=1’0”, there would be 4000 dots and the pattern would look muddy.

If you changed the scale to 1/16”=1’0” you would have 250 dots and the space would be less “dotty”.

Hint: If you have a very dark area (that has a pattern applied to the material) in you view, the drafting pattern is the culprit. You may need to turn the pattern off or ½ tone it.

Model Pattern: The Smarter of the two. They are “real size”. Let’s say you put 1’0” square tiles in a 100 square foot room. Yes math wizards…that’s 100 tiles! If the scale is 1/8” = 1’0”, there will be 100 squares in the room. If the scale is 1/16” = 1’0”, there will be 100 squares in the room. If the scale is 1/4” = 1’0”, there will be 100 squares in the room.

Floor Plan Level 2

Pick the floor we created previously

Properties

Change to Wood Joist 10” Ceramic Tile

Edit Type

OK

Structure: Edit

Pick on Tile Mosaic	Pencil

Surface Pattern: Find the 12” Tile…OK

Make an annoying green color…OK

Identity

Description: CT

Mark: 13

Done

OK

OK..OK

See the tile floor?

SD…yucky green??

HL

Tag it

Annotate

Material Tag

SHS Material Tag

See the CT13 display??

Save as revit2018demo21.rvt

# Creating Fill Pattern

You can create your own custom “fill patterns” by leveraging your AutoCAD .pat files.

Manage

Materials

Tile Mosaic Grey	Pencil

Pick Pattern

See the materials on the left?

See the Surface pattern on the right?

Surface patterns are applied to Materials.

Some patterns are “Drafting Patterns”…and some are “Model Patterns”

Cancel…Done

You should be in the Materials Editor

Make a new Material

Default….New Material

Name the material: SHS TEST MATERIAL……OK

Surface Pattern   hit none

Check Model

New…..Check Custom……Import

Fldstn MODEL

OK…OK…OK…Done

Any AutoCAD pattern can be made into a Drafting Pattern in Revit.

If you want to make the AutoCAD pattern a Model Pattern, you must add the following line after the title of a custom hatch…like this:

Here is an example of an AutoCAD pattern file that is converted to a model pattern for Revit:

Notice the first 2 lines……and the save name.

Notice the first 3 lines…..and the save name!

OK…back to the pattern:

Path to the model pattern you created

Open

Rename it if you like…or keep the same

See Import Scale? You can change….

OK

See the new Pattern in the Fill Patterns box?

OK

The Surface Pattern is assigned to the material.

Note: This pattern will NOT appear in a rendering

Let’s test it:

Create a generic wall, with the scale of the drawing at 1/8”=1’0”

Modify

Paint

Pick the SHS TEST MATERIAL

Paint the wall.

Note: If the pattern is too dense or too sparse, you will have to change the scale of the pattern! Bigger scale numbers make the pattern bigger….. decimal numbers make it smaller.

See it OK?

Note the number of patterns….

Now change the scale to ¼”=1’0”

Note the SAME number of patterns!

I assume you have A LOT of AutoCAD hatch patterns….so you now have A LOT of Revit patterns!

You have learned there is a difference between model and drafting patterns. You applied the patterns to custom materials. You created a custom family tag to look for a targeted parameter. You applied the custom materials to a system family (floor). You learned to control the display of the pattern on the floor.

Save as revit2018demo22.rvt

# Applying Materials To Interior Elevation Surfaces: Your 2 Options

Option 1: You have a material like Paint, Wall Paper, Mirror, (flat stuff) to be placed on the wall.

Option 2: You want a wall to have “thick” material like stone, wood, or brick.

Let’s talk about Option 1 First.

In this option, we will simply apply a material pattern over the face of our element or family:

First let’s make a custom material.

Manage

Materials

Glass (notice there is no surface pattern and the glass is a bluish color).

Rc on Glass….Duplicate

Call it GL-36 		Pencil

Apply an Aluminum surface pattern

Identity

Description: GL

Mark: 36

Done		OK

You now have a new material called “GL-36”

West Elevation

SD to show color

See the horizontal siding

Modify….Geometry

Paint (icon)

Material:

Sort by text view

GL-36

Select the first floor west wall See the pattern?

See in 3D…..it’s GLASS!

Back to West Elevation

Use your SHS Material Tag on it

Annotate

Material

See it??

Undo to cancel placing the material on the entire wall.

Split Face

Let’s say you want to apply a material to part of the wall or use different materials on the wall:

HL…back to black and white.

Modify

Split Face (icon top right)

Use white arrow to pick wall (again, you may have to use the tab key to pick it)

The wall should turn orange-like…this means you are in sketch-mode

Lines

Make a rectangle inside wall.

Finish Split Face

The wall is split into two parts, the rectangle area and the rest of the wall.

Modify

Paint

Brick Common

Pick rectangle just created. See pattern?

SD….Red bricks

HL

Split Face

Use white arrow to pick wall

The wall should turn orange-like again

Make a vertical line from top of wall to bottom of wall

Finish Split Face

You can split a face this way….using the edges of the wall as a boundary

Paint

Brick Common

Pick vertical line we just created so the rectangular area we desire is highlighted

LC…See it?

Let’s try a paint:

Manage

Materials

Metal Paint Finish Blue Sky

RC….Duplicate

Rename to: PT-68

Apply

Add PT for Description

Add 68 for Mark

Surface Pattern: None  DISCUSS

Shading: Pick a purple color

You now have a paint material

Back to elevation view we worked on earlier

Paint

PT-68

Select the glass wall we did earlier.

In HL mode it should look WHITE. Elevation Views are not generally colored….

Now see 3D view.

Discus this as one alternative to annotating your materials systematically

Option 2: You want a wall to have “thick” material like stone, wood, or brick

You will have to decide what thickness of material really needs to be modeled. Paint is so thin, modeling it would be crazy. Same with wall paper. But what about wall paneling? Maybe depends on how thick??

Floor Plan Level 1

Turn off  lineweights…..View….Thin Lines

Detail level: Medium

Scale: 1/4”=1’ (this will allow us to see the components of the wall more clearly)

Pick wall between Bed 1 and Bed 2

Properties

Edit Type

Duplicate (explain why)

Note: Companies generally have a wall naming convention. Refer to your manual when creating new walls

For the demo, we will call the wall “SHS 1/2rdw 5/8gyp3.5core5/8gyp1/2rdw”

OK

Structure: Edit

Click on the preview button

Note the materials on the wall and the core boundary. The structural layer is typically within the core boundary…the 3.5” stud in this case. Gyp board, finish materials, air layers, etc would be placed outside the core boundary.

You will build the (system) family with material included.

We will do a custom wall as follows:

Fin 5			Redwood				½”

Fin 4			Gypsum Wall Board			5/8”

Core Boundary

Structure 1		Timber C16				3.5” ( real 2X4 @ 16” OC)

Core Boundary

Fin 4 			Gypsum Wall Board			5/8”

Fin 5			Redwood				½”

Note: You will need to add C16 Lumber and Redwood to the library)

OK….OK

See the wall update with the new components??

Notice how the wall is joining. That is the “function” at work. Finish 4 wraps with Finish 4. Cores are first in the wrapping hierarchy….cores MUST wrap with cores!

You learned there are 2 strategies for applying materials to objects. You learned to use the paint bucket for thin materials and add components to walls to accurately represent thicker materials. You learned to control surfaces with multiple materials utilizing “Split Faces”. All materials can be “SMART” and be tagged easily with custom or standard Material tags.

Save as Revit2018demo23.rvt

# View Templates

Once you have a view’s graphic settings perfect, you may want to apply those SAME settings to other views.

For example: On an electrical plan, dimensions, window tags, door tags, furniture, and elevation marks are turned off. The scale may be set to ¼” = 1’0”. The display level is Medium. You can save those settings! The tool is a “View Template”

You should have several floors

Floor Level 1

Duplicate…Duplicate with Detailing

Rename it Level 1 Electrical

VH the door and window tags, elevation marks, furniture, and dimensions.

VV

Make sure all of the items above are off

Set the view scale to 3/16”=1’0”

Set the detail level to Medium

Visual Style: Hidden Line

View (from pull down)

View Templates

Create Template from Current View

Name it: Electrical

OK

V/G Overrides Model: Edit

See doors furniture off?

V/G Overrides: Annotation

Note the Scale and Display Level

See dimensions, door tags, window tags, elevations turned off?

If they are not….turn them off here!

So you set how you want the view to show in the View Templates box

OK

Floor 2

Notice sections, elevations, material tags are visible…

View

View Templates

Apply Template Properties to Current View

Select Electrical

Apply

OK

Notice annotations are gone and anything else the template turned off….

Floor 3

View Templates

Apply Template to Current View

Notice Architectural Plan is default (FYI)

Select Electrical

Change View template back to Default View template

Floor 2

View

View Templates

Apply Template Properties to Current View

Select Architectural Plan

You learned to save time and ease of getting view consistency by using View Templates to control view visibility.

Save as Revit2018demo24.rvt

# Creating Custom Linestyles

Open Revitdemo12.rvt

You can create your own linestyle ala hidden, phantom, etc

There are loaded line patterns like solid, center ¼, and Dash. We will make a line pattern…which will be used in a new linestyle.

Manage

Additional settings

Line Patterns

There are actually quite a few  out of the box!

We will make a property line with a longer line in the “line-dash-dash” pattern.

Pick “Double dash 5/8”

See Line pattern Properties box?

Notice there is a space between each element AND one space at the end.

New

Name it Double Dash 1”

Dash	1”

Space	3/16”

Dash	1/8”

Space	3/16”

Dash	1/8”

Space	3/16”

OK…OK

Applying a Line Pattern  to a new Linestyle

Manage

Additional Settings

Linestyles

New

Name it”1” by 7 weight prop line”

OK

Change Lineweight Projection to 7

Change Line Pattern to Double Dash 1”

OK

You now have a CUSTOM linestyle!

Turn off the topography

Modify

Linework

Linestyle: 1” by 7 weight prop line

Pick the property Lines

D O N E !

# Filters

Filters are a really cool way to isolate what you want in a “busy” drawing.

Presentation Plan Level 1

Window in the entire drawing

Hit the filter icon (at top, right of screen)

Note: it shows you how many of each Category you have!!

Check None

Check room tags

OK

Notice ONLY the room tags are selected!

Properties

Edit Types

Uncheck Room Number

OK

They ALL update!

We will fix that gap in the family later….

Note: If you select a bunch of objects and DO NOT see the filter icon…..consider the fact that all of the objects are the same….

You have learned to control the display of your annotation using Custom Linetypes and View Templates, and select objects more efficiently using a Filter.

Save as revit2018demo25.rvt

# Electrical Plan

Open Revit2018demo25.rvt

Level 1 Electrical

Architecture

Component

Place a Component

Load Family

Electrical

Architectural

Electric Power

Terminals

Choose:

Outlet Duplex

Outlet GFI

Switch Single

Switch 3-way

Open

Load Family

Lighting

Architectural

Internal

Choose:

Ceiling Light Flat Round

Downlight Recessed Can

Troffer Light 2X4 Parabolic

Open

Set Electrical Level 1 scale to ¼”=1’0”

Ceiling Plan Level 1

See 2X4 Ceiling and GWB Ceiling?

Set Scale to ¼”=1’0”

Let’s start by deleting the 2X4 Ceiling to the right…we have changed the building since we did that….

Ceiling Level 1

Pick Ceiling…delete key

Let’s put a 2X4 ceiling over the Kitchen. You may notice we can’t see the Kitchen from the ceiling view because the pony wall (short wall) and the room separation lines are not showing in the ceiling view. What to do???

Underlay

Properties

Underlay: Level 1

Underlay Orientation: Plan

See everything now?

Create the 2X4 Ceiling

Architecture

Ceiling

2X4 Act

Use “Sketch Ceiling”

Component

Place a Component

Properties

Troffer Light 2X4 120V

Place 2 lights

Floor Plan Level 1

Do you see the light?

No!

Place a switch:

Architecture

Component

Place a Component

Switch Single

Place two switches at the Kitchen wall

Ceiling Plan Level 1

Place GWB Ceilings in the 3 Bedrooms

Note the sand texture we created earlier…

Place 4 Can Lights in each Bedroom (yea..I know it’s overkill…)

So now the problem….how do we show the switch connecting to the light? Hmmmmm!

Step 1:

In Electrical Plan Level 1

Properties

Underlay: Level 1

Underlay Orientation: Reflected Ceiling

See the ceiling now? It’s grey.

Add the bedroom doors with proper swing

Place the switches in the 3 Bedrooms

Draw swing lines from switch to light:

Annotate

Detail Line

<Hidden>

Start End Radius Arc

Connect the lights

Great…..but you don’t want to see the ceiling or the lights as grey in the Electrical Plan!

Turn off underlay

Properties

Underlay: None

So now you have the switches and swing lines, but no lights…..

Select Ceiling Plan Level 1

RC….Duplicate….Duplicate w/ Detailing

Rename “Copy of Level 1” to “Trick Ceiling Level 1”

OK

VH everything but the lights. Use the filter!

See JUST the lights?

Create a new D- Sized sheet

Name it A202 Electrical Plan Level 1

Drag “Electrical Level 1” onto the sheet

Now the cool part….

Drag Trick Ceiling Level 1on the same sheet, taking care to line up the green, dashed lines.

NOTE: This trick only works if the Ceiling Plan and Floor Plan are at the same scale.

Cool!

You will have 2 title bars

We want to keep Level 1 Electrical

Pick on Trick Ceiling Title bar

Viewport 1

Edit Type

Duplicate

Name it: SHS Viewport No Title

OK

Show Title: No

OK

So now you see the ceiling lights and the floor plan electrical stuff at the same time.

Continue adding electrical components and lights. You will need to (temporarily) turn the underlay on to see where you are going with the swings.

Save as: Revit2018demo26.rvt

# Electrical Legend

Legends provide a way to display a list of the various building components and annotations used in a project. The two most common types of legends are annotation legends and building component legends. In this tutorial, we will create an Electrical Component Legend.

We have a few Electrical fixtures loaded from the last exercise….

View

Legends

Legend

Name it: “Electrical Legend”

Scale: ¼” = 1’0”

OK

You have a blank sheet (Legends are not Revit-like…..maybe someday…)

Annotate

Component

Legend Component

Up to top of screen in Option Bar

Family

Electrical Fixtures …Pick the outlet GFI Single…bring down to screen

(The host length is the wall holding the object….3’0” is OK)

Bring in several…use green lines to align

Text

3/32” Arial

Create text descriptions to the right of the symbols

Drag the Electrical Legend on to the Electrical Plan

You can use the legend in other drawings. Copy to clipboard. Paste into another legend. You can also create a legend and add it to your company template.

# Wall Type Legend

View

Legends

Legend

Name it Wall Type Legend

Set the Scale to 1”=1’0”

OK

Set Detail Level to Medium

Annotate

Component

Legend Component

Up to top of screen…Family

Pick one of the walls YOU created …bring down to screen

Drag onto a Floor Plan sheet…

Save as: Revit2018demo27.rvt

# Creating a Gypsum Board Ceiling

We will use the GYP Board (also called “dry wall”) Ceiling to hide the 2nd floor (subfloor), to attached ceiling-based elements like lights, ceiling fans, air conditioning diffusers, and smoke detectors. We will also use the gyp ceiling on the 2nd floor. It would be attached to the bottom of the roof joists.

We will make a 5/8” drywall:

Architecture

Ceiling

Compound Ceiling: GWB on Mtl Stud

Edit Type

Duplicate

Name it: SHS 5/8” GYP

OK

Structure: Edit

Delete Layer 4

Change Layer 2 to 5/8” thick and apply SHS gyp material

The SHS Gyp should be Flat White Paint and have a surface pattern of “sand”.

OK	OK

Let’s add a ceiling to the Den

Ceiling plan Level 1

Use Sketch Option

Note Height Offset from Level is set at 8’0”. It’s wrong, but we will use it for now.

Make the rectangle

Green Check to finish. See the pattern?

Now cut a section through the room to see where the ceiling is relative to the floor above.

Set Display level to Medium

See how there is space between the ceiling and the subfloor? This is called a “dropped ceiling”. We are not going to drop this ceiling, so we will align the top of the ceiling with the bottom of the sub-floor.

Now for Rooms on the 2nd Floor:

The Truss was not modeled in Revit (the engineers generally do the Truss calculations….. but we would attach our ceiling to the bottom of the truss, which is at the Roof Level Line.

Let’s do Bed 3 on the 2nd Floor.

Cut the section Vertically to get the best view

Back to Ceiling plan Level 2

Let’s add a ceiling to the Bed 3

Ceiling plan Level 2

Architecture

Ceiling

Compound ceiling: SHS 5/8” GYP

Use Sketch Option

Note Height Offset from Level is set at 8’0”. Again, it’s wrong, but we will use it for now.

Make the rectangle

Green Check to finish. See the pattern?

Now go to the vertical section.

Set Display level to Medium

It is too low…align the top of the ceiling with the level line.

Note: Ceilings can be “dropped” when we want to bring the ceiling lights closer to us or create less volume in the space. Bathrooms, Kitchens and closets use dropped ceilings a lot.

# Wall Sweeps

Open Revit2018demo3 (we just need a few walls for this one)

South Elevation

Wall

Wall Sweep

Pick wall

LC to place

See it in 3D

It’s a rectangular extrusion coming out of the wall

South Elevation

Wall

Wall Reveal

Pick wall a bit higher

See it in 3D

Get it?

Select the wall sweep

Properties

Edit Type

See Cornice (default)?

See Profile: Default ?

The default profile is a rectangle

See Material: Default (look familiar?)

Cancel to get out of Element Properties

Delete the sweeps

See the walls clean up.

# Wall Sweeps: Base and Crown Molding

You should have made both the finish flooring and ceiling for the room.

Base Molding is a transition piece between the floor and wall.

Crown Molding is a transition piece between the ceiling and wall.

You must load the profile family of the shape you want for the molding first….

Insert

Load family

Profiles

Finish Carpentry

Let’s load Base 1 and Crown 4

Create an Interior Elevation to view the walls of the Den

View

Elevation

Elevation

Interior Elevation

Place it.

Check all 4 blue boxes

Should look like this:

Go to each elevation and drag crop box up to see ceiling and adjacent walls.

Architecture

Wall

Wall Sweep

Cornice is the default…we need a base molding:

Edit Type

Duplicate

Name it SHS Base 1 White

Profile: Choose the 4-1/2” Base 1

Material: SHS Semi Gloss White (you may have to make this on the fly…)

OK

Attach to wall.

Align the bottom of the base molding with the top of the finish floor

Continue on the other walls in the room. Note you must be in a section or elevation view to add wall sweeps….no plan views.

Crown Molding:

The Crown Molding will be created almost exactly the same as the base molding…..we will actually start with the base molding…

Architecture

Wall

Wall Sweep

SHS Base 1 White

Edit Type

Duplicate

Name it SHS Crown 4 White

Profile: Crown 4

Material: SHS Semi Gloss White (keep same)

OK

Place on wall

Align top of molding with bottom of ceiling

Continue process on other walls in room…..be careful to keep the sweeps aligned!

Hint: Once you have one wall sweep made, use “Create Similar” to make the others.

The room will look better once you apply paint and furnishings!

## Creating a Profile Family

File

New

Family

Profile

Open

Picture the vertical line as a wall

Picture the horizontal line as a ceiling or floor (let’s say floor in this case)

We will create a simple base molding profile:

Lines

We will work in the upper, right quadrant

File

Save as

Family (note the .rfa file extension)

Base Molding 5 X 1-25.rfa

Load into Project

Creating the new Sweep

South Elevation

Wall

Wall Sweep

Type Properties

Duplicate

Name it: SHS Base Sweep 5

Profile: Base Molding 5 X 1-25

Material:

Make an off white color: You will need to load: Misc	Paint	Set the color

Ok…OK

Place the sweep on the right wall

See in 3D

Turn lineweights off to see better

RC

Change Orientation (flip)

Do side wall (East Elev) too

Show align tool to join

3D to see

## Reveals

Now let’s create a Reveal:

First let’s make a simple profile shape we can use as a reveal

File

New

Family

Profile-Reveal

Open

Note the subtle wall inference

The reveal needs to go into the wall

Make a simple cove shape (closed) starting from the wall face.

Save as “Simple Cove Shape”

Load into project

South Elevation

Architecture

Wall

Wall Reveal

Type Properties

Duplicate

Simple Cove

OK

Note no material….

Set profile to “Simple Reveal”

OK

Place on wall

See in 3D

Place reveal on East wall

Try to match

See in 3D

Show: Modify…..Geometry…..Edit Wall Joins

Show grips to edit sweeps around doors and windows

# Creating a “Smart” Furniture Family

Families can be created or found that work in one instance but sometimes need to be modified. What if we only had one size of window, and had to create a new window each time we wanted a different size? We would be AutoCAD! Revit allows you to create modifiable families, so you have the ability to change certain selected properties (like width, length, thickness, radius, etc) to meet your needs. We will create a simple table. We will be able to adjust its length, width, height, top thickness, and base thickness. The strategy of family creation will be emphasized.

File….New…..Family….Furniture.rft (.rft are family templates)

Open

Show reference planes

Cen FB

Cen LR

Ref level

Floor ref level

Make 4 ref planes parallel to axis

Dim EQ to center planes

Dim overall width (do not worry about size)

Dim overall length (do not worry about size)

Note: We have not created any geometry yet

Front view

Reference plane

Create Top reference plane

Name it “TOP”

Create Bottom of Top reference plane (a little below “TOP”)

Name it “BOT”

Create a  horizontal reference plane between BOT and Ref Level

Name it “TOP OF BASE”

Note: We still have not created any geometry yet

Dimension from:	TOP to Ref Level ref plane (tab to get it)

Ref Level ref plane (tab to get it) to TOP OF BASE

TOP to BOT

Move reference planes around. See dimensions change?

Concept: We will be moving the reference planes when we adjust the sizes of our table. The geometry of the table will be locked to the reference planes.

Family Types

Notice there are no dimensions here (yet)

Cancel

Floor ref level

Select the overall length dimension

Label…add parameter

Name it “LENGTH”

Group: Dimensions

OK

Select the overall depth dimension

Label…add parameter

Name it “DEPTH”

Group: Dimensions

OK

Types

See 2 dimensions? LENGTH and DEPTH

Change Length to 6’

Change Depth to 4’

Apply

See ref planes move and dim update. Notice they moved equally from the center planes?

Change Length to 5’

Change Depth to 6’

Apply

This is called “FLEXING”

Cancel

Front view

Select Height dimension

Label…add parameter

Name it “HEIGHT”

Group: Dimensions

OK

Select top thickness dimension

Label…add parameter

Name it “THK”

Group: Dimensions

Select base thickness dimension

Label…add parameter

Name it “BASE HEIGHT”

Group: Dimensions

OK

Family Types

Base Height: 1’  Apply

Height: 4’  Apply

THK: 1”

See the ref planes move?

Note: We STILL have not created any geometry yet!

File

Save as:

Family

Table Ref Layout

OK…Let’s create some geometry

Floor ref level

Create

Set (you must do this first)

BOT	OK

Forms

Extrusion

Make a rectangle inside the ref planes

Align and lock the rectangle to the 4 ref planes

Green Check

Front view

Drag the top down with the blue arrow and lock the rectangle to the TOP ref plane.

See in 3D…SD

Types

Depth to 2’

Length to 4’

THK to 2”

Apply

See the change?

Try:

Family Types

Depth to 4’

Length to 8’

THK to 3”

Apply

See the change?

This is called FLEXING the model. If your model did not change, you need to review the process you used and troubleshoot.

Select the top extrusion

Properties

Material: Glass

3D

SD

See material?

Now let’s do the trapezoidal base

Floor ref level

Make (interior) parallel ref planes about where the base might be

Dimension the same way we did the Top, with EQ’s and overall dimensions

Name overall dim as “BASE LENGTH”….dimensions

Name overall dim as “BASE DEPTH”….dimensions

Create

Set

Level: Ref Level

OK

Create

Blend

Make rectangle

Lock sketch lines to ref planes

Depth: 1’ (will be adjustable)

Edit top

Offset 3”

Use existing lines to create smaller box

Green Check

Front view

Align and lock top of trapezoid to TOP OF BASE ref plane

Modify

Align

Pick Ref Plane….Pick top of lend

Lock it

Time to FLEX!

3D

Types

Base height: 6”

Apply		Did it flex?

Base Length: 3’

Base Depth: 2’

Apply		Did it flex?

OK

Front view

Pick base (trapezoid)

Properties

Material (we will create our own)

Default….duplicate

Name it: COPPER

OK

Shading: pick a copper color

OK

Green Check

3D view

Center piece of table

The center piece will be a revolution (like a turned-spindle)

Front view

Create

Set

Center Front/Back

Forms

Revolve

Axis Line

Make along center…vertically

Lines

Create ½ (profile) object between BOT and TOP OF BASE planes on left side of axis

Align and lock the top line of sketch to BOT

Align and lock the bottom line of sketch to TOP OF BASE plane

Properties

Material: Red Aluminum

Double Click on Red Aluminum

Set Shading to Use Render Appearance

Done	OK

Green Check

3D view

Flex it…all of it

If it works in the family, it will work in the project.

Visibility Settings

You can control what objects you see from particular Views. Example: We may not want to see the revolution from Plan View…..too busy, especially at the smaller scales.

Let’s turn off the revolution from Plan View:

Pick the revolution

Visibility Settings

Uncheck Plan RCP	OK

Discuss Detail Levels Hiding too

Save it as: CRAZY TABLE.rfa

Types

Let’s Create 3 “types” of Crazy Table: Small, Medium, Large

Family Types

New

Name it: SMALL		OK

THK: 2”

Depth: 2’9”

Length: 4’0”

Base Length: Length-1 (in Formulas)

Base Depth: Depth-.5 (in Formulas)

Family Types

New

Name it: MEDIUM		OK

THK: 2”

Length: 5’0”

Height: 4’0”

Depth: Length/2 (in Formulas)

Base Length: Length-1 (in Formulas)

Base Height: 1’0”

Base Depth: Base Length-1 (in Formulas

Family Types

New

Name it: LARGE		OK

THK: 2”

Length: 7’0”

Height: 4’0”

Depth: Length/2 (in Formulas)

Base Length: Length-1 (in Formulas)

Base Height: 1’0”

Base Depth: Base Length-1 (in Formulas

Make some Height changes…Discuss design…GIGO!

How could you adjust formulas?

How could you control sizes (revolution stretching….)

Save it as: CRAZY TABLE.rfa

Load into Project

Component

CRAZY TABLE

Load into Project

Place it…It will be a Component

Properties

Edit/new

Duplicate

This is where you can set up even more sizes for the table. Notice you have options to change sizes based on the PARAMETERS we set up. When we change the parameters, we are actually moving the reference planes….which move the objects locked to them.

You have learned to create a furniture family with modifiable parameters and materials. You learned to create extrusions, revolutions, blends, lock geometry to reference planes, and FLEX the model to check to see if your parameters work.

# Material as a Parameter: Family

File….Open

Open “CRAZY TABLE”

Select table top

Edit Extrusion

Properties

Material

Pick the far, right box

Associate Family Parameter box comes up

Add parameter

Instance (bullet)

Group/Parameter under Materials and Finishes

Name it: Table Top Material

(notice type of parameter is grayed out)

OK…OK

Save it as: Smart Table w/ material parameter

Load into project

Bring in “Component”

3D….SD

LC on object…

Properties

See Material Parameter?

You can make the top any material you want that is loaded in the project.

Discuss advantages of having material as a parameter when multiple materials are in a family or the material will be changed in the project often.

# Families: Steps to Creating Families

- Sketch it

- Do the math

- Make the associations (formulas)

- Create the parameters

- Instance or Type

- Add the formulas

- Draw the reference geometry

- Dimension the reference geometry

- Turn the dimensions into labels

- Flex it

- Draw the masses and linework

- Align and lock the linework to the reference planes and reference lines

- Flex it

# Title Block Family

File …..New…..Title Block

C  24x18 (Family Template  .rft)

Open

Dimension existing lines horizontally and vertically. These lines represent the edge of the paper and will not be printed. Confirm 24X18. Delete the dimensions.

Custom Linestyle for Title Block

(We need a thicker line for the border than is out of the box)

Manage

Object Styles

New

Name it Border9

Subcategory: Titleblocks

OK

Set Lineweight to 9

Linecolor to Black….Solid

Create the borderlines

Create…..line….Subcategory: Border9

½” in from edge of paper

Turn Lineweights ON

Labels and Text

Create

Label

Place Label

Pick somewhere in the center of border to place (temporarily) the label

Sheet Name

Show Rotate

Label Sizes

Pick label tag 1

Edit type….duplicate

Name it : ¼ Arial Bold….OK

Check Bold

Create the following types w/ corresponding text heights

1/16 Arial

3/32 Arial

1/8 Arial

1/4 Arial

Show justification of Labels

I need the following labels with font on your title block:

Label			Font

Project Name		1/4” Arial Bold

Sheet Name		¼” Arial

Sheet Number		¼” Arial

Scale:			3/32” Arial

File Path:		1/16” Arial

Client Name:		1/8” Arial

Drawn By:		1/8” Arial

These are LABELS. We need text in front of them to identify the labels.

Create….. Text

Text Note1

Edit Type…Duplicate

Name it: 3/32 Arial

Create the following text notes (in CAPS):

PROJECT NAME:

SHEET NAME:

CLIENT:

DRAWN BY:

FILE PATH:

SHEET:

Adding a Logo. It can be a small picture or your initials.

Insert…..Image

Save as:FirstLastCBorder.rfa

# Smart Window Tag

Let’s create a useful SMART window tag. One that will give us the width and height of the window in inches. A 1’8”(20”) by 3’4”(40”) window would be a 2040 on our tag.

File

New

Family

Annotation

Window Tag

Open

Label….Type Name

Sample Text: WD” X DP”

Place it above the horizontal ref plane

Label….Family Name

Sample Text: Window Type

Place it below the horizontal ref plane

File

Save as: WindowXXXXTag

Load into project

Tag a window with your new tag. Remember, you can move tags!

Annotate

Tag by Category

Pick a window

See it!

You have learned how to create an annotation family from a template, edit an existing annotation family to make it look for what YOU want, and create “smart” and “dumb” annotation.

## Smart Window Tag: Tag by Instance

Let’s create another useful SMART window tag. One that will give us the ability to tag a window by instance. As you know, window tags are type mark tags. 3 exact windows will have the same tag number.

File

New

Family

Annotation

Window Tag

Open

Label….Mark

Sample Text: A

Place it at the intersection of the reference planes

Create

Line

Polygon

Place the polygon at the intersection

Name it SHS Window Instance tag

Load into project

Pick a window

Properties

Mark:C

Tag a window with your new tag. Remember, you can move tags!

Annotate

Tag by Category

Pick a window

# Drafting Views

Revit needs a place to put .dwgs, .jpgs, etc. Drawings and images that are not attached to the building model. Drafting views provide this opportunity.

Open Revit2018demo8.rvt

I have 2 items that need to go into my Revit drawing, condiment.dwg and Elevation Concept.jpg.

View

Drafting View

Name it: Concept Elevation Views

Scale 1-1/2”=1’0” (it’s a scan)

Insert

Image

Concept Elevation

Open

Place in screen

See it?

Create a new sheet

D size

Drop Drafting View on sheet

You can change the scale by activating the view, just like other Revit views.

Create a new drafting view. Name it”Condiment Bar”

Scale:1”=1’0”

OK

Let’s bring in an AutoCAD file.

Insert

Import CAD

Name it Condiment Bar Details

Scale: 1” = 1’0”

Ok

ZA to show (sometimes they come in out of the view area)

Also, toolbar on top, right corner of drawing area has zoom tool.

VV

Imported categories

See layers? They can be turned on and off ala AutoCAD layer control

Cancel

Drag onto a sheet

You can bring in standard AutoCAD details into Revit this way. Details that are not attached to the building model. You can even create sheets of standard details and save them to your template file for use on each project!

Note #1: You can do a full or partial explode when bringing in .dwg files…just look for the boxes at the top of the drafting view. You can also delete layers in that view.

Note #2: Revit CAN NOT import a Microsoft Word .doc

Note #3: Revit CAN NOT import a .pdf

Note #4: Revit CAN import .gif  .jpg  .jpeg   .png   .tif  files. If you can get your image into one of those fomats, Revit WILL import it.

Did I mention to watch your file size when importing images?? They can make a manageable project a nightmare by making the file oversized!

# Keynotes

Types of Keynotes

Element. A keynote can be applied to an entire element, such as a wall, detail component, or door.

Material. A keynote can be assigned to a material that has been painted on a surface, and to materials assigned to the component layers of an element. Material keynotes are not supported for the insulation drafting tool, the detail components line and filled region, or wireframe views.

User. This option provides a way to use commonly used notes or phrases to address documentation issues. These additional user notes must be added to the provided keynote text files or included in one that you create. See Adding Additional Categories.

Assigning Keynote Values to Materials

- Keynote selection can be applied to materials. Keynotes are assigned to materials on the Identity Tab of the Materials dialog. To access the Materials dialog, click Manage tabSettings panel (Materials).

- When materials are assigned a keynote value, the objects that use these materials inherit the keynote value accordingly.

- A great deal of time can be saved if the materials of a project have their keynote values applied before you apply keynote tags to materials. See Changing Material Identity Data.

Assigning Keynote Values to Elements

- All elements have a Keynote Type parameter. These can be supplied in advance using the Type Properties dialog (see Element Properties), or selected when the tag is placed.

Assigning Keynote Values Using Tags

- After you apply a keynote tag to an element, you can select the keynote tag and click over the keynote value or empty value to open the Keynotes dialog. Select the desired keynote for the element.

- Note If you click OK, Revit Architecture uses the keynote value that is currently selected. If you click Cancel, the keynote value remains empty.

See CSI Masterformat.pdf for categories on the common drive

The Keynote File

(It’s just a little .txt file)

Note the 1 tab between the items

Note the last number is the group heading

Example: 	Concrete Forming 03 11 00 is under Concrete Forming and Accessories 03 10 00

Concrete Slip Forming 03 11 13.13 and Concrete Shoring 03 11 13.16 are under

Structural Cast-in-Place Concrete Forming 03 11 13

It’s an organizational system….

Let’s get started!

First we need to point to the Keynote file we want to use.

Annotate

Tag

Keynote Settings

Choose path to Keynote file

Annotate

User Keynote

Keynote Number-Boxed-Small is OK

Pick the treated Sill Plate

Open +06

Pick 06.02….OK

Done

Now pick insulation (command continues)

Pick 06.01…OK

Done

# Creating the Keynote Legend

View

Legend

Keynote Legend

Name it “Keynotes”	OK

Notice Key Value and Keynote Text are ready to go in the schedule! 	OK

Open sheet that has the Wall Section

Place the Keynote Legend on the sheet!

Open Keynote txt file

Under 09 Finishes, we are going to spec a  countertop material

Put ¾” Willow Green Corian”

Be sure only ONE tab….then 09

Element Keynotes

You can assign Keynotes to Elements too.

Element Keynote

Pick the countertop

Edit Type

See Keynote?

Not there????? Why????

We updated the .txt file, but Revit doesn’t know it was updated yet.

Tag

Keynote settings

Reload

OK

Try again…

Pick the countertop

Edit Type

Keynote

09

See Keynote? Ahhhhh…there it is!

Pick Willow Green 09.01	OK

Now the element has a keynote.

Applying the Keynote to the object

Keynote

Element Keynote

Pick the countertop….see it?

Check the keynote schedule

NOTE: You can create the keynote legend by sheet, so only the keynote you used on that sheet are applied to the keynote legend on that sheet.

# Grouping

Grouping is similar to blocks in AutoCAD. You can put items together and :Group” them, so they come in as one smaller object.

Open Revitdemo 22.rvt

We will start by making a “group” with no special challenges…..

Floor Level 1

In a Bedroom, bring in a Double Bed, Chair and Desk.

Set them up in a Typical Layout

Select the Double Bed

Modify Furniture pull-down comes up

Create

Create Group

Name it Typ Double Bed Layout

Check open in Group Editor

OK

+Add

Select the Desk

Select the Chair….notice when you add it, the component grays out.

Finish

Select the group you should see a dashed box around the group

See it is ONE “object”, that can be used over and over again, like an AutoCAD block.

Use it….

Architecture

Model Group

Place Model Group

Place the Group in another Bedroom

(Note: you can Rotate the group)

Editing the group

Let’s make a design change….. swap out the double beds and replace them with King Beds.

Select the group

Group

Edit Group

Pick the bed

Properties

Pick the Queen Bed

Finish

See all the groups change??

Now let’s eliminate the chair

Pick the group

Edit Group

Pick the chair

-Remove

Finish

Note: All we have done is removed the chair from the group……note the chairs were not deleted….

Undo that….so the group has the chair

Edit group

Pick the chair

Delete the chair

Finish

Note the chairs are gone in all the groups!

Adding to the group

Component

Place a component

Pick a dresser

Place it

Pick the group

Edit Group

+Add

Pick the dresser

Finish

All the groups update!

Note: All the groups update in the ENTIRE PROJECT…..FYI

# Creating Custom/Shared Parameters

As you create and use families, you may find the parameters listed or are available to you are not complete. You have a length and width parameter, but need one that is not listed.

You can create a “shared parameter”…that can be added to ANY family, regardless of category. The parameter is stored in an external file, ensuring consistency across families and projects.

We will start by making a Window Schedule (with a few challenges)

We will create a custom parameters called SHGC, U- Value, and VT. They will be numbers. We will create a yes/no value for LEED CERT. We will create a parameter for “Oracle #” (a client database number)

We will then apply it to our window schedule

We will then apply it to a window family.

We created the window schedule previously. Open that file bldg w schedules.rvt

Adding Parameters

Let’s say there is a field you need that is not in the schedule: we will use the bold/underlined parameters above…..

Manage

Shared Parameters

Create

Let’s place in on the desktop and call it DWE Shared parameters 2.txt (there should be a good place on the network drive normally to store this file)

Save

Groups…New

Enter name DWE Window

Note: You can set up many groups in the DWE Shared Parameters 2.txt file. Examples might be: Sustainable Design Parameters, Electrical Parameters, Specialty Equipment Parameters, Annotation Parameters. Each group can hold a bunch of your custom parameters

Parameters….New

Name: Oracle #

Show Discipline

Show Type of Parameter

We will take “Common” and “Text” on this parameter

OK….OK

You have created a “shared parameter”!!

Parameters….New

Name: SHGC

Show Discipline

Show Type of Parameter

We will take “Common” and “Number” on this parameter

OK

Parameters….New

Name: VT

Show Discipline

Show Type of Parameter

We will take “Common” and “Number on this parameter

OK

Parameters….New

Name: U-Value

Show Discipline

Show Type of Parameter

We will take “Common” and “Number” on this parameter

OK

Parameters….New

Name: LEED CERT

Show Discipline

Show Type of Parameter

We will take “Common” and “YES/NO” on this parameter

OK….OK

In Window Schedule

Add Parameter

Shared parameter

Select

(Edit will path to a different location)

Pick Oracle #

Continue adding the shared parameters…

Group under Green Building Properties

Let’s use it.

Floor Plan Level 1

Select a Window FG3656 on East wall

Edit type

See the parameters out of the box? (remember that)

Cancel

Edit family

Yes

Family Types

Note the Parameters….Remember??? These are your “out-of-the-box” parameters

Parameters : Add

Shared Parameter (Family will not show in schedules)

Discuss “Type” and “Instance” choices

Select

See LEED CERT

Put under Green Building Properties

OK….OK….

See LEED CERT Parameter you created??

Do the others

Add the data

Question: Would SHGC, U-value, VT, Oracle # be instance or type????

OK

File

Save as

Family

Save the Family as DWE 3656

Putting company info in name distinguishes your smarter company family from the “out-of-the-box” families. Some firms have a special folder for their company developed families, so when upgrading, locating the company files is easier.

Load into project

Swap the window with the DWE window

Back to the Window Schedule

Discuss shared parameters in families before schedule is made

Discuss Project Parameters

You have learned to create a custom parameter called a “Shared Parameter”. You learned that shared parameters can be put in schedules and families. You know the shared parameter file is a .txt file and can be updated with new groups and new parameters as time goes on. The shared parameter file can be used on future projects too.

# Room Schedules

Room schedules are very powerful data extraction tools in Revit. A typical project may have many variations of Room Schedules based on needed data….

Open revitdemo23.rvt

View

Schedules

Schedule/Quant

Rooms

OK

Area

Dept

Floor Finish

Level

Name

Number

Perimeter

ADD

(do not hit OK)

Sort/Group

Sort by: Name

> Grand Totals

Formatting

Area>Calc Totals

Perimeter> Calc Totals

Name> Calc Totals (greyed out…can’t add names???)

OK

Looks good, but order is off and Floor Finish data has not been entered (GIGO) yet…

In the Room Schedule , Add Tile in Service areas

NOTE: Only enter the data once…then use the arrows. This is important (really) because the text is case sensitive. “Tile” and “TILE” will be 2 different materials to Revit

Add Wood in Living Areas

Add Carpet in Sleeping areas

Add Painted Concrete in Non-Living areas

Add Tile for Corridor and Service Areas

RC over Room Schedule

Properties

Fields…Edit (let’s re-order)

Level

Number

Name

Dept

Area

Perimeter

Floor Finish

Check result

Why only Level 1 on Room Schedule?

Let’s organize by Room #

RC over Room schedule

Properties

Sorting/Grouping

Number

Check Grand totals

OK….Ok

Back to Level 1

Select a Room

Properties

Notice the Floor Finish is populated.

It’s a BIM!

# Room Bounding…Controlling where your Room Goes

Let’s say you have a room with a strip closet enclosed by a wall and you don’t want Revit to see the room as 2 rooms:

Create a dividing wall

Add a door

Select the dividing wall(s)

Properties

Uncheck “Room Bounding”

OK

Pick the Room now…..

See it flow into the closet

Notice the Area change??

Discuss Splitting walls when the wall goes into multiple Rooms

You have learned to create room schedules to pull data from the project. You learned how to control your room boundaries.

# Sheet List

You can create a list of drawings for your sheet set that will update as you create them.

View (from pulldown)

Schedules

Sheet List

Add

Sheet Number

Sheet Name

Sheet Issue Date

OK

Done!

RC on Drawing List

Properties

Sorting and Grouping

Sort by: Sheet Number

Check Ascending

OK…OK

You can take out a drawing from the list of drawings (of course it is still in the file):

RC over Drawing List

Filter

Filter by: Sheet Number

Does not equal: A900.01

OK…OK

Note A900.01 is not in the Drawing List

You can filter by Sheet Name too:

RC over Drawing List

Filter

Filter by: Sheet Name

Does not equal: Unnamed

OK…OK

Drag Drawing List on Cover Sheet Title Block

Shared Coordinates Demo

Open: FLSITE.rvt

Site view

Hide the topo

# Shared Coordinates Demo

A Revit project has internal coordinates for all the elements that compose the model in a project. Those coordinates are known to this project only. This is acceptable if you have a standalone model whose position is not relevant to other models or to a site. However, if you want the position of the model to be known to other linked models, you need to share coordinates.

Open: FLSITE.rvt

Site view

Hide the topo

Discuss Project Base Point: A convenient location in the project, like the corner of the building.

Discuss Survey Point: A known point on the site like the intersection of property lines.

Use filter to show/isolate survey point

See N/S		E/W	Elev

Pick the survey point

Keep it clipped

Move it to the top, right corner of the site…where the 50’00 and 82.50 line intersect

Turn the topo back on

Save the file as FLSITE.rvt

Close it

Open Revit2018demo10

Site view

VH the elevations

Save as BLD4Coordinates.rvt

Close it

So we have 2 files:

FLSITE.rvt	the site plan

BLD4Coordinates.rvt	the building

We have set the survey point at the corner of the property lines in the Site plan…our “known point”

We have our project base point in the front of the building.

Open FLSITE.rvt		(make sure the building file is not open)

Insert

Link Revit

BLD4Coordinates.rvt

Positioning: Auto center to center (We will move the building, anyway)

Open

Position the building model so it looks like this in the Site Plan view:

Move the site plan levels up 5’

Align the building Level 1 with the Site Level 1. Lock it.

Now the good stuff….

Manage

Coordinates

Publish Coordinates

Pick the link (the building). We are sending the location of the known point …the Survey point…to the building file!

You can keep the Internal (current) name

OK

Save the file

Close it

Open the building file BLD4Coordinates.rvt

Site View

See the Survey point? See the base point?

Let’s test it…

Insert

Link Revit

FLSITE.rvt

Positioning: By Shared Coordinates

Open

Change the orientation to True North

# Note Blocks

Note Blocks (also called an Annotation Schedule) are a list of construction notes.

You can create a note block and place it on a sheet.

The “trick” is to load a generic annotation family into your project, then assign parameters to the family.

We will add a parameter called “Key #” (that will be our tag #) and a parameter called “Construction Notes” (that will list ummmm construction notes)

File

New

Annotation Symbol

Generic Annotation

Open

Family Types

Parameters

Add

Check INSTANCE (explain)

Name: Key #

Type of Parameter: Text

OK

Add

Check INSTANCE (explain)

Name: Construction Notes

Type of Parameter: Text

OK…OK

Delete the red text

Create

Label

Place at intersection of planes

Key # >

Sample Value: A

OK

Create a hexagon around the A

Save as: SHS Key # tag

Load into project

South Elevation

Annotate

Symbol

SHS Key # tag

# of leaders: 1

Place the tag

Stretch the horizontal line to the window

LC on tag

Properties

Note we are in INSTANCE parameter (explain)

Key #: A

Construction Notes: Bullet Proof Glass

Apply

Notice A is in the tag…but there’s more….

View

Schedules

Note Block

Select: SHS Key # tag

OK

Key #.... Add

Construction Notes….Add

OK

See the note block with our key # and Construction Notes?!

Back to South Elevation

Select our key # tag (with the A inside)

Copy

Paste

Move the copy to the window above

LC on the tag

Element Properties

Key #: B

Construction Notes: One-way glass

See Note Block…updated? YES!

Go to Ext Elev’s Sheet A500.01

See the key # tags on the South Elevation?

Drag the Note block near it..on the sheet.

Is this a good option to Keynoting??

# Phasing

Objective: An introduction to PHASING in Revit. We will use Revit phasing to do a little bathroom remodel. The remodel will be to make the Men’s Restroom bigger, eliminate the Closet next to the Women’s Restroom.

Open Phasing Demo 2018.rvt

Manage

Phases

There are 2 out of the box: Existing and New Construction

Show Phase Filters (they define how objects will show graphically)

Escape out of phasing.

Show that all the elements in the model were drawn in “New Construction”

The “Existing” phase is in a time BEFORE the building was built….get it???

Setting up Phases

Manage

Phases

Rename “New Construction” to “AS-Built”

“Demo” after As-Built

“Bath Remodel” after Demo

OK

Creating “Phased” Floor Plans

RC on Level 1

Duplicate w/ Detail

Rename to Level 1 As-Built

RC on Level 1

Duplicate w/ Detail

Rename to Level 1 Demo

RC on Level 1

Duplicate w/ Detail

Rename to Level 1 Bath Remodel

Set up Phasing in the Views

Level 1 As-Built

Properties

Phase: As-Built

Phase Filter: Show All

Level 1 Demo

Properties

Phase: Demo

Phase Filter: Show Previous + Demo

Level 1 Bath Remodel

Properties

Phase: Bath Remodel

Phase Filter: Show All

Demolishing

Back to Level 1 Demo

Modify….Demolish

Pick items to demolish:

Men’s Bathroom fixtures

2 Closet walls

Women’s west wall (note doors go too)

Wall between Men’s and Women’s

Level 1 As Built….still the same

Level 1 Bath Remodel…..note demolished elements are gone!

Creating the New Elements

Still in Level 1 Bath Remodel

Create a new plumbing wall at ref plane “New wall”. RC …”create similar” with the other plumbing wall.

Notice it is black. That is the phase filter.

Pick on the new wall. See phase created: Bath Remodel.

Use the “Create Similar” technique to make the Men’s toilet, sink, towel holder. Bring in the Urinal (there is one loaded)

Note we have no West wall at Women’s and part on Men’s….we demolished it, remember?

Select Men’ s wall….RC….Create similar

Make the wall.

Note the join issue at the bottom. Joins are a little weird in phasing…

Here is a little trick to fix it:

Select wall

RC on blue grip at end of wall

Disallow join

Now you can drag or Align the wall.

Add the Women’s door

Notice the tile floor has a gap (where the old wall was)

RC on Floor……yes, you guessed it…..create similar. Place floor….align the tiles.

(Yes, we probably should have demo’d the tiles under the new wall….)

Home….Rooms…Add  rooms. Name them Men’s, Women’s

Note: Rooms do not copy over in Phasing…….for obvious reasons……

# Copy Monitor

Copy creates a duplicate of a selected element in the current project and monitors it to a selected element in the linked model or current project.

Monitor compares two elements of the same type against eachother, either from a linked model to the current project or within a current project.

We are going to use the copy/monitor system to trach changes to our levels, grids, columns, walls, and floors. If they change in the architecture model, we want to know about it in our MEP model!

For this demo, we are going to work with a linked architecture model for a Mechanical discipline.

Open a new file. Let’s use the Mechanical Template

Be in Floor Plan Mech 1

Insert

Link Revit

MEP-Elementary-School-Architectural.rvt

Positioning: Auto-Origin to Origin

Open

Pin the model

Check Room Bounding button on link so spaces can be added.

Move the Exterior Elevation tags out….careful with blue lines!

Go to the South Elevation

Drag the 2 Project Level lines to the right

Align the Mechanical Level 2 with the linked Level 2. Both should be 13’0”

Collaborate

Copy/Monitor

Select Link

Pick on Arch underlay

Tools

Monitor

Select our Level 1 (mechanical)

Select the Arch Level 1

Select our Level 2 (mechanical)

Select the Arch Level 2

Copy

Check multiple

Cntrl  Select Levels: TOF and Volume Modelling Reference 2

Finish Button (ignore the warning)

Check Finish

Select Levels….move to right so you can see them

Change Level type from Plenum to ¼” Head

Save as copymonitor1.rvt

Create the Mechanical floor plans:

View

Plan View

Floor Plan

Pick

TOF and Volume Modelling Reference 2

View

Plan View

Reflected Ceiling Plan

Pick

TOF and Volume Modelling Reference 2

So the Mechanical Plans went in great.

The Ceiling Plans did not go where we want!

Open Coordination (see above)

Change the Discipline TOF to Mechanical

Change the Subdiscipline to HVAC

See it now?

Do the same with Volume Modelling Reference

Change the view template of each RCP to Mechanical ceiling

Save the file

Close copymonitor1

Open MEP-Elementary-School-Architectural.rvt

Change Volume Modelling Reference Level to from 31’8” to 32’4”

Change TOF Level from -5’6” to -6’0”

Save the file

Close it

Open copymonitor1.rvt

You immediately get a Warning box!!

Expand it

OK

The Architect moved the levels…..we know about it…we were warned.

Collaboration

Coordination Review

Select Link

Pick the linked file

Under Action, let’s choose:

Move Level TOF on the first one

Reject the second one

Apply

OK

Let’s copy monitor the grids

Collaborate

Copy/Monitor

Select Link

Copy

Multiple

Select all of the grids

Finish (in option bar)

Ignore warning

See the heartbeat symbol (monitor)

Green Check to finish

Drag Grids away from each other to see duplicates

Save as copymonitor1.rvt

Close the file

Open MEP-Elementary-School-Architectural.rvt

01 Floor Plan

Turn on the grids

Move grid 19 up 1’0” (it’s at the top, right corner)

Save the file

Close the file

Open Copymonitor1

There it is again!

Collaborate

Coordination Review

Select Link

Note is still is warning us about that level…as well as the Grid 19 being moved.

Let’s fix them both.

Note: Sometimes when the Architect moves something…it’s a mistake….or it impacts the MEP plan in such a way as communication between the MEP Engineer and the Architect must happen.

This is a much better process that just looking for variations!

# Purging

You can purge unused items in Revit to make your drawing smaller in size.

Save you drawing as Revit2018demo27.rvt

Check the size by doing a File: Save as

Mine is 16000KB (that’s 16 MEG)

Manage

Purge Unused

Open Annotation Symbols

See checked boxes?

Those are unused in Revit. Note you have hundreds of unused families!

You can ONLY purge unused items….

OK

File

Save as: Revitdemo27Purged.rvt

Do a file save as again to see the new file size….

Mine is 6800KB………….that’s 6.8 MEG….a significant difference!!

Like with AutoCAD, purging …especially with large files can speed up your project and lighten the file. You can always load the families again if you need them. Don’t purge often….your file saving advantages reduce on each purge.

# Transfer project Standards

You can copy project standards from one project…and apply them to another project. For example, you may have Company Materials in Revitdemo21 that you need in Revitdemo2.

Standards could be things like:

Family Types

Lineweights

Materials

View Templates

Object Styles

You must have BOTH projects open…the target and the source

Open Revitdemo2 and Revitdemo 27.

In Revitdemo2:

Manage

Materials

Note what you have…NO custom materials like PT101

View

Switch Windows

In Revitdemo27:

Manage

Materials

Note what you have… custom materials like PT101

We are going to transfer the materials from Revitdemo24 to Revitdemo2!

View

Switch Windows

In Revitdemo2:

Manage

Transfer project Standards

Careful….Check NONE…Discuss

See all that could come over?

Check Materials

See “Copy From” (above) Revitdemo27

Take “New Only”

Now look at the materials in Revitdemo2….see the new materials?

Transferring one wall type from one project to another

In Revitdemo24, select the custom wall we created (Bills Wall)

Clipboard:Copy

View

Switch Windows

In Revitdemo2

Modify

Clipboard: Paste

Put it anywhere.

Delete it

Architecture

Wall

Wall

Change Element Type

In selection bar see “Bills Wall”?

You have learned you can copy a wall type, roof type, ceiling type …system family stuff…from one project to another.

You could create a file called FG Walls.rvt and put all your created walls in it. You would copy and paste needed walls from FG Walls.rvt to your target drawing!

Note: Putting all the walls you ever created in your template file will make your project file bigger than it needs to be.

# Phasing for Tenant Improvements

Set up	Level 1- As Built

Level 1-Demo

Level 1-Construction

Manage

Phases

We will ignore “Existing” for now

Rename New Construction to “As-Built”

Insert “Demo” Phase after As-Built

Insert “Construction” Phase after Demo

Your phases should be:

1.	Existing

2.	As-Built

3.	Demo

4.	Construction

(Note Past and Future in Phasing box)

Level 1 As Built

RC….View Properties

Phase Filter: Show All

Phase: As Built

Level 1 Demo

RC….View Properties

Phase Filter: Show Previous + Demo

Phase: Demo

You should use the Demolish tool in this (Demo) view!

Level 1 Construction

RC….View Properties

Phase Filter: Show Previous + Demo

Phase: Construction

To create an infill appearance:

In Level 1 Construction

Select demo’d object

RC

Override Graphics in View

By Element

Cut Patterns

Pattern…

Solid fill

OK

# “Stealing” From Other Revit Projects

If you have a Revit file that was completed previously, you can “steal” the following:

Drawing Sheets

Drafting Views (the ones with the AutoCAD files in them)

Schedules

Legends

So if you did a perfect equipment schedule in 7-11.rvt, you can steal it …and use it in Mobil.rvt. The actual equipment families will (of course) not come over…just the parameters and setup.

If you have AutoCAD drawings placed in drafting views (as typ. Details) in 7-11 Riverside.rvt, you can steal them..and use them in 7-11 Buena Park.rvt!

In Target File

Insert

Insert from File

Insert Views from File

Find the source file (the ones with the views you want)

Open

“Insert Views” Dialog box comes up

Check Views and Sheets that you want

OK

If you select a bunch, it will take a few minutes.

You can steal 2D Elements from a file too

In Target File

Create a Drafting View

Name it Header Detail

Insert

Insert from File

Insert 2D Elements from File

Find the source .rvt file (the ones with the views you want)

Open

“Insert 2D Elements” Dialog box comes up

Pick Detail 0

OK

The elements will be placed in the current view and on the clipboard

Finish

# Creating a .PDF of your Revit Sheet

We will create a 24 X 36 .PDF for the client.

Restore down Window (so you are not maximized)

Adjust window so you are cropped proportionally with the Revit sheet.

File

Print

Adobe PDF

Properties

Layout: Portrait

Advanced

Postscript Custom (just to right of Paper Size)

Postscript Custom Page Size

Width: 36

Height: 24

Check Inch

OK

OK

Paper Quality: Black and White

OK

Set up place to save file (Browse) in Project folder

Save

Setup

Zoom:100%

Paper Placement: Center

Preview

Print

Note: If you set up all the sheets by:

Restore down Window (so you are not maximized)

Adjust window so you are cropped proportionally with the Revit sheet….

You can print a SET by checking Selected Views and Sheets and choosing the sheets you want to print.

# Key Schedule

OK….Schedules can have the same characteristics. A room schedule might have 100 rooms with the same floor, ceiling, and base finishes. Rather than add all this info for ALL 100 ROOMS (yuk)….you can define keys that automatically fill in the information (better).

If a room has a defined key, then as that room is added to a schedule, the fields in the schedule automatically update

You define keys using “key schedules”

When you create a key, it is listed as an Instance Property for the element.

When you apply a value for the key, the keys attributes are applied to the element.

Let’s try it………..

Open revit2018demo20.rvt

It should have rooms in it ….but no data added to the rooms yet.

Let’s copy The Master Bedroom ROOM and surrounding walls to the clipboard…then paste level by name…so we have some common rooms.

Select the walls first

Clipboard:Copy

Pick in white space

Clipboard:Pasted Aligned

Select levels by Name

Level 2-7

OK

See in 3D

Back to Level 1

Select the Room

Clipboard:Copy

Pick in white space

Clipboard:Pasted Aligned

Select levels by Name

Level 2-7

OK

The rooms are there..just not tagged

View

Schedules

Schedule/Quantities

Name: Room Style Schedule (def)

Rooms

Check Schedule keys

Key Name: Room Style (def)

OK

“Key Name” is already there….

OK

See the small schedule??

Modify Schedule/Quantities tab comes up (not much there)

Schedule:New

Each row creates a new value for the key….

New

New

New

You should have 4 rows

RC on Room Style Schedule in Browser

Let’s add:

Department

Floor Finish

(Remember having to add Department names for the color fill plans?)

OK

The schedule opens….

Key Name		Dept		Floor Finish	Ceiling Finish		Wall Finish

Patient Room		Sleeping	Carpet		GYP			PL235

Common Area		Living		Wood		70’s cot cheese	PLAM

Restrooms		Service		Tile		2X4 ACT		CT567/PL234

Storage		Non-Living	Concrete	NONE			PL101

OK….Lets test it…

Floor Plan level 1

Let’s try it on the Master Bed ROOM

Select the room

Properties

See the choices in the Identity Data??

Room Schedule

Properties

Fields

Room Style…Add

Change the key names

That’s pretty cool!!!

Show Room Schedule

Show adding Room Style to schedule…..hidden field.

# Demo: Structural Components in Revit

Objective: Students will be able to setup a basic steel structure with concrete floors and footings utilizing Revit software. Students will be able to identify structural steel components such as columns, beams, braces, footings, and concrete/steel flooring.

The setup: Grids

When setting up a steel structure, start with Grids. They are set up horizontally with letters from left to right and vertically with numbers top down. The top, left corner will be Grid A1.

Architecture or Structure Tab

Grid

Make the grid line from bottom to top, vertically. Name the Grid “A”

Grid

Use the “pick lines” tool and offset grids B, C, D, E 	25’0” apart

Grid

Create a horizontal grid from right to left

Name it “1”

Use “pick lines” tool to offset Grids 2, 3, 4 	20’0” apart

Show toggle for turning on Grid bubbles on opposite side of grid line

Show “edit type” on grid line to add grid bubbles on end 1

Structural Columns

Note: Structural Columns are generally placed from top-down

Specify Height/depth in option bar BEFORE adding column

Floor Plan Level 2 (to place columns between Level 1 and Level 2)

Structure

Column

We’ll take the W10x49

Depth: Level 1 (get it….top-down?)

Place the column at Grid A1

Go to level 1. Is it there?

Cut a section to see the column. You may need to turn off the lineweight to see it clearly

Set the Display to Fine

Back to Level 2

Let’s place the columns “by grid”….it’s faster!

Structure

Column

W10x49

At Grids

Use a crossing window to select the grids (bottom right to top left). You can hold the control key to select the remaining grids not picked up by crossing.

Depth: Level 1

Green Check

See in 3D!

Add Footings for Columns

Floor Plan Level 1

Structure

Isolated

Sat “YES” to load family

Structural foundations….Footing rectangular….Open

Edit type….duplicate

Name it F3.0

Make the sizes: 3’0x3’0x1’0

Place at grids…..Finish

Note: You will not be able to see them because they are below Level 1!

Set Level 1 view range to view 2’0” lower

Save as structure1.rvt

Foundation Plan

Duplicate with Detailing Level 1

Name it “Foundation Plan”

Confirm the view Range is set to -2’0” (so we can see the footings)

Creating an exterior wall

Architecture

Wall

Basic wall generic 8” Masonry

Height: Unconnected 20’

Draw the wall 1’1 outside the grid line

Creating the footing

Structure

Foundation panel

Select: Foundation wall: Bearing Footing 36x12

Place on/attach to exterior walls

Save as: structure2.rvt

Creating Beams

Level 2

Set view range to -2’ depth, like we did on foundation. The beams will be between level 1 and Level 2.

Set display to Fine

Structure

Beam

W12x26

Draw a beam from Grid A1 to B1

Draw a beam from Grid A1 to A2

OK

Show “change reference” button to extend beams to web instead of flange.

Continue to draw beams horizontally and vertically between gridlines.

Creating a Beam System

Insert

Load family…structural framing….steel…..W-Wide Flange

W8x31….OK

Structure

Beam System

Automatic Beam System….W8x31

Fixed distance: 5’0

Pick inside top, left area between Grid A1 and B2. Note direction of beams

Set Elevation at 0

Create the rest of the beam systems (do not try to do all at once)

Note: No beams should be spanning the entire building!

Creating the Structural Floor

The structural floor is made of a metal deck w/ concrete poured on top.

Level 2

Structure

Floor: structural

3” LW Conc on 2” metal deck

Offset 5” (the thickness of the deck)

Make the boundary to the face of wall, not to gridline.

Cut a section to show the flooring and steel structure

Save as: structure3.rvt

Creating Framing Elevations

Framing elevations are generally used to create (diagonal) bracing.

Level 1

View….. Elevation…..Framing Elevation

Note: these work differently than regular elevations

Pick a spot on Grid B between Grid 2 and 3

See how the framing elevation snaps to it?

Go to elevation View

Display: Fine

Note only one bay in the grid is selected! The framing elevation is ready for “Bracing”

Bracing

In Elev 1a you just created:

Insert….load family….Structural framing….Steel….

LL-Double Angle……2L6x4x3/8LLBB

OK

Structure

Brace

Create at 45 degree angle

Show in 3D (no connection brackets yet)

# Revit 2018 Path Issues Solution

Windows….Control Panel…..Appearance and Personalization….Folder Options: Show hidden Files and folders….Show hidden files, folders, and drives

In Revit Architecture software:

R…Options….File Locations

Project template Files

It is empty

+

Should be:  C:\ProgramData\Autodesk\RVT 2018\Templates\US Imperial\default.rte

Default Path for Family Template Files

It is here:	\Templates

Should be here 	C:\ProgramData\Autodesk\RVT 2018\Family Templates\English_I\

Places

Library Path

It is here:	C:\Program Files\Autodesk\Revit 2018\

Should be here:	C:\ProgramData\Autodesk\RVT 2018\Libraries\US Imperial\

Detail Library Path

It is here:	C:\Program Files\Autodesk\Revit 2018\

Should be here:	C:\ProgramData\Autodesk\RVT 2018\Libraries\US Imperial\

# Spanish Character Floor Plan 2017-18

Software: Revit 2018

Design a one story home to the following program:

Spanish Architectural style (authentic as possible).

One story

Spanish massing.

2100 square feet +/- 50 square feet

Footprint no bigger than 70 feet long and 50 feet deep (this includes 2-car garage)

Living

Family

Kitchen

Entry

Dining Room

3 Bedrooms (one is the Master)

2 ½ Bathrooms

2-car attached garage (22’ X 22’does not count against square footage) 9’ plate height, two 9’0” wide X 7’6” high garage doors match style of front door

Entry Tower 13’-15’plate height (does not have to be part of internal area)

Authentic Spanish details for doors, windows, surrounds, roof, etc. Research!

Standard closet in secondary bedroom, WIC in master bedroom

Indoor laundry

Fireplace in Living or Family Room

10’ plate height.

8’0” head height for doors and windows

Courtyard (front and/or back)

Loggia off Master Bedroom.

Colors and exterior materials authentic to Spanish character

Step1:	 Bubble Plan. This is where you will organize your room placement, traffic paths, and roughly design your house (not) to scale. Add Roof Plan w slope arrows (color pencil).

Step 2: Detail Sketches: This is where you “study” the Spanish character and get your details together. Doors, windows, Gable, wrought iron, shutters, etc.

Step3:	 Elevation Sketches. Create 3 character elevation sketches (not to scale) with proper massing, potential details, and materials. These should be loose sketches.

Step 4: Elevation Blockout. Create 1/8”=1’0” scale elevation drawing with red pencil/pen. Details, correct roof, exterior materials, doors and windows should be included. Turn in copy for grading.

Step 5: Rendered Front Elevation Sketches. Three Spanish concept elevation sketches. Looking for different details…alternate ideas of the same house

Step 6: Draw construction documents in Revit. See next page for required drawings.

Revit Drawings required: See Checklists for Requirements

- Cover page (4 3D Views + List of Drawings)

- Foundation Plan.

- Floor Plan

- Electrical Plan

- Exterior Elevations

- Building Sections

- Concept Sketches taped to B-size paper

Drawings will be printed on B (11x17) sized paper on the HPM750 at 38% scale and saved to

X:\labs\brown\submit\Period 2 Arch Design

Presentation: Projects will be stapled top, left corner. Pages will be numbered according to provided class standard.

# A Home for Mr. Bond

Builder: WCB ARCHITECTURE

Software: Revit 2018

Specific Character (ex: French, Craftsman, Greek Revival, etc)

2 Story

4400 square feet (+/- 100 sq ft) does not include Garage

4 bedrooms total

4 bathrooms (one will be powder, one in Guest quarters, one in master)

“Guest” quarters: bedroom, bathroom, kitchenette, closet, private access

Home Office

3 Technology Spaces

“Bill Paying” Space

Living Room

Family Room

Formal Dining

Kitchen: Modern. Island, double oven, range or cooktop, microwave, ref, dw, wine chiller, double sink, vegetable sink in island.

2-car attached Garage

Indoor laundry

Mechanical Room (HVAC, Water Heater, Circuit Breaker Box)

Master suite (include WIC, bathtub, shower, his/her vanities, private watercloset)

Gas or “fake” Fireplace

Storage cabinets:

clothes

linens

books (maybe recessed into wall?)

Furniture (for spatial assistance)

Stairs authentic to the character

Sketches Required

1-2	Bubble Plans for 1st and 2nd Floor in pen Your name, Character Name, SHS

3	Front Elevation sketch (pen on flimsy) Black and White, foliage, shadow, GL, Your name, Character Name, “A HOME FOR MR. BOND”, black out windows. 1/8” = 1’0” scale

4-5	2 Colored, rendered Front Elevations (different options), foliage, shadow, GL, Your name, Character Name, “A HOME FOR MR. BOND”, black out windows.

1/8” = 1’0” scale

# Drawings Required  (you will have close to 20 sheets!)

All drawing on D size paper  with Arch border. Create own Revit title block. Proper sheet numbers.

(1) Cover (4 3D views NE, NW, SE, SW, rendered 3D, “A HOME FOR MR. BOND”, Table of Contents/Sheet List)

(2)Site Plan 	Use provided Site Plan. See Site Plan Checklist. Link Building into Site

(3)Foundation Plan 	See Foundation Plan Checklist.

(4)1st Floor Plan 	See Floor Plan Checklist.

(5)2nd Floor Plan 	See Floor Plan Checklist.

(6)1st Floor Plan Electrical 	See Electrical Plan Checklist. Include Electrical Legend.

(7)2nd Floor Plan Electrical 	See Electrical Plan Checklist. Include Electrical Legend.

(8,9)Exterior Elevations See Exterior Elevation Checklist

(10)Building Section 1, 2 with 3D sections (room labels, proper annotation)

(11) 2 Wall Sections w/ annotation. Include 3D Section boxes (small) of each. All one sheet.

Detail Sections w/ annotation (cornice, sill, garage, special wall framing).

Scale ¾” = 1’0”? All one sheet. Make fit. Apply “callout” strategy where applicable.

(12)Interior Elevations Kitchen See Interior Elevation Checklist

(13)Interior Kitchen Plan 3D. 	2 w/section boxes NE, SW and 4 interior renderings utilizing Enscape (NE, SE, NW, SW)

(14)Interior Elevations Baths, Office, Tech, Bill Pay, and Laundry See Interior Elevation Checklist

(15)Interior Baths, Office, Tech, Bill Pay, and Laundry 3D. 	w/section boxes and interior renderings utilizing Enscape

(16)Door Schedule (Tahoma font) same contents as last project

Window Schedule (Tahoma font) same contents as last project

Room Schedule (Tahoma font) Finishes, Dept, Area

(17)Written Proposal Required

400-500 words “explanation paper” describing what you did and why you did it….both inside and out. Use architecture terminology and resources if necessary. This would include explaining your details and why you used them. Copy word doc and paste on sheet 17 in 3/16” text.

Rendered Sketch. Put it on a Drafting View. Name it “Character Elevation Sketch”

I will grade the project in segments to make sure you are progressing to my timeline. No late work. You will have 1 week to revise work for re-grade. That work will likely need to be completed during after school hours. HINT: If you can get ahead……deadlines will be less stressful!

Evaluation

This will be 60% of your 2nd semester grade, and be your main portfolio component. It will be broken down as follows:

Sketches								15%

Written Proposal							10%

Technical Applications (Drawings)					30%

Creative Applications and Accuracy of Character (Drawings)	25%

Rendering								10%

Presentation (mounting, clean/neat, etc)				10%

Extra Credit (ONLY if all above is COMPLETE)			0-20%

SkillsUSA (regional and state) (applied to 1st and/or 2nd semester grade)	points assigned by effort, attendance after school, and results

ACE Mentors participation (applied to 2nd semester grade)		points assigned by effort, attendance after school

Revit User Group meeting(s) (applied to 1st and/or 2nd semester grade)	points assigned by attendance and verbal participation.

Characters:

- Andalusian (Southern Spain)

- Colonial Revival

- Craftsman

- Cuban

- Dutch Colonial

- American Farmhouse

- Folk Victorian

- French Country

- French Eclectic (Formal French)

- Georgian

- Gothic Revival

- Greek Revival

- Italianate

- Japanese

- Mexican

- Mission Revival

- Monterrey

- Prairie

- Queen Anne

- Richardsonian Romanesque

- Second Empire

- Shingle

- Spanish Colonial/Santa Barbara

- Stick

- Tudor

- Tuscan

# Residential Floor Plan Checklist

- Location of all doors, windows and walls (dimensions)

- Length of walls (sub-dimensions or overall)

- Dimension walls to face of core

- Door and window tags (for schedules)

- Properly designed hallways leading to bedrooms

- Kitchen designed with all necessary elements to include: sink, stove/range/oven, dishwasher, refrigerator, pantry, upper cabinets, base cabinets

- Fireplace (fake or Gas). No more wood burning fireplaces in California!

- Entry/Foyer

- Stairs (must be functional and match style of character)

- Linen closets

- Storage space: clothes, books, etc. Would like to see creativity here…recessed walls, surprise spaces.

- Properly designed hallway endings

- Bedrooms: Guest, Master, 2 secondary

- Bedroom closets

- Bathrooms with a minimum of: lav, W/C, tub and or shower with adequate space to move

- 4 bathrooms total: one in Guest Bed, one in Master bed, one for the 2 secondary bedrooms, and a powder room downstairs.

- Laundry room

- List square feet on drawing (4400 SQ FT+-100). Show 1st floor, 2nd Floor, and Gar. totals

- Living Room

- Family Room

- Home Office

- Tech Spaces

- Bill Paying Space

- Formal dining

- 2-car Garage

- Room labels: underlined, capitalized, include all rooms, hallways, key exterior adjacent features like porte-cocheres, loggias, tandem garage, courtyard, etc

- North arrow

- Doors: proper size, swing direction, and fit

- Marketable design

- Interior elevation marks

- Building Section marks where needed

- Wall section marks where needed

- Furniture will be later….just freeze for now. Just need fixtures

- Flooring: if tile, wood, stone, marble, etc show ½ tone gray (kitchen, bathrooms, dining, etc)

- Notes for dropped ceilings: give height at location

- Print medium detail (I want to see your wall construction)

- Complete title block with character name, scale, Santiago High, your name, Mr. Bond’s name, First Floor Plan, Second Floor Plan, etc. Logo OK.

# Bathroom Elements: Checklist

Toilet/Water Closet

Tub

Tub surround including splash on walls

Tub faucet

Shower (base with drain)

Shower glass door

Showerhead

Shower ceramic or glass tile surround

Vanity w/PLAM applied

Sink (lavatory)

Faucets (separate from sink)

Countertop (2’5 1/2” to start)

Splash

Towel Rack

Medicine Cabinet near sink(s)

Mirror over sink(s)

Paint: Semi-Gloss

Vanity Light

Fan/Light Combo

Vapor resistant light over shower and Tub

Ceiling Gyp (paint semi-white) same as Kitchen

Adjust flooring under Tub, Vanities.

Base molding where appropriate (best to do after placing bath fixtures)

Material colors should complement each other.

GFCI Outlets

Switches

Flooring (not under casework or fixtures)

# A HOME FOR MR. BOND: Floor Plan Design Notes

Area of each floor. You can get this by creating an “Area Plan”. Remember to count the stairs only once. Give total area (less Garage) on each Floor.

Turn Underlay off. RC. View Properties. Don’t want to see first floor walls on 2nd floor.

Title block family. You will create a custom title block.

Dimension Hallways. Looking for 3’6” wall face to face. No more. No less. Hallways should take you to rooms….that’s it!

Room size guidelines:

Master Bed: 225 sq ft. No less than 13’ clear.

Master Bath: ¾*225= 168 sq ft.

Master WIC 100+ sq ft

Secondary Bedrooms: 144 sq ft. No less than 11’ clear.

Guest Bedroom: 160 sq ft. No less than 12’ clear.

Office/Den Library: 160 sq ft. No less than 12’ clear. Try to keep “Open” (not 4 walls)

Tech: 20-50 sq ft

Bill Paying Space: 10-30 sq ft (generally a niche)

Mechanical Room: 50-80 sq ft. Clearance for tech to “service” fixtures

Living: 250 sq ft. No less than 15’ clear.

Family: 275 sq ft. No less than 15’ clear.

Kitchen: 200-250 sq ft. Make sizes divisible by 2’0”….10X12….12X14…..8X16…..etc

Stairs: 3’6 wide. Straight the least desirable. L and U shapes best. Authentic to character.

Garage: 22X22’ is typical 2-car. Can be 25’X25’ (“Full Double”) ATTACHED

Need:  Fixtures and casework, closets/storage, furniture, doors and windows, smart walls everywhere, dimensions, room tags, North Arrow, etc. See Floor Plan Checklist

Residential Electrical Checklist

- GFCI’s in Kitchen.

- 20 amp GFCI in Bathrooms

- AFCI circuit in Bedrooms (use a text note to specify….”DEDICATED AFCI CIRCUIT IN BEDROOMS”)

- At least one wall switch in bedroom. OK to outlet or light.

- Smoke detectors in bedrooms

- Smoke detectors in hallways leading to bedrooms.

- Hallway lights w/ switches

- S3 for stairs to light (above and below).

- Hidden line swings arcs (not straight)

- Ceiling lights in Kitchen

- Ceiling light fixtures displayed as solid lines, circles

- Room Tags (no area)

- Switches not behind doors

- Garage: one GFCI outlet min.

- Duplex Outlets every 12’0” linear

- Front outlet GFCI water proof

- Rear outlet GFCI waterproof

- Laundry outlet 20 amp GFCI

- Hallway outlets when walls are >10’

- Porch light w/ switch inside

- Light in foyer

- Bathroom: no lights within 3’ horizontally to tub/shower unless Vapor Proof

- Range, DW, and REF outlet

- Service panel

- Casework, plumbing fixtures, specialty equipment half-toned (so electrical outlets show better)

- Turn off Floor Pattern

- Use overlay method to show ceiling fixtures on electrical plan

# Cover Sheet Checklist

3D view of Building SE corner: Shaded View

3D view of Building NW corner: Shaded View

List of Drawings: In order of Drawing Sheets. “C” drawings first.

Rendering. Looking at the building from the Front view with a slight tilt to show depth.

Background pic file .png, Render materials applied to building, shrubs in front of building and cropping of building.

Title block to List: Your Name, Your Character, Client name, Santiago High Architecture. Large Text (use your best judgment). No border Lines. You will need to make a Title Block family.

# Site Plan Checklist

We will use the Site from the “Tuscan Plan”.

- Property lines and their length and compass orientation

- Corner elevation marks

- Size and Location of dwelling on site

- Size and Location of additional structures on site

- Contour lines/slope of property

- Street(s) adjacent to property

- North arrow

- Scale

- Driveway size and location

- Sidewalk

- Fencing: Location, height, lineal footage

- Lot coverage %

- Legal address

- Type of Construction  (V-NR)

- Occupancy  (R-3)

- Square footage of dwelling and additional structures

- Year of building code

- Date of last revision

- Gas line

- Water line

- Electrical service panel

- Setbacks (dimensioned and labeled)

- Dimensions set to 0’0”. No small fractions.

- Property line and dwelling created with thick lines

- Title block completed and fit on paper correctly

- If building is on Site, turn off roof pattern

- 2D Site plan no color

- Materials on Site

# Presentation Plan Checklist

- Room Labels with Name and Area. Capitalize. No number

- Color walls black. VV….. Walls… Cut pattern solid fill.

- No annotation (tags, dimensions, etc)

- Color Scheme by Sleeping, Living, Corridor, Non-Living, and Service

- Color Scheme Legend

- Pastel colors for Color Scheme

- List Gross Area of Level

- List Gross Area of Garage

- Show Furniture and Fixtures

- Turn off Floors

# Interior Elevation Checklist

Kitchen, Baths, Tech, Bill Pay, Laundry

- Vertical and horizontal dimensions using assigned dimension style.

- Tag plumbing fixtures PF, Lighting Fixtures LF, Casework CW, and specialty equipment SE

- Upper Cabinets

- Base Cabinets

- Outlets and switches on countertop.

: 3/8”=1’0”

- No color

# Foundation Plan Checklist

Dimension location of foundation footings with class dimension style

Overall dimensions

Garage to house footings display correctly: with hidden line-solid line-hidden line. The solid line would be where the garage drop is.

Section marks

No elevation marks

Interior bearing wall footings represented with hidden lines 12” wide

Exterior footings represented with a solid line (for exterior of slab) and a hidden line for edge of footing underground.

Block-out for garage doors displayed and noted.

North arrow proper direction (reference site plan)

Concrete notes:

- 5" CONC. SLAB , 6" BELOW HOUSE SLAB, 0VER 5" GRAVEL FILL - SLOPE 1/4" MIN. PER FT. FOR DRAINAGE

- 5" CONC. SLAB OVER 5" SAND FILL W/ 6X6-W2.0XW2.0 WWM PROVIDE .006 VAPOR BARRIER & 2" X 24"RIGID INSULATION AROUND ENTIRE PERIMETER

- BLOCK OUT FOR 8’0” GARAGE DOOR

- BLOCK OUT FOR 2’8” GARAGE DOOR

- 2 X 6 DFPT SILL W/ 7/8" X 20" A.B. @ 6'-0" O.C.  17" MIN. INTO CONC & 12" MAX FROM ANY CORNER W/ 3" DIA. WASHERS.

.

# Exterior Elevation Checklist

- Vertical dimensions using assigned dimension style. Dimension from Floor line to top of doors and windows, not sills.

- NO horizontal dimensions, except for roof overhang

- Smart exterior materials to be tagged

- Model Rafter Tails 2’0” O.C. if present

- Model Shutters if present

- Model Balconettes if present

- Model Chimney.

- Use Model Lines for Wrought Iron on door and shutter dogs.

- Windows and Doors should look exactly like the elevation drawings provided

- Door and Window tags

- Roof Pitch symbols

- Roof Pattern lighter. Smart material tag to be applied

- Roof gutter applied where appropriate. Downspouts for ambitious students.

- Do not show stucco as pattern on elevation

- Freeze toposurface

- Hidden lines for foundation (detail lines)

- Thick ground-line (use 7 lineweight)

- Floor line is hidden (linework…may need to go to wireframe to set up)

- Lineweights should have “POP” on all elevations

- No color

# Kitchen Plan Checklist

Assign appliances, flooring, plumbing fixtures, casework, island (optional), ceiling, lights, and countertops to the Kitchen.

Create a Presentation Plan for the kitchen to include:

- Rendered View of Kitchen

- Presentation View of the Kitchen Expanded Plan

- Two, 3D Section Boxes

- Two, Interior Elevations with “smart” material tags. You can duplicate the existing Kitchen Interior Elevations.

# Section Checklist

Building Section Checklist

Scale: ¼” = 1’0”

Detail Level: Medium

NO COLOR

Room Tags

Callouts for Wall Sections and/or Detail Sections

Insulation

Turn Off furniture, fixtures

Roof Truss

“Title on Sheet”over-ride. Describe Section….Ex: “Section Thru Kitchen North View”

2X4, 2X6, etc as detail components

Ground under sand/gravel (as filled region)

2” Sand/gravel under slab(as filled region)

Ground line label

Ridgeboard

2X6 ceiling joist (detail lines)

2X6 double top plate

2X6 Sill Plate

2X6 under windows

4X6 Header over doors and windows

Slab thickness dimension

Footing width dimension

3D view of Section in color

Wall Section Checklist

Detail Level: Medium

NO COLOR

2X4’s, 2X6’s etc

Insulation

Annotation for wall and roof elements seen in this view (Example: 5/8 GYP, 2X4, 1/2PLY, .006” VAPOR RETARDER, 1” STUCCO)

Callout for Footing(s) Detail

Callout for Eave Detail

Scale: ½”=1’0”

“Title on Sheet”over-ride. Describe Section….Ex: “Section Thru Kitchen North View”

Turn OFF crop view

2X6 TREATED SILL PLATE (at foundation)

Footing Detail Section Checklist

Detail Level: Medium

NO COLOR

Copy to clipboard…paste same place…all 2D elements in Wall Section

7/8” Anchor Bolt - 20” DEEP @ 6’0” O.C.

#4 rebar

Welded Wire mesh:  6X6-W2.0XW2.0 WWM

Concrete floor and concrete footing should be joined (graphically)

POP of exterior footing

Annotation for wall and footing elements seen in this view

Scale: 1”=1’0”

“Title on Sheet”over-ride. Describe Section….Ex: “Section Thru Kitchen North View”

Break Line

Turn Off Crop view

Eave Detail Section Checklist

Detail Level: Medium

NO COLOR

Roof to wall elements (like double, top plate, ceiling, roof joist, brackets, etc)

Smart Roof Elements

Copy to clipboard…paste same place…all 2D elements in Wall Section

Annotation for wall and roof elements seen in this view

Scale: 1”=1’0”

“Title on Sheet”over-ride. Describe Section….Ex: “Section Thru Kitchen North View”

Break Line

Turn Off Crop view

# Schedule Checklist

Door Schedule

Mark			Door #

Family			Door Style

Type			Door Type

Level			Floor #

Thickness		THK

Height			Height

Width			Width

Head Height		Head Height

Frame Material	Fr. Mat

Comments		Notes

From Room: Name	From Room: Name

To Room: Name	To Room: Name

Subdivide Thickness, Height, Width under category “Door Sizes”

Notes: “Hollow Core” or “Solid Core”

In this order.

Sort by Mark

Make sure schedule columns look proper on sheet!

Window Schedule

Count			QTY

Type Mark		Win #

Level			Floor #

Family			Window Style

Height			Height

Width			Width

Type Comments	Notes

Uncheck itemize every instance

Sort by: Type Mark then by Level

Notes: “Dual Pane Glass, Gas Filled”

Make sure schedule columns look proper on sheet!

# Project 2 Assignment

# Revit Resources

Santiago Architecture’s website

http://www.green-architecture-corona.com/

Autodesk Education Community

This is where you can download Revit 2018 and AutoCAD 2018 for free. It is a 3-year license. Your computer will need at least 8 GIG RAM and a compatible video card. It takes a couple of hours…minimum.

https://www.autodesk.com/education/home

Revit City….for Revit content. (You will need to sign up….it’s FREE)

Note: you can download Revit families at Architecture…..you do not need Revit to download them. Just download to a USB drive or CD.

http://www.revitcity.com/index.php

bimobject

This is Autodesk’s place to get additional families. Families from this site are generally pretty good.

http://bimobject.com/en-us/product

AUGI-Revit

This is a great place to get your questions answered. Note the “Student Support” section!

http://forums.augi.com/forumdisplay.php?22-Revit-Student-Support

Revit User Group Inland Empire

This site will give you information about the monthly Revit User Group meetings in Rancho Cucamonga/Ontario

https://www.meetup.com/BUG-IE/

South Coast Revit User Group (SCRUG)

This site will give you information about the monthly Revit User Group meetings in Orange County

https://www.meetup.com/SC-RUG/

Big BIM Theory

This site will give you information about the monthly Revit User Group meetings in San Diego

https://www.meetup.com/Big-BIM-Theory/

# Curtain Systems

Curtain systems are not walls, and they are not windows.  Like walls, they can define space and separate the exterior from the interior. They are typically  not load-bearing and are not cut for doors or windows.  Like windows, they can usually include mullions and have glazed panels. Unlike windows, curtain systems are usually assembled on site as a single unit.

A typical curtain system comprises a wall, panel, grid lines, and mullions, and you can change these elements individually. This affects the entire curtain system. For example, to resize the system, you need to change the length of the wall. To switch panel types, you need to select a panel. To change grids, you select the grid.

You can add curtain systems with  the wall command, or you can use a specific curtain system command.

Flat Curtain System

In this lesson, you further  develop the building information model by creating a flat curtain system at the entrance of the model.

Creating an Entrance

In this exercise, you create a curtain system using the wall command.  This type of curtain system is also referred to as a curtain  wall.

Provided Training File

■    Click File menu ➤ Open…..2014CurtainWalls.rvt

1 In the Project Browser, expand Views (all) ➤ Floor Plans, and double-click  Ground  Floor.

2 Zoom in to Grid F where it meets the arc in the floor.

This floor  edge will  be the entrance to the building.

Creating a curtain system using the wall command

3 On the Architecture tab of the Ribbon, click Wall….Architectural Wall

4 In the Type Selector, select Curtain  Wall: Curtain  Wall 1.

5 Start the curtain system where shown.

6 Move the cursor down along the edge of the floor, and finish  at the outside face of the exterior wall as shown.

7 Zoom in to the join between the curtain  system and the exterior  wall.

8 In the Project Browser, expand Views (all) ➤ 3D Views, and double-click  Southeast Isometric.

9 View Cube: NE

10 Zoom in to the curtain system you just added.

11 Select the curtain system…Properties

Note that a curtain system created from the wall command has similar properties to other walls:

base constraint,  top constraint, top and base attachments,  and room bounding. You want the curtain system to attach to the roof.

12 In the Element Properties dialog:

■    For Constraints ➤ Top Constraint,  select Up to level: TOP OF ROOF.

■    For Top Offset, enter 4'0".

■    Apply

The curtain system is now at the same height as the other existing curtain systems.

13 The curtain  system is a single glazed panel; you are going to subdivide the panel into  several smaller panels, using curtain  grids.

Adding curtain grids

14 In the Project Browser, under Floor Plans, double-click  GROUND FLOOR.

15 On the View tab of the Ribbon, click Elevation….Elevation…Building Elevation (properties)

16 Place the elevation symbol in front  of the curtain  wall and click Modify on the Ribbon.

17 Click the elevation arrow to display the crop boundary, and resize the crop boundary as shown.

18 In the Project Browser, right-click Elevation 1 - a, and click Rename.

19 Enter Entrance Elevation, and click OK.

20 In the drawing  area, double-click  the elevation  symbol arrow to open the new elevation  view.

21 Architecture…. . Curtain  Grid.

22 Move the cursor along the edge of the wall, and click to place a horizontal curtain grid 4'0" above the ground floor  level line.

23 Place another grid 5'0" above the first grid.

24 Place another  grid so that it snaps to the SECOND FLOOR level line.

25 On the Ribbon, click Modify.

26 While  pressing CTRL, select the 3 grid lines you just placed. Pick the 2nd Floor grid line first to make the process easier.

27 Click Edit menu ➤ Clipboard…Copy

28 Click Edit menu ➤ Paste ➤ Paste Alignedtoselectedlevels

29 In the Select Levels dialog, while pressing CTRL, select SECOND FLOOR, THIRD FLOOR, FOURTH FLOOR, FIFTH FLOOR, SIXTH FLOOR, and SEVENTH FLOOR, and click OK.

The selected grid lines are now at each of these levels.

Save the file as: FirstLastCurtainWall.rvt

30 Architecture…. . Curtain  Grid.

31 Place the cursor on grid 2 so that it highlights. Click to create a vertical grid.

This divides the curtain wall vertically into  2 panels, one larger than the other.

32 Move the cursor along the GROUND FLOOR level (left of the vertical grid line) until it snaps to the midpoint of the larger vertical panel. Click to place another  grid line.

TIP To be sure that the curtain grid is at the midpoint of the panel, watch the tooltips and the Status

Bar.

33 Architecture…. Modify.

Next, you add a doorway to the curtain  system. Instead of using the Door command,  you set up a doorway on a curtain  panel, and then you replace that panel with  a curtain  system door panel.

Try Placing Door

Ground Floor in Browser Architecture….Door…..Single door. Place in wall.

Did the door go in? Problem?

Adding the doorway

34 Zoom in to the ground floor  level in the Entrance Elevation view.

35 Select the left vertical grid line, and on the Options  Bar, click Add or Remove Segments.

You enter an editor that lets you select segments of the grid line to remove them.

36 Select the lowest segment of the left grid line, and then select the segment above it.

The segment line style changes to dashed.

37 Click in any white  space to exit the editor.

The two segments are removed.

38 Using the same method,  remove the horizontal segment that intersects the vertical segment removed in the previous step.

39 Architecture…. Curtain  Grid.

40 On the Options Bar, select One Segment.

41 Place 4 vertical grid lines as shown.

42 Place dimensions  as shown. Remember, the line you pick is the line that moves. Pick the 1’6”

lines first!

TIP You may need to adjust your zoom settings to view the dimensions.

43 Delete any dimensions.

44 You now have two 6-foot wide panels between smaller rectangular panels.

45 Select the horizontal grid line that is 4'0" above the ground floor.

46 Click Add or Remove Segments on the Options  Bar.

You are going to add more segments to an existing curtain  grid.

47 Click the horizontal grid line between the smaller panels. The line style changes from  dashed to solid to indicate  a grid segment has been added. Do not click between the 6-foot panels.

Use the following image as a guide.

TIP The middle grid lines are centered between the long vertical grid above them. To get the horizontal grid to display between the vertical grids, click once to the right of the long grid and then click once to the left of it.

48 When the grids are placed (line style have changed to solid), click in any white space to exit the editor.

Next, you replace the 2 larger 6-foot panels with  curtain  system doors.

49 Insert➤ Load Family.

50 Doors….Curtain Wall-Store Front-Dbl.rfa….Open

51 Select the left 6-foot panel. You will  have to press TAB to highlight it. Watch the Status Bar to be sure you are highlighting the panel.

52 On the Type Selector, click Curtain  Wall-Store Front-Dbl: Store Front Double Door.

The panel changes to a double door.

53 Use the same method  to replace the other panel with  a double door.

54 On the Ribbon, click Modify.

These panels schedule as doors, not as curtain panels. They are part of the curtain panel category.

55 In the Project Browser, under Floor Plans, double-click  GROUND FLOOR to see the plan view door swings of the curtain  wall doors.

Now, on the new curtain  system you added, you replace some of the transparent panels with solid ones.

Save your file as: FirstLastCurtainWall.rvt

56 In the Project Browser, under Elevations, double-click  Entrance Elevation.

57 On the View Control Bar, click   , and click Shaded. This changes the graphics style of the view.

Next, you change panels in front  of ceilings from  glazed to solid.

Changing panels

58 Zoom in to the FIFTH FLOOR level.

59 Select one of the panels below the FIFTH FLOOR level line.

60 On the Type Selector, click System Panel : Solid.

The glazed panel is changed to a solid panel.

61 With the panel still  selected, click P r ope r t i es

62 In the Element Properties dialog, click Edit/New  to open the Type Properties of the panel.

The Offset property  specifies the distance from  the centerline  of the curtain  wall. Thickness specifies the depth of the panel, and Material  specifies the shading and patterning.

63 Click OK twice.

64 Replace the two adjacent glazed panels with  solid panels.

65 The glazed panels display in blue, and the solid panels display in white.

66 Change the remaining glazed panels in front  of ceilings to solid panels.

67 A little trick! Modify….Match TypeProperties (paint brush)..Pick the solid panel….now pick the panel next to it. This avoids having to TAB TAB TAB to cycle to what you need (curtain panels. DISCUSS

68 Save your file as: FirstLastCurtainWall.rvt

This completes the exercise for creating an entrance.

Adding Mullions to the Curtain System

In this exercise, you place mullions on curtain  panel grids.

Training File:  Open FirstLastCurtainWall.rvt

1 Architecture…. Mullion.

2 In Ribbon, select Grid Line Segment.

3 Place a mullion on the grid segment at the ground floor immediately  to the right of the right set of doors.

4 On the Options  Bar, select Grid Line.

5 Place a mullion on the horizontal grid that is 4'0" above the ground floor.

6 On the Options  Bar, select All Grid Lines.

7 Place the cursor on any empty grid segment on the curtain  system and click.

Now all empty grid segments have mullions on them; however, there are a few that you do not want, so you remove them next. The two mullions below the doors are not necessary, because their   width reduces the  size of  the  doors…..and people would trip over them entering the building!

Removing mullions

8 Zoom in to the set of doors.

9 Pick the mullion below the doors

10 Delete the mullions below them.

Mullions can change their joins to other mullions. You are going to change some mullion joins.

Changing mullion joins

11 On the View Control Bar, click Model Graphics Style ➤ Hidden  Line.

12 Select the vertical mullion above and between the set of double doors.

Two mullion join  controls  display.

13 Click the lower mullion join  control.

The bottom of the mullion meets the top of the horizontal mullion.

14 Click the top mullion control.

The top of the vertical mullion now meets the bottom of the horizontal mullion above.

TIP After selecting the vertical mullion, you can also right-click, and click Join Conditions ➤ Break at

Join to break the mullion at both joins.

15 On the Ribbon, click Modify.

16 In the Project Browser, double-click  Southeast Isometric.

17 Save the file.

This completes the exercise for adding mullions.

This also completes the lesson on creating a flat curtain  system. You learned how to create a basic curtain wall system and how to subdivide  it with grids. You also learned how to modify grids and change panels. Finally,  you learned how to add mullions and change their joins.

Save your file as: FirstLastCurtainWall.rvt

Curved Curtain System

In this lesson, you create a curved curtain  system near the entrance of the model that was just completed. You also create a custom curtain panel for the system. Finally, you add both predefined and custom mullions to the system.

Adding a Curved Curtain System

In this exercise, you add a curtain  system using the wall command.  The curtain  wall is sketched as an arc.

Training File:  Open FirstLastCurtainWall.rvt

1 In the Project Browser, under Floor Plans, double-click  GROUND FLOOR.

2 Zoom in to the circular space above the entrance that was just completed.

3 On the Architecture… Wall.

4 In the Type Selector, select Curtain  Wall : Curtain  Wall 1.

5/6 Properties (yea…I skipped a number…)

■    Under Constraints,  for Top Constraint,  specify Up to level: FIFTH FLOOR.

■    For Top Offset, enter 4'0".

■    Click Apply

7 On the Ribbon, click   (Arc Start-End-Radius).

8 Start the wall by clicking  at the left edge of the circle at the centerline  of the intersecting  wall.

9 Place the second point  at the lower right  side.

10 Finish the wall by clicking  at the top of the arc.

(look for midpoint…like an OSNAP)

Check it out in 3D. See the arc wall? NO?

Back to Ground Floor

The curtain  wall displays as one flat panel between the first and second points placed. Next, you place grids on the system. The curtain  grid command  snaps only to the curved arc. You are going to use one of these snaps points.

11 Architecture….Curtain Grid.

12 Snap the cursor to the midpoint of the curved arc and click. Watch the Status Bar at the bottom left corner of the screen to ensure you are at the midpoint.

13 Continue to place more grids by using the snap points on the arc. Divide the halves into quarters, eighths, and then sixteenths. It gets easier as you attempt smaller segments.

14 In the Project Browser, under Elevations, double-click  East.

15 Zoom in to the cylinder, and snap the cursor to each level line and click to add horizontal curtain grids. Do not add a grid on the GROUND FLOOR level.

16 Next, you change some panels in the system.

Using walls as panels

17 Using a selection box, select the bottom layer of panels. Remember to click   to filter  out all other elements from the selection except Curtain  Panels.

18 In the Type Selector, select Basic Wall: Generic - 12" .

19 Save as:  FirstLastCurtainWall.rvt

This completes the first exercise for creating a curved system. Next you create a custom panel and add it to the system.

# New Features in Revit 2018

Railing hosted to Topography

Collaboration for Revit (work-sharing on the cloud)

Schedule Groups and Links

Global Parameters

Multi-story stair improvements (you can connect stairs to the levels)

“SDS2 Connect” Share your SDS2 Model with Revit. Sorry, connections do not come in

https://www.autodesk.com/products/revit-family/features