Santiago High School

SDS/2 2016 Structural Steel Detailing

Hands-on Tutorial

Contents

Resources	4

Detailer Training Series Online Course	4

Starting the Project	5

Job or Fabricator Options	6

Modeling	10

Construction Lines	14

Grid Lines	16

Create Plan Views	18

Plate Definition Schedule	20

Columns	26

Beams	31

Roof Plan	31

Joist	37

Interface	38

Bracing	39

Vertical Braces	43

Front Deck	50

Stairs	61

Handrail	71

Group Members	76

Sequencing	77

Hip Roof	78

Material	80

Detailing Erection Views	84

Erection Views cleanup	92

Detailing Members	94

Detailing Submaterials	100

Sheet Outlines	102

Labels	104

Gather Sheets	108

Loading sheets	110

Detail Sheet Autoloading	112

Gather Sheets	114

# Resources

AISC American Institute of Steel Construction

https://www.aisc.org/content.aspx?id=38846

### Detailer Training Series Online Course

The AISC Detailer Training Series was originally developed in 2000 by AISC and the National Institute of Steel Detailing (NISD) as a series of 12 CDs. Thanks to funding from the Ironworker Management Progressive Action Cooperative Trust (IMPACT), the series is being made available as a free web-based service. This online course provides an introduction and overview of the roles and responsibilities of the steel detailer. This program consists of 10 modules -- most with multiple parts. Each of these parts consists of a series of videos, followed by a quiz pertaining to the concepts covered in that section. After completing all 10 modules, the user may take a final exam. Upon passing the exam, the user is issued a certificate of completion.

The Detailer Training Series is free, and open to everyone. You work thought the course at your own pace, and it takes approximately 100 hours to complete. It is STRONGLY recommended that you go through these videos if you want a job in the industry or plan on Structural/Architectural Engineering as a major in college. They are a little dated…..and a little dry…..and a little confusing. They make more sense as you go through the training.

# Starting the Project

Change Project (upper left corner of Main Menu)

New

Name Job: FirstLastj

Name Fabricator: FirstLastf

Job or Fabricator Options (under Setup/Utility)

Job Setup (tab) top, left

Note: Resource Center bottom right: forum posts jobs, etc SHOW

Design Criteria (Review) Note ASD 14th edition (the current code) OOTB settings in SDS/2 SHOW

Note: Help menu…shows really good info…SHOW

Plate Design Criteria	Show Help again	OOTB

Show shared gusset, beam/column gusset, stiffener clearance

Moment Plate Design Criteria 	Show Help again	OOTB

What is a “Moment”? Take a moment to watch the video (me so funny)

https://www.youtube.com/watch?v=NULtvdoq1rQ

Weld Design Criteria

Field Welds, Shop welds

Students watch AISC Module 3A #2-8 and 19-22

Bolt Design Criteria 	Show Help again	OOTB

¾	A325N

AISC 14th Edition Design Code

Nut and Washer Schedule

Washer Criteria (Note: RC in field to get decimal equivalent….RC to go back to fraction)

Zone and Sequence (in Job Setup)

Zone and Sequence

Max: 	4 	tab

See Zone names 1,2,3,4? That allows for sequencing the job……like a “phase” in Revit.

Ok…OK

We will use these later when we add “Sequences”

Provide students with SDS2 “Online” drawings

Note Format for entering dimensions: 99-2	not 99’2  or  99 2	16 (would be 16 inches)

2 ½ (space between)  SHOW

# Modeling

Save new view as Ground Level

Set the Elevation to 106-0

Note intersection is 0,0

See the green box? That is “Terrain”

See the TV icon in the top left corner? That is your “Display Options”. (DD also works)Pick the TV icon.

Uncheck Terrain	OK

Bam! Gone!

Change the Background color:

Options…..User Options….Colors

Set to a light background so we can see the darker colored lines. You could do dark background w light lines….

Show drop-down menus

Show tear off (dashed lines at top of window)

If “select Items” is showing, there are no current commands going

Pan MC (middle mouse click)

Select LC (left click)

Menu RC (right click)

Show x,y,z

Show Depth Check

Show collapsible icon arrows for more model space

Show Point Locators (like OSNAPS)

Show alt key and shift key to change commands in those colored boxes

Options

Toolbar configuration

Left click and drag ON

Give students a couple commands to add to their toolbar

Make one: Solids Opaque all

Make one: Undo View change

Save changes …just save over name

For saving default toolbar

Options…User Options

Configuration Files

Modeling

Browse….name…OK

Show: Options….keyboard shortcuts (Give students shortcut sheet)

Note: keyboard shortcuts save to toolbar file

# Construction Lines

Grid Lines S1: Go to that sheet

Construction lines don’t print. They just help with placing things like grids, columns, beams, bracing, etc. Their intersections are key. They can be deleted when you are done with them. As you get into the process, you will want to delete some to make the screen less cluttered.

Construction Lines icon

Finite Construction Line Icon

NOTE: LOOK AT THE PIC ABOVE!! BASE OFF CONSTRUCTION LINE when laying out construction lines.

Layout construction lines per drawing Ground Level

Show a finite construction line….then how to delete it (using filter)

SHOW DELETING CONS LINE

Model….construction line….erase  or (CS keyboard shortcut)

Show Ruler (to get distances)

NOTE:  +

-

+

-

Note:Grid A1 should be in the top, left corner

A	B	C	D

1

2

3

4

Note: Plan Views are “Erection Views”

To make Grids from construction lines:

# Grid Lines

Grid bubble setback in view properties. Use 6’-0”

Check Finite

View Depth (bottom to top) like section

Top: 126-0

Bottom: 99-2

Double clicking on bubble takes you to view. Some with Plan view names (in section)

Make A.1 and B.1 and 1.5 Secondary views with 3’-0” grid bubble setbacks

Rename Grid/View

Utility Function on main Menu…..rename job items…

Modify Erection View

Select Filter to Grid Lines (white box)….edit (double click)…then back to default.

DELETING A GRID LINE: File…..Views/Grids…..Delete View

MOVING A GRID LINE: Make the Grid line where you want it. Name it exactly the same as the mis-placed grid line. Accept the over-ride message. The grid line represents a view (like a section)

Filter: Change it back to “Default” Keep an eye on this choice….they change as you perform different tasks! I guarantee you will try selecting an object and won’t be able to pick it because the filter setting wasn’t changed GUARANTEEIT!

# Create Plan Views

Plan View Icon

114-6

Save View as: First Floor

Ok

Plan View Icon

99-2

Save View as: Basement Level

Ok

Plan View Icon

107-2

Save View as: Anchor Plan

Ok

Plan View Icon

126-0

Save View as: Roof Plan

Ok

Note (again): If Grid is in wrong place, create another grid….give it the exact same name….OK to replace…Yes. You can’t move a grid in SDS2

Note: Deleting a Grid line deletes the view. Am I getting repetitive??

# Plate Definition Schedule

Main Menu

Job or Fabricator Options (under Setup/Utility)

Job Setup

Base/Cap Plate Schedule (this is where you will create the plates that go at the bottom of the columns…that attach the columns to the concrete slab). See sheet S3.

# Columns

Open Columns Sheet S2 AND Plate Definition Schedule Sheet S3

H is 0 degrees

I is 90 degrees

Note Columns   		90 degrees

0 degrees

At A1, place W10X49		90 degree rotation

Bottom: 107-2	Top 126-0 (remember format!!)

Input connection: user base/ cap plate

BP1

Place same column at C1 (exact)

Columns

Format:

W10x49

W		10		x		49

Shape		depth				weight in lbs/inch

Note Columns   		90 degrees

0 degrees

Column (icon) at left. Note: See flyout arrow to change members.

Place column using “Intersection of Construction Line”. Note: they don’t always stay selected.

A2 changes a little…

Still W10x49	rotate 0

Same cap plate

+9 ½ ….so….126-9 ½

C2 is same as above (use middle click for repeat)

B2 is same as above, just rotation 90

C1.5	column rotate 0

NOTE: C1.5 Top of Column is 114-6 (stops at first floor)

(126’-0” – 11’-6” = 114’-6”)	-11-6 so end is 114-6

B1	W10x30 (same as above)  Note: W10x30 has BP2

B1.5 Has a splice. The first W8X40 goes from 99-2 to 107-2. BP1 for base. Splice plate for top. The second (top) has a base of splice, and a top of Auto Std. 107-2 to 114-6.

See sheet S3. HSS start at First Floor, not Anchor Plan. You don’t have to be in the First Floor to start the column there.

HSS6x6x1/2

Auto standard

114-6	126-0

# Beams

Open Sheet S4 Beams

First Floor (114-6)

Beam

W18X50 from A1 to A2 NOTE: Intersection of Construction Line!

Do middle click at C1 to repeat

A1 to B1	W18x40

Repeat at B1

Repeat at B2

A2 to B2	W18x55

B1 to B2	W14x43

A1.5 to B1.5 	W12x26

MC to repeat at B1.5

Add construction lines for others…change pen colors….5-0 for filler beams

MC to repeat….remember, left side match!

# Roof Plan

Open Sheet S5 Roof Plan

Roof Beams 126-0

Careful…watch elevations!	126-9 ½	Roof slopes!!

A1 (at 126-0)	A2 (at 126-9 ½)

W14x43

OK

MC to repeat at C1

B1 is W12x26 (to B2)

Use “exact Point”for A2 to B2 W12x26 (not intersection construction line)

Show Member Tree

A1 to A.1	W12x26

Repeat B.1 to C

Roof Openings

Construction lines (from 2)	-5-6(change color)	-5-0

From A	7-6	5-0	web normal

Beam

Intersection of Cons Line/Member

Do W12x26 (both sides of member)

Web rotation: web normal

Opening C10x20 on left (keep Intersection of Cons Line/Member)

On right: Toe direction IN

# Joist

Joist (in member)

Exact Point	10K1	seated input

Process!

Solid

Shift/middle

Uncheck depth

View: solid types/views

Joists are just stick lines (not processed)

You can solid style selected

# Interface

Shift MC to rotate

View….undo view change….”U” on keyboard…

Cnt r	back to last open view

Shift LC to add to selections

Cnt LC to subtract from selections

RC on member…..select other…select all…get all of same type

Windows (same as Revit and AutoCAD)

Selection filters (top of screen)

Esc does not clear selection…LC in open space does

Model Tree-can select from there

Copy members: select member….RC…copy member

Options: single (2 pts), linear (horiz or Vert), fill (like an array)

Must say “YES” to accept

Mirror under copy

To move a member: select…move/stretch

No UNDO

F5 Process

To stretch:

Green box is “member end” (can use selection filter)

Select member end….RC…..move/stretch

If both member ends were selected, you could move (not stretch) member.

# Bracing

Sheet S6

Start Horizontal bracing on First Floor

First Floor

Too many construction lines…..so…..

Model….Cons Line…Erase

Shift select	enter or RC

Member add…horiz brace

Int Cons Lines

L4x3x5/16

Process!	Solid

See Gusset Plates? They are bolted to the top of the Flange on the beams

Other brace on right (-3)

Trick on -3	x,y,z filter on bottom of screen	use int of cons lines

Set Z to -3

X icon to zero out filter (careful, or it will stay for the next operation)

# Vertical Braces

Sheet S7 Vertical Braces (at A)

View A

Set work points to ½ nominal depth of beams framing to. See 9”. Half of 18” depth

Construction line

Member….add Vertical Brace

Failed connection later

Show shop welds and field bolts

C View

See same construction lines?

View 1

Add construction lines	-6 from roof	-9 from first floor

Plus blue crossing

Failed Connection:

There are many reasons a connection fails.

Check the accuracy of your modeled elements. Do you have the right base plate? Is the member facing the correct direction? Is it the right member?

Shear Load Kips could be too high. Set lower. Get out. Get back in again. See what it needs. Set it.

Main Menu….Job and Fab Options……..Job Setup…….Design Criteria….% of uniform comp load: change 50 to 25

Make it a Forced Connection

Make it a Graphical Connection

Vertical Braces (3)	Pipe

2-L3x3x3/8 (the 2 is “double material”) Type it : L3X3X3/8	No “2”

Some failed connections

See gussets attached to column base?

See connections at Bracing?

# Front Deck

Sheet S8

First Floor (114-6)

Create construction lines

Beam	W14x22	Remember….left to right!

W12x26	Use -2 1/2 in Z at bottom

X clear -2 ½

Member add joists	10K1

Connection Specification: bolted

MC to repeat joists (at left)

Process		Solid

Plates are “dummy plates”

Columns

Roof Plan

Erase construction lines

Need new construction lines for rolled canopy

Beam	W12x26 (2)

The W12x50 have a “rolling operation”

Rolling operation: strong axis

Rolling radius: 38-10

Process		Solid		See in 3D!

Roof Elevation

Need columns

Issue: A.1 and B.1 do not show the area we need. Go to B.1. Don’t see any members past Grid 1

When we made B.1, we only went from 1 to 3….so we don’t see any members past Grid 1 (they are sort of cropped out)

Go to Roof Plan

We are going to mofify A.1 and B.1 so we can see past grid 1

We do it by re-creating the grid (carefully) and saving as the EXACT same name.

Add Grid A.1	Say Yes to Replace

Open B.1 	See canopy?

Column		Pipe 5 std

Create grids @ 6-0, 2-0, 1-0

Process		Solid		3D

Show pipe and base plates into beams

A.1 and create same columns as B.1

Process		3D	Show all in 3D

See how it is cluttered with all the annotation?

DD (display options/TV icon)

You can turn off different annotations like piecemarks and member #’s

You can save and area of your plan (like the front deck)

File	Save Area	Crossing window

Name it “Front Deck”

Nice to work in small areas?

# Stairs

Look at the Stair Tread Definition Schedule

We will have a bolted, grating stair

First Floor

Stair Landing above Grid 2

Setup construction lines

1-6, 3-0, 6

Left arrow 3-6		3 right arrow

Create beams W12x26

W12x19 (2)

Columns HSS3x3x3/8

BP2	User base/cap plate

Watch out for #2 heights!

Process	3D

Some failed connections

Member….stair

Esc to end

See issue with stairs? “Coping” is needed

Pick Stair….edit

Note: Connections are not generated. No clash either

Reprocess stair

Do same for bottom

Notice stringer is going to center of beam web!

Process

Still no connections. We will build later.

To make stringer extend past first riser, (by 6” in this example)

# Handrail

S10

Front Deck View

Get rid of extra construction lines

Add construction lines

=/- icon is relative depth icon

3 7/8 both

5” down (see Section A-A)

Make Rail	NO

Handrail Overview

Rails

Post/Pickets default

Main Section def		p-end both

Uncheck depth before processing

Our handrail is on outside of front deck

The points picked will be top of steel

So we will go to -2 ½….on the W12x26 beam

# Group Members

Purpose: To group members into 1 member (like Revit groups)

First Floor

Stair landing to group

Model….group member……create

Pick 4 members + columns (shift)

Enter

Select main member (like a base point member)

Pick beam

Should process

Now it is one member. You can edit the group, add to the group, subtract from the group…just like Revit groups….just double click….edit

# Sequencing

(kind of like phasing in Revit)

Model….member….sequence by area

Roof

Members….filter

Pick purple pieces…choose 1

Uncheck depth…..get orange stuff

First Floor level

Get Green

Model….status display

General status….sequence (modify condition)

Show item as cyan

Priority: new	2

Change color to orange

Priority: new	3

Change color to green

Bottom of screen check status display

# Hip Roof

Roof view

It will be on opposite side of canopy

Work relative depth: -1-6 (124-6)

Beams W12x22 to outside

Use exact point when going to B2	W12x14

On W8x10, use Xcons member

Column

Pipe 10 std

106-0	124-6	BP4

Top: CP2

B3 column has different base plate BP1 (instead of BP3)

See A3 column base? Uncheck depth? Rotate 180

See C3 column		rotate 90

Process!

# Material

Model….material….add

Note: left first, right second

First Floor

1-0 construction lines	uncheck finite in cons line props

11-0 from B (right) and 1 (twice up)

Bent plate

Select beam B2 to C2 (horiz)

B2 to first 11-0

See rotate material diagog? OK

Pick lower right corner locate material dim ref point

Do next one….OK

If you keep going (on #3 it will add to horiz beam (NO!)

ESC

Pick Vertical beam

Material not smart….does not trim at column

Cope plate around column at B2

Model….material…fit cope

Pick bent plate

Pick column….RC

Clearance: ½	OK

See preview?	YES

Change all options	OK

Miter

Model….material….fit miter

Pick horiz and vert (shift) both

RC….OK

Fit clearance 1/16	OK

# Fabricator Setup

Main Menu

Job or Fabricator Options

Dimensioning Crteria

General tab

Show Help: Dimension style for SDS2

Beam

Show Help

OK

Main Menu

Job or Fabricator Options

Drawing Cosmetics

Text Size (in MM)

Fonts (change to Arial?)

Pens

Bill of Material

Primary Dimensions

Main Menu

Job or Fabricator Options

Fabricator Setup

Member and Material Piecemarking Options

Main Member

Prefixes (show students prefixes!)

# Detailing Erection Views

In Main Menu

Process/Detail….Detail Erection Views

Pick first view….Settings (box on right)

Member style: stick

Annotations

Check Annotation Erection views on the very first time you run it

When you re-detail, make sure it is checked

Check Anchor bolt layout (just for this view)

Select view to detail

Anchor Plan

OK

Now do the rest

Detail Erection views

OK

Pick Views (shift)

OK

You can do it from Modeling too!

To look at views

Main Menu

Drawing Editor

“Home Screen” of drawing editor

Erection Views

Look at Erection Views!

Pick file first….OK

Open (yellow folder)

Next/Previous

Go to Roof Plan

See overlapping details? Drag where you want to make readable.

In drawing editor, there is UNDO (YEA!)

Which means you MUST SAVE (YEA again). Use the diskette so save.

# Erection Views cleanup

Roof Plan

Select and Delete (doesn’t remove from model)

To get it back

Display Options (TV)

Show hidden members

Double click on item

Check on empty box to bring back

Uncheck show hidden members

Main Menu

# Detailing Members

Select Piecemarks

Anything in blue needs to be detailed

OK

Settings

Member annotation box

Uncheck Detail Using Templates

Ok….OK

Starts detailing

Warnings are fine…WOW!

OK

To view details

Drawing Editor

Check Select details (on right at top of select column)

Pick first one B_11??

OK

Use next folder….

Unshorten drawing if edited

Shift select to move

To add dimensions:

Objects….dimension…..add (or use icon)

Watch LC   RC area to place

Save

Shorten drawing

Save

Show tools to add labels, notes

# Detailing Submaterials

Main Menu

Process/Detail

Detail Submaterial

On Select Submaterial dialog box:

Hide

Category and Misc

Check main material (Main materials are not shown as submaterials)

(Total of 4 checked?)

Base plate and cap plates not in list automatically. Pick them too!

OK

Check Detail using templates

OK

It’s working!

OK to warnings

Drawing Editor

Submaterial

Pick first one (a3)

Arrow keys

See baseplates, stair stringer, shear plates, pipe, gusset plates

Not much cleanup!

# Sheet Outlines

Some sheet outlines already in 2016

Need sheet outlines before adding views to sheets

Drawing editor

Sheet outlines

New

Name it: E24x36 (E for Erection Sheet)

Arch D

Check Landscape

OK

See edge. That is plotable sheet area. Note: you could import .dwg titleblock

Std detail

Check Global std

36x24 border

OK

Locate it (RC done)

In title block

Bring in labels (that go on all sheets)

# Labels

Help

Index

L

Labels

Labels special (link)

$NAME (all Caps …case sensitive)

Labels only update when sheet is plotted…just like Revit labels!

Label (icon on left)	$(str)

MC to activate label text

$NAME at DWG NO.

See E24X36 (that is the name of the sheet)

$JOB at JOB

$USER at Made by

$DATE2 at DATE

Delete the white dots

Save (diskette)

Need a new outline sheet for stuff with BOM

Green Floppy

Next drawing name: D24x36BOM

OK

See drawing name D24x36BOM?

To add BOM

Objects

Bill of Materail

Place Bill

Place on top, right corner

LC to accept….Esc

Note: you can place multiple BOM’s on a sheet

Double click on BOM

BOM layout

Appearance

Set minimum number of rows to 10	Max 65

See it is shorter now. As stuff is added, it gets longer (up to 65 items….get it?)

Save

# Gather Sheets

One more sheet outline…Gather sheets

These will be smaller sheets that will hold just one part.

File open (yellow folder)

Check sheet outlines

New

Name it G11x17

OK…..Paper size: ANSIB

Landscape	OK

Draw construction lines ½” in, the 1 ½ “ up from lower construction line to form border layout

Line command over

Will start at center of screen….a little weird….MC to where you want to start

LC to place

Edit lines to solid if they are not (RC…edit)

Add label

$NAME		10 character height	change font

Save

All 3 sheet outlines are now created!

# Loading sheets

Drawing editor

Check Erection sheets

New

Name: E1

OK

E24x36

OK

See E1 in sheet

To add:

Objects

Sheet items

Add

Pick erection view

Anchor Plan

OK

Locate it

You can change scale….try .2

Next sheet

Save/Reuse (green disk)

Next drawing name:E2

Check Discard Sheet Items

Obj

Sheet items

Add Bring Plan views on sheet

OK

Save Reuse

E3

Add rest on sheet (lettered)

E4	Finish

These views were added MANUALLY to sheets

Save

Note: show sheet add icon at bottom of screenDetail Sheets

Open

Detail Sheets

New

Name: 1	OK

D36x24BOM  (see BOM?)

Sheet Item add

Bring on beams

See BOM filling out?

# Detail Sheet Autoloading

Now the good part!

Main menu

Detail Sheet Autoloading

D24x36BOM

Starting sheet 1

Pick all details	OK (select all icon)

Starts autoloading

OK to error. You will have to load manually all drawings that did not autoload. They were too big to fit on sheet at that scale.

HINT: write down piecemarks so you know which ones you need to add to sheet manually!

Detail Sheets

Drawing Editor

Detail sheets

Pick #2

OK

Note piecemarks are all numbered!	2B6…..2B7…etc

Detail sheets

See sheets?!!!!!

For details that did not fit on sheet:

Open

Details

Pick one

Clean up dimensions

Objects

Viewports

Add

Create vport

Make another

Name it

Save

Detail sheets

New	next #A

BOM

Edit

Viewport

Just puts vport in

Next #B

OK….OK

Change to other VP

Save

# Gather Sheets

Open

Check Gather Sheets…..NO…NO….NO! That would be CRAZY!

Main Menu

Gather Sheet Autoloading

G11X17

Use default in box

OK

Select all

Open Gather sheets….there should be a bunch!

Open Gather sheet

See them??

# Moving/Archiving the SDS2 File

Main Menu

Project Transfer

# Printing and Creating PDF’s

Main Menu

See bookmarks to go to sheets and views organized in you pdf program. Note: the company who owns Bluebeam recently bought SDS2…so pdf’s should be great!

# SDS2 to Revit

There are a few ways to get your SDS2 model into Revit. None are perfect yet.

1.SDS2 in ModelLink  exporting model as .xml	In Revit: Import xml using SDS2 Connect

Or Model…..Revit…..export…

Good: Members come in accurately

Bad: No connections come in

Bad: No grids come in

Good: Easy to create grids leveraging accurate geometry

Bad: It’s not a linked file, so you can’t turn it off with one button. You could create a Revit structure file that could be linked with the other disciplines.

2. Open IFC

SDS2 in ModelLink  exporting model as IFC (EM11IFC)

In Revit:

Open (arrow) IFC Options

Default Template for IFC Import-Proj IFC

Open (arrow) IFC

Pick file from SDS2

2. Link IFC

SDS2 in ModelLink  exporting model as IFC (EM11IFC)

In Revit:

Insert

Link IFC

Pick the linked file.